****************************************************************************

*   Pocket Ethernet Adapter Series  ---  Driver information         V4.20  *

****************************************************************************

Date: 06/25/1997

Compatibility Report

====================

Pocket Ethernet LAN Adapters have been tested and found fully

compatible with the following NOS:

Novell NetWare V2.2

Novell NetWare V3.11

Novell NetWare V3.12

Novell NetWare V4.01

Novell NetWare V4.1

Novell Personal NetWare V1.0

Microsoft LAN Manager V2.0C & V2.1

Microsoft Windows For Workgroups V3.11

FTP software PC/TCP V2.05

ARTISOFT LANtastic V4.x, 5.x & V6.x.

SUN PC-NFS V5.0

IBM LAN Server V4.0

Microsoft Windows NT 3.5

Microsoft Windows 95

EPP (Enhanced Parallel Port) Mode Installation

==============================================

To install the EPP.EXE, include a command line of the following

form in your CONFIG.SYS file:

device=[drive][path]EPP.EXE

for example:

device=C:\PE\epp.exe

Or install this driver under DOS prompt

for example:

A:\EPP  <Enter>

Note: IF the parallel port of your PC is setup to EPP mode, do not use

this program to setup again.

MUX (Multiplexer Printer Port) Driver Installation

==================================================

This driver is provided for users to print from local printer port, while

the pocket LAN adapter is attached but the network driver is not loaded,

To install the MUX.COM driver, type "MUX" under DOS prompt

for example:

A:\MUX  <Enter>

Note: If the network driver is installed, the MUX driver will be loaded

automatically. You do not need to run MUX.COM driver.

AUTOINST (Pocket Ethernet LAN Drivers Auto Installation)

==================================================================

This program is provided for users to auto install the LAN drivers for

the Pocket LAN Series Ethernet Adapter.

To execute the AUTOINST.EXE , type "AUTOINST" under DOS prompt.

for example:

A:\AUTOINST  <Enter>

The program will instruct user to auto-install the driver step by step.