LevelOne FBR-1800TX

1-Port Broadband router w/8 LAN Ports

+ Printer Server

User’s Manual

Version:1.1

Table of Contents

Chapter 1 Introduction	1

LevelOne FBR-1800TX Broadband Router Features	1

Package Contents	3

Physical Details	4

Chapter 2 Installation	6

Requirements	6

Procedure	6

Chapter 3 Setup	8

Overview	8

Configuration Program	9

Setup Wizard	11

LAN Screen	14

Password Screen	16

Chapter 4 PC Configuration	17

Overview	17

Windows Clients	17

Printer Setup for Windows	29

Macintosh Clients	35

Linux Clients	35

Other Unix Systems	36

Chapter 5 Operation and Status	37

Operation	37

Status Screen	37

Connection Status - PPPoE	39

Connection Status - PPTP	41

Connection Status - Telstra Big Pond	42

Connection Details - SingTel RAS	43

Connection Details - Fixed/Dynamic IP Address	44

Chapter 6 Advanced Features	46

Overview	46

Advanced Menu Screen	46

Advanced Internet Screen	47

Access Control	50

Remote Management	58

Virtual Servers	59

Dynamic DNS (Domain Name Server)	62

Upgrade Firmware	64

Chapter 7 Advanced Configuration	65

Overview	65

PC Database	66

Options	70

Security	72

Logs	74

MAC Address	76

Printer Port	77

Routing	78

Appendix A Troubleshooting	82

Overview	82

General Problems	82

Internet Access	82

Printing	83

Appendix B Specifications	86

Multi-Function LevelOne FBR-1800TX Broadband Router	86

FCC Statement	86

P/N: 9560J500A0

Chapter 1
Introduction

1

This Chapter provides an overview of the LevelOne FBR-1800TX Broadband Router's features and capabilities.

Congratulations on the purchase of your new LevelOne FBR-1800TX Broadband Router. The LevelOne FBR-1800TX Broadband Router is a multi-function device providing the following services:

- Shared Broadband Internet Access for all LAN  users.

- 8-Port Switching Hub for 10BaseT or 100BaseT connections.

- Network Printer  - LAN users can share the printer attached to the LevelOne FBR-1800TX Broadband Router.

- Figure 1: LevelOne FBR-1800TX Broadband Router

## LevelOne FBR-1800TX Broadband Router Features

The LevelOne FBR-1800TX Broadband Router incorporates many advanced features, carefully designed to provide sophisticated functions while being easy to use.

### Internet Access Features

- Shared Internet Access.  All users on the LAN or WLAN can access the Internet through the LevelOne FBR-1800TX Broadband Router, using only a single external IP Address. The local (invalid) IP Addresses are hidden from external sources. This process is called NAT (Network Address Translation).

- DSL & Cable Modem Support.  The LevelOne FBR-1800TX Broadband Router has a 10BaseT Ethernet port for connecting a DSL or Cable Modem. All popular DSL and Cable Modems are supported. SingTel RAS and Big Pond (Australia) login support is also included.

- PPPoE, PPTP, SingTel RAS and Telstra Big Pond Support.  The Internet (WAN port) connection supports PPPoE (PPP over Ethernet), PPTP (Peer-to-Peer Tunneling Protocol), SingTel RAS and Telstra Big Pond (Australia), as well as "Direct Connection" type services.

- Fixed or Dynamic IP Address.  On the Internet (WAN port) connection, the LevelOne FBR-1800TX Broadband Router supports both Dynamic IP Address (IP Address is allocated on connection) and Fixed IP Address.

### Advanced Internet Functions

- Communication Applications.  Support for Internet communication applications, such as interactive Games, Telephony, and Conferencing applications, which are often difficult to use when behind a Firewall, is included.

- Special Internet Applications.  Applications which use non-standard connections or port numbers are normally blocked by the Firewall. The ability to define and allow such applications is provided, to enable such applications to be used normally.

- Virtual Servers.  This feature allows Internet users to access Internet servers on your LAN. The required setup is quick and easy.

- DMZ.  One (1) PC on your local LAN can be configured to allow unrestricted 2-way communication with Servers or individual users on the Internet. This provides the ability to run programs which are incompatible with Firewalls.

- URL Filter.  Use the URL Filter to block access to undesirable Web sites by LAN users.

- Internet Access Log.  See which Internet connections have been made.

- VPN Pass through Support.  PCs with VPN (Virtual Private Networking) software using PPTP, L2TP and IPSec are transparently supported - no configuration is required.

### LAN Features

- 8-Port Switching Hub.  The LevelOne FBR-1800TX Broadband Router incorporates a 8-port 10/100BaseT switching hub, making it easy to create or extend your LAN.

- DHCP Server Support.  Dynamic Host Configuration Protocol provides a dynamic IP address to PCs and other devices upon request. The LevelOne FBR-1800TX Broadband Router can act as a DHCP Server for devices on your local LAN and WLAN.

- Multi Segment LAN Support.  LANs containing one or more segments are supported, via the LevelOne FBR-1800TX Broadband Router's RIP (Routing Information Protocol) support and built-in static routing table.

### Network Printer

- Shared Printer (Network Printer).  A printer connected to the LevelOne FBR-1800TX Broadband Router's parallel port can be shared by all PCs on the LAN or WLAN.

- Multiple OS Support.  Clients may use any of the following operating systems:

- Windows 95/98/ME

- Windows NT 4.0, 2000, or XP.

- Unix

- Multi-protocol Support.  The following printing methods are supported:

- Windows peer-to-peer printing over TCP/IP, using the supplied port driver.

- Windows LPD printing, using a Windows Server running NT 4.0 or Windows 2000 Server. In this situation, no software needs to be installed on the client PCs.

- Unix LPD printing. No additional software needs to be installed.

### Configuration & Management

- Easy Setup.  Use your WEB browser from anywhere on the LAN or WLAN for configuration.

- Remote Management.  The LevelOne FBR-1800TX Broadband Router can be managed from any PC on your LAN. And, if the Internet connection exists, it can also (optionally) be configured via the Internet.

- UPnP Support.  UPnP (Universal Plug and Play) allows automatic discovery and configuration of the LevelOne FBR-1800TX Broadband Router. UPnP is by supported by Windows ME, XP, or later.

### Security Features

- Password - protected Configuration.  Optional password protection is provided to prevent unauthorized users from modifying the configuration data and settings.

- NAT Protection.  An intrinsic side effect of NAT (Network Address Translation) technology is that by allowing all LAN users to share a single IP address, the location and even the existence of each PC is hidden. From the external viewpoint, there is no network, only a single device - the LevelOne FBR-1800TX Broadband Router.

- Stateful Inspection Firewall.  All incoming data packets are monitored and all incoming server requests are filtered, thus protecting your network from malicious attacks from external sources.

- Protection against DoS attacks.  DoS (Denial of Service) attacks can flood your Internet connection with invalid packets and connection requests, using so much bandwidth and so many resources that Internet access becomes unavailable. The LevelOne FBR-1800TX Broadband Router incorporates protection against DoS attacks.

## Package Contents

The following items should be included:

- The LevelOne FBR-1800TX Broadband Router Unit

- Power Adapter

- Quick Installation Guide

- CD-ROM containing the on-line manual and Print Port Driver for Windows.

If any of the above items are damaged or missing, please contact your dealer immediately.

## Physical Details

### Front-mounted LEDs

Figure 2: Front Panel

| Power | On - Power on. Off - No power. |
| --- | --- |
| Status (Red) | On - Error condition. Off - Normal operation. Blinking - This LED blinks during start up. |
| Print Act | On - Connection to printer established. Off - No connection to printer; printer is Off or Off-line. Flashing - Data is being transmitted to the printer. |
| Print Error | On - Printer error detected. Off - No printer error detected. |
| WAN | Link/Act - Connection to the modem attached to the WAN (Internet) port is established. 100 - Corresponding WAN port is using 100BaseT. |
| LAN | For each port, there are 2 LEDs - Link/Act - On - Corresponding LAN (hub) port is active. - Off - No active connection on the corresponding LAN (hub) port. - Flashing - Data is being transmitted or received via the corresponding LAN (hub) port. - 100 - On - Corresponding LAN (hub) port is using 100BaseT. - Off - Corresponding LAN (hub) port connection is using 10BaseT, or no active connection. |

### Rear Panel

Figure 3: Rear Panel

| Printer Port | Standard parallel printer port. If you wish to share a printer, connect it here. |
| --- | --- |
| Reset Button | This button has three (3) functions: - Reboot. When pressed and released, the LevelOne FBR-1800TX Broadband Router will reboot (restart). - Diagnostic print-out. If held down for 3 seconds, a diagnostic print-out will be sent to the attached printer. - Ensure the printer is ready. - Both Print LEDs will flash simultaneously during the diagnostic printing. - Clear All Data. This button can also be used to clear ALL data and restore ALL settings to the factory default values. To Clear All Data and restore the factory default values: - Power Off. - Hold the Reset Button down while you Power On. - Keep holding the Reset Button for a few seconds, until the RED LED has flashed TWICE. - Release the Reset Button. The LevelOne FBR-1800TX Broadband Router is now using the factory default values. |
| 10/100BaseT LAN connections | Use standard LAN cables (RJ45 connectors) to connect your PCs to these ports. Note: Any LAN port on the LevelOne FBR-1800TX Broadband Router will automatically function as an "Uplink" port when required. Just connect any port to a normal port on the other hub, using a standard LAN cable. |
| WAN port (10/100BaseT) | Connect the DSL or Cable Modem here. If your modem came with a cable, use the supplied cable. Otherwise, use a standard LAN cable. |
| Power port | Connect the supplied power adapter here. |

Chapter 2
Installation

2

This Chapter covers the physical installation of the LevelOne FBR-1800TX Broadband Router.

## Requirements

- Network cables. Use standard 10/100BaseT network (UTP) cables with RJ45 connectors.

- TCP/IP protocol must be installed on all PCs.

- For Internet Access, an Internet Access account with an ISP, and either of a DSL or Cable modem (for WAN port usage)

- For shared access to the attached printer, the following clients are supported:

- Windows 95/98/ME

- Windows NT 4.0, 2000 or XP

- Unix (LPD printing)

## Procedure

Figure 4: Installation Diagram

1. Choose an Installation Site

Select a suitable place on the network to install the LevelOne FBR-1800TX Broadband Router. 
Ensure the LevelOne FBR-1800TX Broadband Router and the DSL/Cable modem are powered OFF.

2. Connect LAN Cables

Use standard LAN cables to connect PCs to the Switching Hub ports on the LevelOne FBR-1800TX Broadband Router. Both 10BaseT and 100BaseT connections can be used simultaneously.

If required, connect any port to a normal port on another Hub, using a standard LAN cable. Any LAN port on the LevelOne FBR-1800TX Broadband Router will automatically function as an "Uplink" port when required.

3. Connect WAN Cable

Connect the DSL or Cable modem to the WAN port on the LevelOne FBR-1800TX Broadband Router. Use the cable supplied with your DSL/Cable modem. If no cable was supplied, use a standard cable.

4. Connect Printer Cable

Use a standard parallel printer cable to connect your printer to the Printer port on the LevelOne FBR-1800TX Broadband Router.

5. Power Up

- Power on the Cable or DSL modem.

- Connect the supplied power adapter to the LevelOne FBR-1800TX Broadband Router and power up. 
Use only the power adapter provided. Using a different one may cause hardware damage

6. Check the LEDs

- The Power LED should be ON.

- The Status LED should flash, then turn Off. If it stays on, there is a hardware error.

- For each LAN (PC) connection, the LAN Link/Act LED should be ON (provided the PC is also ON.)

- The WAN LED should be ON.

- The Print Act LED should be ON.

For more information, refer to Front-mounted LEDs in Chapter 1.

Chapter 3
Setup

3

This Chapter provides Setup details of the LevelOne FBR-1800TX Broadband Router.

## Overview

This chapter describes the setup procedure for:

- Internet Access

- LAN configuration

- Assigning a Password to protect the configuration data.

PCs on your local LAN may also require configuration. For details, see Chapter 4 - PC Configuration.

Other configuration may also be required, depending on which features and functions of the LevelOne FBR-1800TX Broadband Router you wish to use. Use the table below to locate detailed instructions for the required functions.

| To Do this: | Refer to: |
| --- | --- |
| Configure PCs on your LAN. | Chapter 4: PC Configuration |
| Check LevelOne FBR-1800TX Broadband Router operation and Status. | Chapter 5: Operation and Status |
| Use any of the following Internet features: - Special Applications - DMZ - Virtual Servers - Dynamic DNS - Remote Management - Firmware Upgrade | Chapter 6: Advanced Features |
| Use any of the following Advanced Configuration settings: - PC Database - Options (Backup DNS, TFTP, UPnP, Firewall) - Printer Port setup (for Linux/Unix only) - Routing (RIP and static Routing) | Chapter 7 Advanced Configuration |

|  | Where use of a certain feature requires that PCs or other LAN devices be configured, this is also explained in the relevant chapter. |
| --- | --- |

## Configuration Program

The LevelOne FBR-1800TX Broadband Router contains an HTTP server. This enables you to connect to it, and configure it, using your Web Browser. Your Browser must support JavaScript. The configuration program has been tested on the following browsers:

- Netscape V4.08 or later

- Internet Explorer V4 or later

### Preparation

Before attempting to configure the LevelOne FBR-1800TX Broadband Router, please ensure that:

- Your PC can establish a physical connection to the LevelOne FBR-1800TX Broadband Router. The PC and the LevelOne FBR-1800TX Broadband Router must be directly connected (using the Hub ports on the LevelOne FBR-1800TX Broadband Router) or on the same LAN segment.

- The LevelOne FBR-1800TX Broadband Router must be installed and powered ON.

- If the LevelOne FBR-1800TX Broadband Router's default IP Address (192.168.0.1) is already used by another device, the other device must be turned OFF until the LevelOne FBR-1800TX Broadband Router is allocated a new IP Address during configuration.

#### Using UPnP

If your Windows system supports UPnP, an icon for the LevelOne FBR-1800TX Broadband Router will appear in the system tray, notifying you that a new network device has been found, and offering to create a new desktop shortcut to the newly-discovered device.

- Unless you intend to change the IP Address of the LevelOne FBR-1800TX Broadband Router, you can accept the desktop shortcut.

- Whether you accept the desktop shortcut or not, you can always find UPnP devices in My Network Places (previously called Network Neighborhood).

- Double - click the icon for the LevelOne FBR-1800TX Broadband Router (either on the Desktop, or in My Network Places) to start the configuration. Refer to the following section Setup Wizard for details of the initial configuration process.

#### Using your Web Browser

To establish a connection from your PC to the LevelOne FBR-1800TX Broadband Router:

- After installing the LevelOne FBR-1800TX Broadband Router in your LAN, start your PC. If your PC is already running, restart it.

- Start your WEB browser.

- In the Address box, enter "HTTP://" and the IP Address of the LevelOne FBR-1800TX Broadband Router, as in this example, which uses the LevelOne FBR-1800TX Broadband Router's default IP Address:

HTTP://192.168.0.1

If you can't connect If the LevelOne FBR-1800TX Broadband Router does not respond, check the following: - The LevelOne FBR-1800TX Broadband Router is properly installed, LAN connection is OK, and it is powered ON. You can test the connection by using the "Ping" command: - Open the MS-DOS window or command prompt window. - Enter the command: ping 192.168.0.1 If no response is received, either the connection is not working, or your PC's IP address is not compatible with the LevelOne FBR-1800TX Broadband Router's IP Address. (See next item.) - If your PC is using a fixed IP Address, its IP Address must be within the range 192.168.0.2 to 192.168.0.254 to be compatible with the LevelOne FBR-1800TX Broadband Router's default IP Address of 192.168.0.1. Also, the Network Mask must be set to 255.255.255.0. See Chapter 4 - PC Configuration for details on checking your PC's TCP/IP settings. - Ensure that your PC and the LevelOne FBR-1800TX Broadband Router are on the same network segment. (If you don't have a router, this must be the case.)

## Setup Wizard

The first time you connect to the LevelOne FBR-1800TX Broadband Router, the Setup Wizard will run automatically. (The Setup Wizard will also run if the LevelOne FBR-1800TX Broadband Router's default setting are restored.)

- Step through the Wizard until finished.

- You need to know the type of Internet connection service used by your ISP. Check the data supplied by your ISP.

- The common connection types are explained in the tables below.

- On the final screen of the Wizard, run the test and check that an Internet connection can be established.

- If the connection test fails:

- Check your data, the Cable/DSL modem, and all connections.

- Check that you have entered all data correctly.

- If using a Cable modem, your ISP may have recorded the MAC (physical) address of your PC. Run the Wizard, and on the Cable Modem screen, use the "Clone MAC address" button to copy the MAC address from your PC to the LevelOne FBR-1800TX Broadband Router.

### Common Connection Types

Cable Modems

| Type | Details | ISP Data required |
| --- | --- | --- |
| Dynamic IP Address | Your IP Address is allocated automatically, when you connect to you ISP. | Usually, none. However, some ISP's may require you to use a particular Hostname, Domain name, or MAC (physical) address. |
| Static (Fixed) IP Address | Your ISP allocates a permanent IP Address to you. | IP Address allocated to you. Some ISP's may also require you to use a particular Hostname, Domain name, or MAC (physical) address. |

DSL Modems

| Type | Details | ISP Data required |
| --- | --- | --- |
| Dynamic IP Address | Your IP Address is allocated automatically, when you connect to you ISP. | None. |
| Static (Fixed) IP Address | Your ISP allocates a permanent IP Address to you. | IP Address allocated to you. |
| PPPoE | You connect to the ISP only when required. The IP address is usually allocated automatically. | User name and password. |
| PPTP | Mainly used in Europe. You connect to the ISP only when required. The IP address is usually allocated automatically, but may be Static (Fixed). | - PPTP Server IP Address. - User name and password. - IP Address allocated to you, if Static (Fixed). |

Other Modems (e.g. Broadband Wireless)

| Type | Details | ISP Data required |
| --- | --- | --- |
| Dynamic IP Address | Your IP Address is allocated automatically, when you connect to you ISP. | None. |
| Static (Fixed) IP Address | Your ISP allocates a permanent IP Address to you. | IP Address allocated to you. |

Big Pond (Australia)

For this connection method, the following data is required:

- User Name

- Password

- Big Pond Server IP address

SingTel RAS

For this connection method, the following data is required:

- User Name

- Password

- RAS Plan

### Home Screen

After finishing the Setup Wizard, you will see the Home screen. When you connect in future, you will see this screen when you connect. An example screen is shown below.

Figure 5: Home Screen

#### Navigation & Data Input

- Use the menu bar on the top of the screen, and the "Back" button on your Browser, for navigation.

- Changing to another screen without clicking "Save" does NOT save any changes you may have made. You must "Save" before changing screens or your data will be ignored.

|  | On each screen, clicking the "Help" button will display help for that screen. From any help screen, you can access the list of all help files (help index). |
| --- | --- |

## LAN Screen

Use the LAN link on the main menu to reach the LAN screen An example screen is shown below.

Figure 6: LAN Screen

#### Data - LAN Screen

| TCP/IP |  |
| --- | --- |
| IP Address | IP address for the LevelOne FBR-1800TX Broadband Router, as seen from the local LAN. Use the default value unless the address is already in use or your LAN is using a different IP address range. In the latter case, enter an unused IP Address from within the range used by your LAN. |
| Subnet Mask | The default value 255.255.255.0 is standard for small (class "C") networks. For other networks, use the Subnet Mask for the LAN segment to which the LevelOne FBR-1800TX Broadband Router is attached (the same value as the PCs on that LAN segment). |
| DHCP Server | - If Enabled, the LevelOne FBR-1800TX Broadband Router will allocate IP Addresses to PCs (DHCP clients) on your LAN when they start up. The default (and recommended) value is Enabled. - If you are already using a DHCP Server, this setting must be Disabled, and the existing DHCP server must be re-configured to treat the LevelOne FBR-1800TX Broadband Router as the default Gateway. See the following section for further details. - The Start IP Address and Finish IP Address fields set the values used by the DHCP server when allocating IP Addresses to DHCP clients. This range also determines the number of DHCP clients supported. - See the following section for further details on using DHCP. |
| Buttons |  |
| Save | Save the data on screen. |
| Cancel | The "Cancel" button will discard any data you have entered and reload the file from the LevelOne FBR-1800TX Broadband Router. |

### DHCP

#### What DHCP Does

A DHCP (Dynamic Host Configuration Protocol) Server allocates a valid IP address to a DHCP Client (PC or device) upon request.

- The client request is made when the client device starts up (boots).

- The DHCP Server provides the Gateway and DNS addresses to the client, as well as allocating an IP Address.

- The LevelOne FBR-1800TX Broadband Router can act as a DHCP server.

- Windows 95/98/ME and other non-Server versions of Windows will act as a DHCP client. This is the default Windows setting for the TCP/IP network protocol. However, Windows uses the term Obtain an IP Address automatically instead of "DHCP Client".

- You must NOT have two (2) or more DHCP Servers on the same LAN segment. (If your LAN does not have other Routers, this means there must only be one (1) DHCP Server on your LAN.)

#### Using the LevelOne FBR-1800TX Broadband Router's DHCP Server

This is the default setting. The DHCP Server settings are on the LAN screen. On this screen, you can:

- Enable or Disable the LevelOne FBR-1800TX Broadband Router's DHCP Server function.

- Set the range of IP Addresses allocated to PCs by the DHCP Server function.

|  | You can assign Fixed IP Addresses to some devices while using DHCP, provided that the Fixed IP Addresses are NOT within the range used by the DHCP Server. |
| --- | --- |

#### Using another DHCP Server

You can only use one (1) DHCP Server per LAN segment. If you wish to use another DHCP Server, rather than the LevelOne FBR-1800TX Broadband Router's, the following procedure is required.

- Disable the DHCP Server feature in the LevelOne FBR-1800TX Broadband Router. This setting is on the LAN screen.

- Configure the DHCP Server to provide the LevelOne FBR-1800TX Broadband Router's IP Address as the Default Gateway.

#### To Configure your PCs to use DHCP

This is the default setting for TCP/IP under Windows 95/98/ME.

See Chapter 4 - Client Configuration for the procedure to check these settings.

## Password Screen

The password screen allows you to assign a password to the LevelOne FBR-1800TX Broadband Router.

Figure 7: Password Screen

Once you have assigned a password to the LevelOne FBR-1800TX Broadband Router (on the Password screen above) you will be prompted for the password when you connect, as shown below. (If no password has been set, this dialog will not appear.)

Figure 8: Password Dialog

- Leave the "User Name" blank.

- Enter the password for the LevelOne FBR-1800TX Broadband Router, as set on the Password screen above.

Chapter 4
PC Configuration

4

This Chapter details the PC Configuration required on the local ("Internal") LAN.

## Overview

For each PC, the following may need to be configured:

- TCP/IP network settings

- Internet Access configuration

- Printer configuration

## Windows Clients

This section describes how to configure Windows clients for:

- Internet access via the LevelOne FBR-1800TX Broadband Router

- Sharing the Printer connected to the LevelOne FBR-1800TX Broadband Router.

The first step is to check the PC's TCP/IP settings.

The LevelOne FBR-1800TX Broadband Router uses the TCP/IP network protocol for all functions, so it is essential that the TCP/IP protocol be installed and configured on each PC.

### TCP/IP Settings - Overview

If using the default LevelOne FBR-1800TX Broadband Router settings, and the default Windows TCP/IP settings, no changes need to be made.

- By default, the LevelOne FBR-1800TX Broadband Router will act as a DHCP Server, automatically providing a suitable IP Address (and related information) to each PC when the PC boots.

- For all non-Server versions of Windows, the default TCP/IP setting is to act as a DHCP client.

If using a Fixed (specified) IP address, the following changes are required:

- The Gateway must be set to the IP address of the LevelOne FBR-1800TX Broadband Router

- The DNS should be set to the address provided by your ISP.

|  | If your LAN has a Router, the LAN Administrator must re-configure the Router itself. Refer to Chapter 8 - Advanced Setup for details. |
| --- | --- |

### Checking TCP/IP Settings - Windows 9x/ME:

- Select Control Panel - Network. You should see a screen like the following:

- Figure 9: Network Configuration

- Select the TCP/IP protocol for your network card.

- Click on the Properties button. You should then see a screen like the following.

- Figure 10: IP Address (Win 95)

Ensure your TCP/IP settings are correct, as follows:

Using DHCP

To use DHCP, select the radio button Obtain an IP Address automatically. This is the default Windows setting. Using this is recommended. By default, the LevelOne FBR-1800TX Broadband Router will act as a DHCP Server.

Restart your PC to ensure it obtains an IP Address from the LevelOne FBR-1800TX Broadband Router.

Using "Specify an IP Address"

If your PC is already configured, check with your network administrator before making the following changes:

- On the Gateway tab, enter the LevelOne FBR-1800TX Broadband Router's IP address in the New Gateway field and click Add, as shown below. Your LAN administrator can advise you of the IP Address they assigned to the LevelOne FBR-1800TX Broadband Router.

Figure 11: Gateway Tab (Win 95/98)

- On the DNS Configuration tab, ensure Enable DNS is selected. If the DNS Server Search Order list is empty, enter the DNS address provided by your ISP in the fields beside the Add button, then click Add.

Figure 12: DNS Tab (Win 95/98)

### Checking TCP/IP Settings - Windows NT4.0

- Select Control Panel - Network, and, on the Protocols tab, select the TCP/IP protocol, as shown below.

Figure 13: Windows NT4.0 - TCP/IP

- Click the Properties button to see a screen like the one below.

Figure 14: Windows NT4.0 - IP Address

- Select the network card for your LAN.

- Select the appropriate radio button - Obtain an IP address from a DHCP Server or Specify an IP Address, as explained below.

Obtain an IP address from a DHCP Server

This is the default Windows setting. Using this is recommended. By default, the LevelOne FBR-1800TX Broadband Router will act as a DHCP Server.

Restart your PC to ensure it obtains an IP Address from the LevelOne FBR-1800TX Broadband Router.

Specify an IP Address

If your PC is already configured, check with your network administrator before making the following changes.

- The Default Gateway must be set to the IP address of the LevelOne FBR-1800TX Broadband Router. To set this:

- Click the Advanced button on the screen above.

- On the following screen, click the Add button in the Gateways panel, and enter the LevelOne FBR-1800TX Broadband Router's IP address, as shown in Figure 15 below.

- If necessary, use the Up button to make the LevelOne FBR-1800TX Broadband Router the first entry in the Gateways list.

Figure 15 - Windows NT4.0 - Add Gateway

- The DNS should be set to the address provided by your ISP, as follows:

- Click the DNS tab.

- On the DNS screen, shown below, click the Add button (under DNS Service Search Order), and enter the DNS provided by your ISP.

Figure 16: Windows NT4.0 - DNS

### Checking TCP/IP Settings - Windows 2000:

- Select Control Panel - Network and Dial-up Connection.

- Right - click the Local Area Connection icon and select Properties. You should see a screen like the following:

Figure 17: Network Configuration (Win 2000)

- Select the TCP/IP protocol for your network card.

- Click on the Properties button. You should then see a screen like the following.

Figure 18: TCP/IP Properties (Win 2000)

- Ensure your TCP/IP settings are correct, as described below.

Using DHCP

To use DHCP, select the radio button Obtain an IP Address automatically. This is the default Windows setting. Using this is recommended. By default, the LevelOne FBR-1800TX Broadband Router will act as a DHCP Server.

Restart your PC to ensure it obtains an IP Address from the LevelOne FBR-1800TX Broadband Router.

Using a fixed IP Address ("Use the following IP Address")

If your PC is already configured, check with your network administrator before making the following changes.

- Enter the LevelOne FBR-1800TX Broadband Router's IP address in the Default gateway field and click OK. (Your LAN administrator can advise you of the IP Address they assigned to the LevelOne FBR-1800TX Broadband Router.)

- If the DNS Server fields are empty, select Use the following DNS server addresses, and enter the DNS address or addresses provided by your ISP, then click OK.

### Checking TCP/IP Settings - Windows XP

- Select Control Panel - Network Connection.

- Right click the Local Area Connection and choose Properties. You should see a screen like the following:

Figure 19: Network Configuration (Windows  XP)

- Select the TCP/IP protocol for your network card.

- Click on the Properties button. You should then see a screen like the following.

Figure 20: TCP/IP Properties (Windows XP)

- Ensure your TCP/IP settings are correct.

Using DHCP

To use DHCP, select the radio button Obtain an IP Address automatically. This is the default Windows setting. Using this is recommended. By default, the LevelOne FBR-1800TX Broadband Router will act as a DHCP Server.

Restart your PC to ensure it obtains an IP Address from the LevelOne FBR-1800TX Broadband Router.

Using a fixed IP Address ("Use the following IP Address")

If your PC is already configured, check with your network administrator before making the following changes.

- In the Default gateway field, enter the LevelOne FBR-1800TX Broadband Router's IP address and click OK. Your LAN administrator can advise you of the IP Address they assigned to the LevelOne FBR-1800TX Broadband Router.

- If the DNS Server fields are empty, select Use the following DNS server addresses, and enter the DNS address or addresses provided by your ISP, then click OK.

### Internet Access

To configure your PCs to use the LevelOne FBR-1800TX Broadband Router for Internet access:

- Ensure that the DSL modem, Cable modem, or other permanent connection is functional.

- Use the following procedure to configure your Browser to access the Internet via the LAN, rather than by a Dial-up connection.

#### For Windows 9x/ME/2000

- Select Start Menu - Settings - Control Panel - Internet Options.

- Select the Connection tab, and click the Setup button.

- Select "I want to set up my Internet connection manually, or I want to connect through a local area network (LAN)" and click Next.

- Select "I connect through a local area network (LAN)" and click Next.

- Ensure all of the boxes on the following Local area network Internet Configuration screen are unchecked.

- Check the "No" option when prompted "Do you want to set up an Internet mail account now?".

- Click Finish to close the Internet Connection Wizard. 
Setup is now completed.

#### For Windows XP

- Select Start Menu - Control Panel - Network and Internet Connections.

- Select Set up or change your Internet Connection.

- Select the Connection tab, and click the Setup button.

- Cancel the pop-up "Location Information" screen.

- Click Next on the "New Connection Wizard" screen.

- Select "Connect to the Internet" and click Next.

- Select "Set up my connection manually" and click Next.

- Check "Connect using a broadband connection that is always on" and click Next.

- Click Finish to close the New Connection Wizard.
Setup is now completed.

#### Accessing AOL

To access AOL (America On Line) through the LevelOne FBR-1800TX Broadband Router, the AOL for Windows software must be configured to use TCP/IP network access, rather than a dial-up connection. The configuration process is as follows:

- Start the AOL for Windows communication software. Ensure that it is Version 2.5, 3.0 or later. This procedure will not work with earlier versions.

- Click the Setup button.

- Select Create Location, and change the location name from "New Locality" to "LevelOne FBR-1800TX Broadband Router".

- Click Edit Location. Select TCP/IP for the Network field. (Leave the Phone Number blank.)

- Click Save, then OK. 
Configuration is now complete.

- Before clicking "Sign On", always ensure that you are using the "LevelOne FBR-1800TX Broadband Router" location.

## Printer Setup for Windows

The LevelOne FBR-1800TX Broadband Router provides printing support for 2 methods for printing from Windows:

- Print Port Driver.  After installing the Print Port Driver, Windows users can print directly to the LevelOne FBR-1800TX Broadband Router. Print jobs are spooled (queued) on each PC.
The supplied Print Port Driver supports Windows 95/98, Windows ME, Windows NT4.0,  Windows 2000 and Windows XP.

- LPD/LPR Printing.  If using Windows NT 4.0 Server or Windows 2000 Server, LPD/LPR printing can be used. No software needs to be installed on either the Windows Server or each client PC. Print jobs will be spooled (queued) on the Windows Server, and can be managed using the standard Windows Server tools.

### Print Port Driver Setup

The following procedure is for all versions of Windows (95/98/ME, NT4.0, 2000, XP). The Windows "Add Printer" screens will vary depending on your version or Windows, but the procedure is the same:

- Insert the supplied CD-ROM into your drive. If the setup program does not start automatically, run SETUP.EXE in the root folder.

- At the Select Components screen, select the Print Port Driver option.

- Follow the prompts to complete the installation.

- The Print Port Setup will then run, and the following screen will be displayed.

Figure 21: Print Port Setup

- Select the desired device and port, and then click the "Add" button.

|  | Under Windows 95, if you see the following error message, either install Internet Explorer 4 or later, or follow the procedure in the "Trouble Shooting - Printing" section of Appendix A. |
| --- | --- |

- A pop-up message will inform you if the port has been created successfully, and then the Windows Add Printer wizard will start.

- Select the correct Printer Manufacturer and Model, or use the "Have Disk" option if appropriate.

- If desired, change the Printer name so it indicates the device used (e.g. HP2100 on SCA43600)

- If prompted about Sharing, do NOT enable Sharing.

- Installation is now complete. You can now print using this printer.

- To install additional Printers (on different devices), repeat steps 6 and 7.

- Use the Start menu to run this program in future. The default installation is Start - Programs - Broadband Internet Router - Add Port.

Management

- Print jobs can be managed like any Windows printer. Open the Printers folder (Start - Settings - Printers) and double-click any printer to see the current print jobs.

- If the printer attached to the LevelOne FBR-1800TX Broadband Router is changed, just run this program again, and select the correct printer.

- To delete a port created by this setup program, use the Windows Delete Port facility:

- Right-click any printer in the Printers folder, and select Properties.

- Locate the Delete Port button. This button is on the Details or Ports tab, depending on your version or Windows.

- If the LevelOne FBR-1800TX Broadband Router's IP Address is changed, and you can no longer print, delete the port (see procedure above) and re-install it.

Port Options

The options for the Print Port Driver are accessed via the Windows Port Settings button.

Use Start - Settings - Printers to open the Printers folder, then right-click the Printer, and select Properties. The Port Settings button is on the Details or Port tab, depending on your version of Windows.

An example screen is shown below:

Figure 22: Print Port Configuration

Items shown on this screen are as follows:

| Port | If desired, click Browse to select a different device. (The Select Device Port button is provided to allow this software to work with multi-port models.) The Port Name is shown in the Printer's Properties. |
| --- | --- |
| Banner | Check this option to print a banner page before each print job. - If using a PostScript Printer, check the PostScript box. - The User Name will be printed on the banner page. |
| Retry Interval | Sets how often Windows will poll the LevelOne FBR-1800TX Broadband Router to establish a connection when the printer is busy. Increase this value if you get too many warning messages. |

### LPD/LPR Printing

LPD/LPR printing can be used with Windows NT 4.0 Server or Windows 2000. No software needs to be installed on client PCs.

#### Windows NT 4.0 Server Configuration

To use LPD printing, Microsoft TCP/IP Printing must be installed and enabled. This can be checked using Start-Settings-Control Panel-Network - Services.

To install LPD printing using the LevelOne FBR-1800TX Broadband Router, follow this procedure:

- Go to Start-Settings-Printer and invoke the Add Printer wizard.

- When prompted with "This printer will be managed by..", select My Computer and click Next.

- Select Add Port, then select LPR Port and click New Port.

- In the Dialog requesting Name of Address of server providing lpd, enter the IP address of the LevelOne FBR-1800TX Broadband Router.

- For Name of printer or print queue on that server, enter L1

- Click OK. When returned to the Printer Ports window, simply select Close and then install your printer driver as usual.

- When prompted whether or not the printer will be shared, select the Sharing radio button.

- In the Shared dialog box, enter the shared printer name. The shared name is how other users will see this printer. You should advise client PCs of the Server name and this printer name.

- Click OK to save and exit.

#### Windows 2000 Server Configuration

The LPD/LPR Port is not enabled by default. To enable it, use this procedure:

- In Control Panel, select Add/Remove Programs, then Windows Components.

- Select Other Network File and Print Services, then click the Details button.

Figure 23: Adding LPD/LPR Port (Win 2000)

- Enable Print Services for Unix, and click OK.

- Click Next and complete the Wizard.

Adding the Printer

- Open your Printers folder, and start the Add Printer Wizard.

- When prompted, select Local Printer.

- On the Select the Printer Port screen, select LPR Port, as shown below. Click Next to continue.

Figure 24: Windows 2000: Select Port

- In the Dialog requesting Name or Address of server providing lpd, enter the IP address of the LevelOne FBR-1800TX Broadband Router.

- For Name of printer or print queue on that server, enter L1

- Click OK, and then Next, and continue the Wizard.

- At the Select Sharing screen, select the Radio Button for Share As, and enter the shared printer name. The shared name is how other users will see this printer. You should advise client PCs of the Server name and this printer name.

- Complete the Add Printer wizard.

### Client PC Setup for LPD/LPR Printing

After configuring the Windows Server, client PCs on the LAN can install the new printer.

The following procedure is for Windows 95/98/ME, Windows NT4.0, and Windows 2000 workstation.

- Open your Printers folder, and start the Add Printer Wizard.

- When prompted, select Network Printer

- When prompted for Network Path or Queue Name, click the Browse button, and locate the Server and Printer which your Network Administrator advised you to use.

- Click OK, then Next.

- Select the correct printer Manufacturer and Model, as advised by your Network Administrator, and click Next.

- Follow the prompts to complete the Wizard.

- The new printer will be listed with any other installed printers, and may be selected when printing from any Windows application.

## Macintosh Clients

From your Macintosh, you can access the Internet via the LevelOne FBR-1800TX Broadband Router. The procedure is as follows.

- Open the TCP/IP Control Panel.

- Select Ethernet from the Connect via pop-up menu.

- Select Using DHCP Server from the Configure pop-up menu. The DHCP Client ID field can be left blank.

- Close the TCP/IP panel, saving your settings.

Note:

If using manually assigned IP addresses instead of DHCP, the required changes are:

- Set the Router Address field to the LevelOne FBR-1800TX Broadband Router's IP Address.

- Ensure your DNS settings are correct.

## Linux Clients

To access the Internet via the LevelOne FBR-1800TX Broadband Router, it is only necessary to set the LevelOne FBR-1800TX Broadband Router as the "Gateway".

Ensure you are logged in as "root" before attempting any changes.

Fixed IP Address

By default, most Unix installations use a fixed IP Address. If you wish to continue using a fixed IP Address, make the following changes to your configuration.

- Set your "Default Gateway" to the IP Address of the LevelOne FBR-1800TX Broadband Router.

- Ensure your DNS (Name server) settings are correct.

To act as a DHCP Client (recommended)

The procedure below may vary according to your version of Linux and X -windows shell.

- Start your X Windows client.

- Select Control Panel - Network

- Select the "Interface" entry for your Network card. Normally, this will be called "eth0".

- Click the Edit button, set the "protocol" to "DHCP", and save this data.

- To apply your changes

- Use the "Deactivate" and "Activate" buttons, if available.

- OR, restart your system.

### Printing Setup on Linux

The LevelOne FBR-1800TX Broadband Router supports LPD Printing on Linux.

- The LevelOne FBR-1800TX Broadband Router supports 3 "Logical Printers" under LPD. To configure the "Logical Printers" on the LevelOne FBR-1800TX Broadband Router, refer to Printer Port in Chapter 7.

- The procedure to install a LPD printer is detailed below, but may vary according to your version of Linux and X -windows shell.

- In your X Windows shell, select Control Panel, then Printer Configuration.

- Select Add. For the printer type, select Remote Unix (lpd) Queue.

- Use the following data to complete the resulting dialog.

| Field | Data | Example |
| --- | --- | --- |
| Name | Enter a name for this printer | gw_prn |
| Spool Directory | /var/spool/lpd/printer_name Where printer_name is the "Name" entry above. | /var/spool/lpd/gw_prn |
| File Limit | Enter a suitable number. | 0 (no limit) |
| Remote Host | LevelOne FBR-1800TX Broadband Router's IP address | 192.168.0.1 |
|  | Note: If you have made a host file entry, you can use the name from the host file instead of the IP Address. |  |
| Remote Queue | Ln Where n is the Logical Printer number (L1, L2, L3). Logical Printers can be configured on the LevelOne FBR-1800TX Broadband Router's Options- Printer Port screen. | L1 |

- Save this data, and exit the Printer Configuration. Configuration is now completed, and the printer is now available for use.

## Other Unix Systems

To access the Internet via the LevelOne FBR-1800TX Broadband Router:

- Ensure the "Gateway" field for your network card is set to the IP Address of the LevelOne FBR-1800TX Broadband Router.

- Ensure your DNS (Name Server) settings are correct.

### Printing Setup

To use LPD printing to the LevelOne FBR-1800TX Broadband Router's printer, install an LPD printer using the standard procedure for your system.

- Use the LevelOne FBR-1800TX Broadband Router's IP Address as the location of the remote host

- Use L1, L2, or L3 for the name of the printer on the remote host.

On the LevelOne FBR-1800TX Broadband Router, the logical printers (L1, L2, and L3) can be configured on the Advanced - Printer Port screen. See Printer Port in Chapter 7 for details.

Chapter 5
Operation and Status

5

This Chapter details the operation of the LevelOne FBR-1800TX Broadband Router and the status screens.

## Operation

Once both the LevelOne FBR-1800TX Broadband Router and the PCs are configured, operation is automatic.

However, there are some situations where additional Internet configuration may be required:

- If using Internet-based Communication Applications, it may be necessary to specify which PC receives an incoming connection. Refer to Chapter 6 - Advanced Features for further details.

- Applications which use non-standard connections or port numbers may be blocked by the LevelOne FBR-1800TX Broadband Router's built-in firewall. You can define such applications as Special Applications to allow them to function normally. Refer to Chapter 6 - Advanced Features for further details.

- Some non-standard applications may require use of the DMZ feature. Refer to Chapter 6 - Advanced Features for further details.

## Status Screen

Use the Status link on the main menu to view this screen.

Figure 25: Status Screen

#### Data - Status Screen

| Internet |  |
| --- | --- |
| Connection Method | This indicates the current connection method, as set in the Setup Wizard. |
| Broadband Modem | This shows the connection status of the modem. |
| Internet Connection | Current connection status: - Active - Idle - Unknown - Failed If there is an error, you can click the "Connection Details" button to find out more information. |
| Internet IP Address | This IP Address is allocated by the ISP (Internet Service Provider). |
| "Connection Details" Button | Click this button to open a sub-window and view a detailed description of the current connection. Depending on the type of connection, a "log" may also be available. |
| LAN |  |
| IP Address | The IP Address of the LevelOne FBR-1800TX Broadband Router. |
| Network Mask | The Network Mask (Subnet Mask) for the IP Address above. |
| DHCP Server | This shows the status of the DHCP Server function - either "Enabled" or "Disabled". For additional information about the PCs on your LAN, and the IP addresses allocated to them, use the PC Database option on the Advanced menu. |
| System |  |
| Device Name | This displays the current name of the LevelOne FBR-1800TX Broadband Router. |
| Firmware Version | The current version of the firmware installed in the LevelOne FBR-1800TX Broadband Router. |
| "System Data" Button | Clicking this button will open a Window which lists all system details and settings. |
| Printer |  |
| Printer Status | This indicates the current status of the printer. Possible values are: - Idle - Printing - Off-line - Out of paper |
| "Abort Current Print Job" Button | Click this button to terminate the current print job. This button should be used if the current print job is not printing correctly. |
| Buttons |  |
| Connection Details | View the details of the current Internet connection. The sub-screen displayed will depend on the connection method used. See the following sections for details of each sub-screen. |
| System Data | Display all system information in a sub-window. |
| "Abort Current Print Job" Button | Click this button to terminate the current print job. This button should be used if the current print job is not printing correctly. |
| Refresh Screen | Update the data displayed on screen. |

## Connection Status - PPPoE

If using PPPoE (PPP over Ethernet), a screen like the following example will be displayed when the "Connection Details" button is clicked.

Figure 26: PPPoE Status Screen

#### Data - PPPoE

| Connection |  |
| --- | --- |
| Physical Address | The hardware address of this device, as seen by remote devices on the Internet. (This is different to the hardware address seen by devices on the local LAN.) |
| IP Address | The IP Address of this device, as seen by Internet users. This address is allocated by your ISP (Internet Service Provider). |
| Network Mask | The Network Mask associated with the IP Address above. |
| PPPoE Link Status | This indicates whether or not the connection is currently established. - If the connection does not exist, the "Connect" button can be used to establish a connection. - If the connection currently exists, the "Disconnect" button can be used to break the connection. |
| Connection Log |  |
| Connection Log | - The Connection Log shows status messages relating to the existing connection. - The most common messages are listed in the table below. - The "Clear Log" button will restart the Log, while the Refresh button will update the messages shown on screen. |
| Buttons |  |
| Connect | If not connected, establish a connection to your ISP. |
| Disconnect | If connected to your ISP, hang up the connection. |
| Clear Log | Delete all data currently in the Log. This will make it easier to read new messages. |
| Refresh | Update the data on screen. |

#### Connection Log Messages

| Message | Description |
| --- | --- |
| Connect on Demand | Connection attempt has been triggered by the "Connect automatically, as required" setting. |
| Manual connection | Connection attempt started by the "Connect" button. |
| Reset physical connection | Preparing line for connection attempt. |
| Connecting to remote server | Attempting to connect to the ISP's server. |
| Remote Server located | ISP's Server has responded to connection attempt. |
| Start PPP | Attempting to login to ISP's Server and establish a PPP connection. |
| PPP up successfully | Able to login to ISP's Server and establish a PPP connection. |
| Idle time-out reached | The connection has been idle for the time period specified in the "Idle Time-out" field. The connection will now be terminated. |
| Disconnecting | The current connection is being terminated, due to either the "Idle Time-out" above, or "Disconnect" button being clicked. |
| Error: Remote Server not found | ISP's Server did not respond. This could be a Server problem, or a problem with the link to the Server. |
| Error: PPP Connection failed | Unable to establish a PPP connection with the ISP's Server. This could be a login problem (name or password) or a Server problem. |
| Error: Connection to Server lost | The existing connection has been lost. This could be caused by a power failure, a link failure, or Server failure. |
| Error: Invalid or unknown packet type | The data received from the ISP's Server could not be processed. This could be caused by data corruption (from a bad link), or the Server using a protocol which is not supported by this device. |

## Connection Status - PPTP

If using PPTP (Peer-to-Peer Tunneling Protocol), a screen like the following example will be displayed when the "Connection Details" button is clicked.

Figure 27: PPTP Status Screen

#### Data - PPTP

| Connection |  |
| --- | --- |
| Physical Address | The hardware address of this device, as seen by remote devices on the Internet. (This is different to the hardware address seen by devices on the local LAN.) |
| IP Address | The IP Address of this device, as seen by Internet users. This address is allocated by your ISP (Internet Service Provider). |
| PPTP Status | This indicates whether or not the connection is currently established. - If the connection does not exist, the "Connect" button can be used to establish a connection. - If the connection currently exists, the "Disconnect" button can be used to break the connection. |
| Connection Log |  |
| Connection Log | - The Connection Log shows status messages relating to the existing connection. - The "Clear Log" button will restart the Log, while the Refresh button will update the messages shown on screen. |
| Buttons |  |
| Connect | If not connected, establish a connection to your ISP. |
| Disconnect | If connected to your ISP, hang up the connection. |
| Clear Log | Delete all data currently in the Log. This will make it easier to read new messages. |
| Refresh | Update the data on screen. |

## Connection Status - Telstra Big Pond

An example screen is shown below.

Figure 28: Telstra Big Pond Status Screen

#### Data - Big Pond

| Connection |  |
| --- | --- |
| Physical Address | The hardware address of this device, as seen by remote devices. (This is different to the hardware address seen by devices on the local LAN.) |
| IP Address | The IP Address of this device, as seen by Internet users. This address is allocated by your ISP (Internet Service Provider). |
| Connection Status | This indicates whether or not the connection is currently established. - If the connection does not exist, the "Connect" button can be used to establish a connection. - If the connection currently exists, the "Disconnect" button can be used to break the connection. - Normally, it is not necessary to use the Connect and Disconnect buttons unless the setting "Connect automatically, as required" is disabled. |
| Connection Log |  |
| Connection Log | - The Connection Log shows status messages relating to the existing connection. - The Clear Log button will restart the Log, while the Refresh button will update the messages shown on screen. |
| Buttons |  |
| Connect | If not connected, establish a connection to Telstra Big Pond. |
| Disconnect | If connected to Telstra Big Pond, terminate the connection. |
| Clear Log | Delete all data currently in the Log. This will make it easier to read new messages. |
| Refresh | Update the data on screen. |

## Connection Details - SingTel RAS

If using the SingTel RAS access method, a screen like the following example will be displayed when the "Connection Details" button is clicked.

Figure 29: Connection Details - RAS

#### Data - RAS

| Internet |  |
| --- | --- |
| RAS Plan | The RAS Plan which is currently used. |
| Physical Address | The hardware address of this device, as seen by remote devices on the Internet. (This is different to the hardware address seen by devices on the local LAN.) |
| IP Address | The IP Address of this device, as seen by Internet users. This address is allocated by your ISP (Internet Service Provider). |
| Network Mask | The Network Mask associated with the IP Address above. |
| Default Gateway | The IP Address of the remote Gateway or Router associated with the IP Address above. |
| DNS IP Address | The IP Address of the Domain Name Server which is currently used. |
| DHCP Client | This will show "Enabled" or "Disabled", depending on whether or not this device is functioning as a DHCP client. If "Enabled" the "Remaining lease time" field indicates when the IP Address allocated by the DHCP Server will expire. The lease is automatically renewed on expiry; use the "Renew" button if you wish to manually renew the lease immediately. |
| Buttons |  |
| Release/Renew Button will display EITHER "Release" OR "Renew" | This button is only useful if the IP address shown above is allocated automatically on connection. (Dynamic IP address). If you have a Fixed (Static) IP address, this button has no effect. - If the ISP's DHCP Server has NOT allocated an IP Address for the LevelOne FBR-1800TX Broadband Router, this button will say "Renew". Clicking the "Renew" button will attempt to re-establish the connection and obtain an IP Address from the ISP's DHCP Server. - If an IP Address has been allocated to the LevelOne FBR-1800TX Broadband Router (by the ISP's DHCP Server), this button will say "Release". Clicking the "Release" button will break the connection and release the IP Address. |
| Refresh | Update the data shown on screen. |

## Connection Details - Fixed/Dynamic IP Address

If your access method is "Direct" (no login), a screen like the following example will be displayed when the "Connection Details" button is clicked.

Figure 30: Connection Details - Fixed/Dynamic IP Address

#### Data - Fixed/Dynamic IP address

| Internet |  |
| --- | --- |
| Physical Address | The hardware address of this device, as seen by remote devices on the Internet. (This is different to the hardware address seen by devices on the local LAN.) |
| IP Address | The IP Address of this device, as seen by Internet users. This address is allocated by your ISP (Internet Service Provider). |
| Network Mask | The Network Mask associated with the IP Address above. |
| Default Gateway | The IP Address of the remote Gateway or Router associated with the IP Address above. |
| DNS IP Address | The IP Address of the Domain Name Server which is currently used. |
| DHCP Client | This will show "Enabled" or "Disabled", depending on whether or not this device is functioning as a DHCP client. If "Enabled" the "Remaining lease time" field indicates when the IP Address allocated by the DHCP Server will expire. The lease is automatically renewed on expiry; use the "Renew" button if you wish to manually renew the lease immediately. |
| Buttons |  |
| Release/Renew Button will display EITHER "Release" OR "Renew" | This button is only useful if the IP address shown above is allocated automatically on connection. (Dynamic IP address). If you have a Fixed (Static) IP address, this button has no effect. - If the ISP's DHCP Server has NOT allocated an IP Address for the LevelOne FBR-1800TX Broadband Router, this button will say "Renew". Clicking the "Renew" button will attempt to re-establish the connection and obtain an IP Address from the ISP's DHCP Server. - If an IP Address has been allocated to the LevelOne FBR-1800TX Broadband Router (by the ISP's DHCP Server), this button will say "Release". Clicking the "Release" button will break the connection and release the IP Address. |
| Refresh | Update the data shown on screen. |

Chapter 6
Advanced Features

6

This Chapter explains when and how to use the LevelOne FBR-1800TX Broadband Router's "Advanced" Features.

## Overview

The following advanced features are provided.

- Advanced Internet

- Communication Applications

- Special Applications

- DMZ

- URL filter

- Access Control

- Remote Management

- Virtual Servers

- Dynamic DNS

- Firmware Upgrade

## Advanced Menu Screen

This screen provides access to the advanced features. An example screen is shown below.

Figure 31: Advanced Menu

## Advanced Internet Screen

This screen allows configuration of all advanced features relating to Internet access.

- Communication Applications

- Special Applications

- DMZ

- URL filter

An example screen is shown below.

Figure 32: Internet Screen

### Communication Applications

Most applications are supported transparently by the LevelOne FBR-1800TX Broadband Router. But sometimes it is not clear which PC should receive an incoming connection. This problem could arise with the  Communication Applications listed on this screen.

If this problem arises, you can use this screen to set which PC should receive an incoming connection, as described below.

| Communication Applications |  |
| --- | --- |
| Select an Application | This lists applications which may generate incoming connections, where the destination PC (on your local LAN) is unknown. |
| Send incoming calls to | This lists the PCs on your LAN. - If necessary, you can add PCs manually, using the "PC Database" option on the advanced menu. - For each application listed above, you can choose a destination PC. - There is no need to "Save" after each change; you can set the destination PC for each application, then click "Save". |

### Special Applications

If you use Internet applications which use non-standard connections or port numbers, you may find that they do not function correctly because they are blocked by the LevelOne FBR-1800TX Broadband Router's firewall. In this case, you can define the application as a "Special Application".

### Special Applications Screen

This screen can be reached by clicking the Special Applications button on the Internet screen.

You can then define your Special Applications. You will need detailed information about the application; this is normally available from the supplier of the application.

Also, note that the terms "Incoming" and "Outgoing" on this screen refer to traffic from the client (PC) viewpoint

Figure 33: Special Applications Screen

#### Data - Special Applications Screen

| Checkbox | Use this to Enable or Disable this Special Application as required. |
| --- | --- |
| Name | Enter a descriptive name to identify this Special Application. |
| Incoming Ports | - Type - Select the protocol (TCP or UDP) used when you receive data from the special application or service. (Note: Some applications use different protocols for outgoing and incoming data). - Start - Enter the beginning of the range of port numbers used by the application server, for data you receive. If the application uses a single port number, enter it in both the "Start" and "Finish" fields. - Finish - Enter the end of the range of port numbers used by the application server, for data you receive. |
| Outgoing Ports | - Type - Select the protocol (TCP or UDP) used when you send data to the remote system or service. - Start - Enter the beginning of the range of port numbers used by the application server, for data you send to it. If the application uses a single port number, enter it in both the "Start" and "Finish" fields. - Finish - Enter the end of the range of port numbers used by the application server, for data you send to it. If the application uses a single port number, enter it in both the "Start" and "Finish" fields. |

### Using a Special Application

- Configure the Special Applications screen as required.

- On your PC, use the application normally. Remember that only one (1) PC can use each Special application at any time. Also, when 1 PC is finished using a particular Special Application, there may need to be a "Time-out" before another PC can use the same Special Application. The "Time-out" period may be up to 3 minutes.

|  | If an application still cannot function correctly, try using the "DMZ" feature. |
| --- | --- |

### DMZ

This feature, if enabled, allows one (1) computer on your LAN to be exposed to all users on the Internet, allowing unrestricted 2-way communication between the "DMZ PC" and other Internet users or Servers.

- This allows almost any application to be used on the "DMZ PC".

- The "DMZ PC" will receive all "Unknown" connections and data.

- If the DMZ feature is enabled, you must select the PC to be used as the "DMZ PC".

- The DMZ feature can be Enabled and Disabled on the Advanced Internet screen.

|  | The "DMZ PC" is effectively outside the Firewall, making it more vulnerable to attacks. For this reason, you should only enable the DMZ feature when required. |
| --- | --- |

### URL Filter

The URL Filter allows you to block access to undesirable Web site

- To use this feature, you must define "filter strings". If the "filter string" appears in a requested URL, the request is blocked.

- Enabling the URL Filter also affects the Internet Access Log. If Enabled, the "Destination" field in the log will display the URL. Otherwise, it will display the IP Address.

- The URL Filter can be Enabled or Disabled on the Advanced Internet screen.

### URL Filter Screen

Click the "Configure URL Filter" button on the Internet screen to access the URL Filter screen. An example screen is shown below.

Figure 34: URL Filter Screen

#### Data - URL Filter Screen

| Filter Strings |  |
| --- | --- |
| Current Entries | This lists any existing entries. If you have not entered any values, this list will be empty. |
| Add Filter String | To add an entry to the list, enter it here, and click the "Add" button. An entry may be a Domain name (e.g. www.trash.com) or simply a string. (e.g. ads/ ) Any URL which contains ANY entry ANYWHERE in the URL will be blocked. |
| Buttons |  |
| Delete/Delete All | Use these buttons to delete the selected entry or all entries, as required. Multiple entries can be selected by holding down the CTRL key while selecting.(On the Macintosh, hold the SHIFT key while selecting.) |
| Add | Use this to add the current Filter String to the site list. |

## Access Control

This feature is accessed by the Access Control link on the Advanced menu.

### Overview

The Access Control feature allows administrators to restrict the level of Internet Access available to PCs on your LAN. With the default settings, everyone has unrestricted Internet access.

To use this feature:

- Set the desired restrictions on the "Default" group. All PCs are in the "Default" group unless explicitly moved to another group.

- Set the desired restrictions on the other groups ("Group 1", "Group 2", "Group 3" and "Group 4") as needed.

- Assign PC to the groups as required.

|  | Restrictions are imposed by blocking "Services", or types of connections. All common Services are pre-defined. If required, you can also define your own Services. |
| --- | --- |

### Access Control Screen

To view this screen, select the Access Control link on the Advanced menu.

Figure 35: Access Control Screen

#### Data - Access Control Screen

| Group |  |
| --- | --- |
| Group | Select the desired Group. The screen will update to display the settings for the selected Group. Groups are named "Default", "Group 1", "Group 2", "Group 3" and "Group 4", and cannot be re-named. |
| "Members" Button | Click this button to add or remove members from the current Group. - If the current group is "Default", then members can not be added or deleted. This group contains PCs not allocated to any other group. - To remove PCs from the Default Group, assign them to another Group. - To assign PCs to the Default Group, delete them from the Group they are currently in. See the following section for details of the Group Members screen. |
| Internet Access |  |
| Restrictions | Select the desired options for the current group: - None - Nothing is blocked. Use this to create the least restrictive group. - Block all Internet access - All traffic via the WAN port is blocked. Use this to create the most restrictive group. - Block selected Services - You can select which Services are to block. Use this to gain fine control over the Internet access for a group. |
| Block by Schedule | If Internet access is being blocked, you can choose to apply the blocking only during scheduled times. (If access is not blocked, no Scheduling is possible, and this setting has no effect.) |
| Define Schedule Button | Clicking this will open a sub-window where you can define or modify the Schedule. |
| Services | This lists all defined Services. Select the Services you wish to block. To select multiple services, hold the CTRL key while selecting. (On the Macintosh, hold the SHIFT key rather than CTRL.) |
| Edit Service List Button | If you wish to define additional Services, or manage the Service list, click this button to open the "Services" screen. |
| Buttons |  |
| Members | Click this button to add or remove members from the current Group. If the current group is "Default", then members can not be added or deleted. This group contains PCs not allocated to any other group. See the following section for details of the Group Members screen. |
| Define Schedule | Click this to open a sub-window where you can define or modify the Schedule. |
| Edit Service List | If you wish to define additional Services, or manage the Service list, click this button to open the "Services" screen. |
| Save | Save the data on screen. |
| Cancel | Reverse any changes made since the last "Save". |
| View Log | Click this to open a sub-window where you can view the "Access Control" log. This log shows attempted Internet accesses which have been blocked by the Access Control feature. |
| Clear Log | Click this to clear and restart the "Access Control" log, making new entries easier to read. |

### Group Members Screen

This screen is displayed when the Members button on the Access Control screen is clicked.

Figure 36: Group Members

Use this screen to add or remove members (PCs) from the current group.

- The "Del >>" button will remove the selected PC (in the Members list) from the current group.

- The "<< Add" button will add the selected PC (in the Other PCs list) to the current group.

|  | PCs not assigned to any group will be in the "Default" group. PCs deleted from any other Group will be added to the "Default" group. |
| --- | --- |

### Default Schedule Screen

This screen is displayed when the Define Schedule button on the Access Control screen is clicked.

- This schedule can be (optionally) applied to any Access Control Group.

- Blocking will be performed during the scheduled time (between the "Start" and "Finish" times.)

- Two (2) separate sessions or periods can be defined.

- Times must be entered using a 24 hr clock.

- If the time for a particular day is blank, no action will be performed.

Figure 37: Default Schedule Screen

#### Data - Default Schedule Screen

| Day | Each day of the week can scheduled independently. |
| --- | --- |
| Session 1 Session 2 | Two (2) separate sessions or periods can be defined. Session 2 can be left blank if not required. |
| Start Time | Enter the start using a 24 hr clock. |
| Finish Time | Enter the finish time using a 24 hr clock. |

### Services Screen

This screen is displayed when the Edit Service List button on the Access Control screen is clicked.

Figure 38: Access Control - Services

#### Data - Services Screen

| Available Services |  |
| --- | --- |
| Available Services | This lists all the available services. |
| "Delete" button | Use this to delete any Service you have added. Pre-defined Services can not be deleted. |
| Add New Service |  |
| Name | Enter a descriptive name to identify this service. |
| Type | Select the protocol (TCP, UDP, ICMP) used to the remote system or service. |
| Start Port | For TCP and UDP Services, enter the beginning of the range of port numbers used by the service. If the service uses a single port number, enter it in both the "Start" and "Finish" fields. |
| Finish Port | For TCP and UDP Services, enter the end of the range of port numbers used by the service. If the service uses a single port number, enter it in both the "Start" and "Finish" fields. |
| ICMP Type | For ICMP Services, enter the type number of the required service. |
| Buttons |  |
| Delete | Delete the selected service from the list. |
| Save | Add a new entry to the Service list, using the data shown in the "Add New Service" area on screen. |
| Cancel | Clear the " Add New Service " area, ready for entering data for a new Service. |

### Access Control Log

To check the operation of the Access Control feature, an Access Control Log is provided. Click the View Log button on the Access Control screen to view this log.

This log shows attempted Internet accesses which have been blocked by the Access Control function.

Data shown in this log is as follows:

| Date/Time | Date and Time of the attempted access. |
| --- | --- |
| Name | If known, the name of the PC whose access was blocked. This name is taken from the Network Clients database |
| Source IP address | The IP Address of the PC or device whose access request was blocked |
| MAC address | The hardware or physical address of the PC or device whose access request was blocked |
| Destination | The destination URL or IP address |

## Remote Management

This feature allows you to manage the LevelOne FBR-1800TX Broadband Router via the Internet.

Figure 39: Remote Management Screen

#### Data - Remote Management Screen

| Remote Management |  |
| --- | --- |
| Enable Remote Management | Enable to allow management via the Internet. If Disabled, this device will ignore management connection attempts from the Internet. |
| Port Number | Enter a port number between 1024 and 65535 (8080 is recommended). This port number must be specified when you connect (see below). Note: The default port number for HTTP (Web) connections is port 80, but using port 80 here will prevent the use of a Web "Virtual Server" on your LAN. (See Advanced Internet - Virtual Servers) |
| Current IP Address | You must use this IP Address to connect (see below). This IP Address is allocated by your ISP. But if using a Dynamic IP Address, this value can change each time you connect to your ISP. So it is better if your ISP allocates you a Fixed IP Address. |

To connect from a remote PC via the Internet

- Ensure your Internet connection is established, and start your Web Browser.

- In the "Address" bar, enter "HTTP://" followed by the Internet IP Address of the LevelOne FBR-1800TX Broadband Router. If the port number is not 80, the port number is also required. (After the IP Address, enter ":" followed by the port number.) 
e.g.

HTTP://123.123.123.123:8080

This example assumes the WAN IP Address is 123.123.123.123, and the port number is 8080.

## Virtual Servers

This feature allows you to make Servers on your LAN accessible to Internet users. Normally, Internet users would not be able to access a server on your LAN because:

- Your Server does not have a valid external IP Address.

- Attempts to connect to devices on your LAN are blocked by the firewall in this device.

The "Virtual Server" feature solves these problems and allows Internet users to connect to your servers, as illustrated below.

Figure 40: Virtual Servers

### IP Address seen by Internet Users

Note that, in this illustration, both Internet users are connecting to the same IP Address, but using different protocols.

To Internet users, all virtual Servers on your LAN have the same IP Address. This IP Address is allocated by your ISP.

This address should be static, rather than dynamic, to make it easier for Internet users to connect to your Servers.

However, you can use the DDNS (Dynamic DNS) feature to allow users to connect to your Virtual Servers using a URL, instead of an IP Address.

### Virtual Servers Screen

The Virtual Servers screen is reached by the Virtual Servers link on the Advanced screen. An example screen is shown below.

Figure 41: Virtual Servers Screen

This screen lists a number of pre-defined Servers, and allows you to define your own Servers. Details of the selected Server are shown in the "Properties" area.

#### Data - Virtual Servers Screen

| Servers |  |
| --- | --- |
| Servers | This lists a number of pre-defined Servers, plus any Servers you have defined. Details of the selected Server are shown in the "Properties" area. |
| Properties |  |
| Enable | Use this to Enable or Disable support for this Server, as required. - If Enabled, any incoming connections will be forwarded to the selected PC. - If Disabled, any incoming connection attempts will be blocked. |
| PC (Server) | Select the PC for this Server. The PC must be running the appropriate Server software. |
| Protocol | Select the protocol (TCP or UDP) used by the Server. |
| Internal Port No. | Enter the port number which the Server software is configured to use. |
| External Port No. | The port number used by Internet users when connecting to the Server. This is normally the same as the Internal Port Number. If it is different, this device will perform a "mapping" or "translation" function, allowing the server to use one port address, while clients use a different port address. |
| Buttons |  |
| Defaults | This will delete any Servers you have defined, and set the pre-defined Servers to use their default port numbers. |
| Disable All | This will cause the "Enable" setting of all Virtual Servers to be set OFF. |
| Update Selected Server | Update the current Virtual Server entry, using the data shown in the "Properties" area on screen. |
| Add as new Server | Add a new entry to the Virtual Server list, using the data shown in the "Properties" area on screen. The entry selected in the list is ignored, and has no effect. |
| Delete | Delete the current Virtual Server entry. Note that the pre-defined Servers can not be deleted. Only Servers you have defined yourself can be deleted. |
| Clear Form | Clear all data from the "Properties" area, ready for input of a new Virtual Server entry. |

|  | For each entry, the PC must be running the appropriate Server software. |
| --- | --- |

### Defining your own Virtual Servers

If the type of Server you wish to use is not listed on the Virtual Servers screen, you can define and manage your own Servers:

| Create a new Server: | - Click "Clear Form" - Enter the required data, as described above. - Click "Add". - The new Server will now appear in the list. |
| --- | --- |
| Modify (Edit) a Server: | - Select the desired Server from the list - Make any desired changes (for example, change the Enable/Disable setting). - Click "Update" to save changes to the selected Server. |
| Delete a Server: | - Select the entry from the list. - Click "Delete". Note: You can only delete Servers you have defined. Pre-defined Server cannot be deleted. |

|  | From the Internet, ALL Virtual Servers have the IP Address allocated by your ISP. |
| --- | --- |

### Connecting to the Virtual Servers

Once configured, anyone on the Internet can connect to your Virtual Servers. They must use the Internet IP Address (the IP Address allocated to you by your ISP). 
e.g.

http://203.70.212.52

ftp://203.70.212.52

It is more convenient if you are using a Fixed IP Address from your ISP, rather than Dynamic.  However, you can use the Dynamic DNS feature, described in the following section, to allow users to connect to your Virtual Servers using a URL, rather than an IP Address.

## Dynamic DNS (Domain Name Server)

This free service is very useful when combined with the Virtual Server feature. It allows Internet users to connect to your Virtual Servers using a URL, rather than an IP Address.

This also solves the problem of having a dynamic IP address. With a dynamic IP address, your IP address may change whenever you connect, which makes it difficult to connect to you.

The Service works as follows:

- You must register for the service at http://www.dyndns.org (Registration is free). Your password will be E-mailed to you.

- After registration, use the "Create New Host" option (at www.dyndns.org) to request your desired Domain name.

- Enter your data from www.dyndns.org in the LevelOne FBR-1800TX Broadband Router's DDNS screen.

- The LevelOne FBR-1800TX Broadband Router will then automatically ensure that your current IP Address is recorded at http://www.dyndns.org

- From the Internet, users will be able to connect to your Virtual Servers (or DMZ PC) using your Domain name, as shown on this screen.

### Dynamic DNS Screen

Select Advanced on the main menu, then Dynamic DNS, to see a screen like the following:

Figure 42: DDNS Screen

#### Data - Dynamic DNS Screen

| DDNS Service |  |
| --- | --- |
| DDNS Service | - You must sign up first to create a new account before using the service. The service is free. - Click this link to connect to the www.dyndns.org Web site. - Your initial password will be E-mailed to you; you can change this later if you wish. - After registration, use the "Create New Host" link (on the www.dyndns.org Web site) to request a domain name. |
| DDNS Data |  |
| User Name | Enter the "User name" specified at the www.dyndns.org Web site when you registered. |
| Password | Enter your current password for www.dyndns.org |
| Domain Name | - Enter your domain name, as allocated at www.dyndns.org. - The name should consist only of letters and the hyphen (dash). Using any other characters may cause problems.. |
| DDNS Status | This message is returned by the DDNS Server at www.dyndns.org - Normally, this message should be "Update successful" (current IP address was updated on the www.dyndns.org server). - If the message is "No host", this indicates the host name entered was not allocated to you. You need to connect to www.dyndns.org and correct this problem. |

## Upgrade Firmware

The firmware (software) in the LevelOne FBR-1800TX Broadband Router can be upgraded using your Web Browser.

You must first download the upgrade file, then select Upgrade on the Advanced menu. You will see a screen like the following.

Figure 43: Upgrade Firmware Screen

To perform the Firmware Upgrade:

- Click the "Browse" button and navigate to the location of the upgrade file.

- Select the upgrade file. It's name will appear in the Upgrade File field.

- Click the "Start Upgrade" button to commence the firmware upgrade.

|  | The LevelOne FBR-1800TX Broadband Router is unavailable during the upgrade process, and must restart when the upgrade is completed. Any connections to or through the LevelOne FBR-1800TX Broadband Router will be lost. |
| --- | --- |

Chapter 7
Advanced Configuration

7

This Chapter explains the settings available via the Advanced configuration section of the "Advanced" menu.

## Overview

Normally, it is not necessary to use these screens, or change any settings. These screens and settings are provided to deal with non-standard situations, or to provide additional options for advanced users.

The settings available are:

| PC Database | This is the list of PCs shown when you select the "DMZ PC" or a "Virtual Server". This database is maintained automatically, but you can add and delete entries for PCs which use a Fixed (Static) IP Address. |
| --- | --- |
| Options | Various options, such as backup DNS, UPnP, and enable TFTP firmware upgrade option. |
| Security | Firewall and other security-related settings. |
| Logs | View or clear all logs, set E-Mailing of log files. |
| Printer Port | Configure Logical Printers for use under Unix |
| MAC Address | Set the MAC (hardware) address used on the Internet (WAN port) interface. This is required only if your ISP has recorded the MAC (hardware) address you should use. |
| Routing | Only required if your LAN has other Routers or Gateways. |

## PC Database

The PC Database is used whenever you need to select a PC (e.g. for the "DMZ" PC). It eliminates the need to enter IP addresses. Also, you do not need to use fixed IP addresses on your LAN.

### PC Database Screen

An example PC Database screen is shown below.

- Figure 44: PC Database

- PCs which are "DHCP Clients" are automatically added to the database, and updated as required.

- By default, non-Server versions of Windows act as "DHCP Clients"; this setting is called "Obtain an IP Address automatically".

- The LevelOne FBR-1800TX Broadband Router uses the "Hardware Address" to identify each PC, not the name or IP address. The "Hardware Address" can only change if you change the PC's network card or adapter.

- This system means you do NOT need to use Fixed (static) IP addresses on your LAN. However, you can add PCs using Fixed (static) IP Addresses to the PC database if required.

#### Data - PC Database Screen

| Known PCs | This lists all current entries. Data displayed is name (IP Address) type. The "type" indicates whether the PC is connected to the LAN. |
| --- | --- |
| Name | If adding a new PC to the list, enter its name here. It is best if this matches the PC's "hostname". |
| IP Address | Enter the IP Address of the PC. The PC will be sent a "ping" to determine its hardware address. If the PC is not available (not connected, or not powered On) you will not be able to add it. |
| Buttons |  |
| Add | This will add the new PC to the list. The PC will be sent a "ping" to determine its hardware address. If the PC is not available (not connected, or not powered On) you will not be able to add it. |
| Delete | Delete the selected PC from the list. This should be done in 2 situations: - The PC has been removed from your LAN. - The entry is incorrect. |
| Refresh | Update the data on screen. |
| Generate Report | Display a read-only list showing full details of all entries in the PC database. |
| Advanced Administration | View the Advanced version of the PC database screen. See below for details. |

### PC Database (Admin)

This screen is displayed if the "Advanced Administration" button on the PC Database is clicked. It provides more control than the standard PC Database screen.

Figure 45: PC Database (Admin)

#### Data - PC Database ( Admin) Screen

| Known PCs | This lists all current entries. Data displayed is name (IP Address) type. The "type" indicates whether the PC is connected to the LAN. |
| --- | --- |
| PC Properties |  |
| Name | If adding a new PC to the list, enter its name here. It is best if this matches the PC's "hostname". |
| IP Address | Select the appropriate option: - Automatic - The PC is set to be a DHCP client (Windows: "Obtain an IP address automatically"). The LevelOne FBR-1800TX Broadband Router will allocate an IP address to this PC when requested to do so. The IP address could change, but normally won't. - DCHP Client - Reserved IP Address - Select this if the PC is set to be a DCHP client, and you wish to guarantee that the LevelOne FBR-1800TX Broadband Router will always allocate the same IP Address to this PC. Enter the required IP address. Only the last field is required; the other fields must match the LevelOne FBR-1800TX Broadband Router's IP address. - Fixed IP Address - Select this if the PC is using a Fixed (Static) IP address. Enter the IP address allocated to the PC. (The PC must be configured to use this IP address.) |
| MAC Address | Select the appropriate option - Automatic discovery - Select this to have the LevelOne FBR-1800TX Broadband Router contact the PC and find its MAC address. This is only possible if the PC is connected to the LAN and powered On. - MAC is - Enter the MAC address on the PC. The MAC address is also called the "Hardware Address", "Physical Address", or "Network Adapter Address". The LevelOne FBR-1800TX Broadband Router uses this to provide a unique identifier for each PC. Because of this, the MAC address can NOT be left blank. |
| Buttons |  |
| Add as New Entry | Add a new PC to the list, using the data in the "Properties" box. If "Automatic discovery" (for MAC address) is selected, the PC will be sent a "ping" to determine its hardware address. This will fail unless the PC is connected to the LAN, and powered on. |
| Update Selected PC | Update (modify) the selected PC, using the data in the "Properties" box. |
| Clear Form | Clear the "Properties" box, ready for entering data for a new PC. |
| Refresh | Update the data on screen. |
| Generate Report | Display a read-only list showing full details of all entries in the PC database. |
| Standard Screen | Click this to view the standard "PC Database" screen. |

## Options

This screen allows advanced users to enter or change a number of settings. For normal operation, there is no need to use this screen or change any settings.

An example Options screen is shown below.

- Figure 46: Options Screen

#### Data - Options Screen

| Backup DNS |  |
| --- | --- |
| IP Address | Enter the IP Address of the DNS (Domain Name Servers) here. These DNS will be used only if the primary DNS is unavailable. |
| TFTP |  |
| Enable Firmware Upgrade using TFTP | - If enabled, TFTP (Trivial FTP) can be used to upgrade the firmware in this device. This is normally not required; a Windows utility is available for this purpose. - You must obtain the firmware upgrade file first; instructions for using TFTP will be available with the upgrade. |
| UPnP |  |
| Enable UPnP Services | - UPnP (Universal Plug and Play) allows automatic discovery and configuration of equipment attached to your LAN. UPnP is supported by Windows ME, XP, or later. - If Enabled, this device will be visible via UPnP. - If Disabled, this device will not be visible via UPnP. |
| Allow Configuration... | - If checked, then UPnP users can change the configuration. - If Disabled, UPnP users can only view the configuration. But currently, this restriction only applies to users running Windows XP, who access the Properties via UPnP. (e.g. Right - click the LevelOne FBR-1800TX Broadband Router in My Network Places, and select Properties) |
| Allow Internet access to be disabled | - If checked, then UPnP users can disable Internet access via this device. - If Disabled, UPnP users can NOT disable Internet access via this device. But currently, this restriction only applies to users running Windows XP, who access the Properties via UPnP. (e.g. Right - click the LevelOne FBR-1800TX Broadband Router in My Network Places, and select Properties) |
| MTU |  |
| MTU size | MTU (Maximum Transmission Unit) value should only be changed if advised to do so by Technical Support. - Enter a value between 1 and 1500. - This device will still auto-negotiate with the remote server, to set the MTU size. The smaller of the 2 values (auto-negotiated, or entered here) will be used. - For direct connections (not PPPoE or PPTP), the MTU used is always 1500. |

## Security

This screen allows you to set Firewall and other security-related options.

Figure 47: Security Screen

#### Data - Security Screen

| Firewall |  |
| --- | --- |
| Enable DoS Firewall | If enabled, DoS (Denial of Service) attacks will be detected and blocked. The default is enabled. It is strongly recommended that this setting be left enabled. Note: - A DoS attack does not attempt to steal data or damage your PCs, but overloads your Internet connection so you can not use it - the service is unavailable. - This device uses "Stateful Inspection" technology. This system can detect situations where individual TCP/IP packets are valid, but collectively they become a DoS attack. |
| Threshold | This setting affects the number of "half-open" connections allowed. - A "half-open" connection arises when a remote client contacts the Server with a connection request, but then does not reply to the Server's response. - While the optimum number of "half-open" connections allowed (the "Threshold") depends on many factors, the most important factor is the available bandwidth of your Internet connection. - Select the setting to match the bandwidth of your Internet connection. |
| Options |  |
| Respond to ICMP | The ICMP protocol is used by the "ping" and "traceroute" programs, and by network monitoring and diagnostic programs. - If checked, the LevelOne FBR-1800TX Broadband Router will repond to ICMP packets received from the Internet. - If not checked, ICMP packets from the Internet will be ignored. Disabling this option provides a slight increase in security. |
| Allow IPsec | The IPSec protocol is used to establish a secure connection, and is widely used by VPN (Virtual Private Networking) programs. - If checked, IPSec connections are allowed. - If not checked, IPSec connections are blocked. |
| Allow PPTP | PPTP (Point to Point Tunneling Protocol) is widely used by VPN (Virtual Private Networking) programs. - If checked, PPTP connections are allowed. - If not checked, PPTP connections are blocked. |
| Allow L2TP | L2TP is a protocol developed by Cisco for VPNs (Virtual Private Networks). - If checked, L2TP connections are allowed. - If not checked, L2TP connections are blocked. |

## Logs

The Logs record various types of activity on the LevelOne FBR-1800TX Broadband Router. This data is useful for troubleshooting, but enabling all logs will generate a large amount of data and adversely affect performance.

Since only a limited amount of log data can be stored in the LevelOne FBR-1800TX Broadband Router, log data can also be E-mailed to your PC.

Figure 48: Logs Screen

#### Data - Logs Screen

| Enable Logs |  |
| --- | --- |
| Outgoing Connections | If selected, Outgoing Internet connections are logged. Normally, the (Internet) "Destination" will be shown as an IP address. But if the "URL Filter" is enabled, the "Destination" will be shown as a URL. |
| Access Control | If enabled, the log will include attempted outgoing connections which have been blocked by the "Access Control" feature. |
| DoS Attacks | If enabled, this log will show details of DoS (Denial of Service) attacks which have been blocked by the built-in Firewall. |
| Timezone | Select the correct Timezone for your location. This is required for the date/time shown on the logs to be correct. |
| E-Mail Reports |  |
| Send E-mail alert | If enabled, an E-mail will be sent immediately if a DoS (Denial of Service) attack is detected. If enabled, the E-mail address information must be provided. |
| E-mail Logs | You can choose to have the logs E-mailed to you, by enabling either or both checkboxes. If enabled, the Log will sent to the specified E-mail address. The interval between E-mails is determined by the "Send" setting. |
| Send | Select the desired option for sending the log by E-mail. - When log is full - The time is not fixed. The log will be sent when the log is full, which will depend on the volume of traffic. - Every day, Every Monday ... - The log is sent on the interval specified. - If "Every day" is selected, the log is sent at the time specified. - If the day is specified, the log is sent once per week, on the specified day. - Select the time of day you wish the E-mail to be sent. - If the log is full before the time specified to send it, it will be sent regardless. |
| E-Mail Address |  |
| E-mail Address | Enter the E-mail address the Log is to be sent to. The E-mail will also show this address as the Sender's address. |
| Subject | Enter the text string to be shown in the "Subject" field for the E-mail. |
| SMTP Server | Enter the address or IP address of the SMTP (Simple Mail Transport Protocol) Server you use for outgoing E-mail. |
| Port No. | Enter the port number used to connect to the SMTP Server. The default value is 25. |

## MAC Address

The MAC (hardware) address is a low-level network identifier. It may be called "MAC Address", "Hardware Address", or "Physical Address". On a PC, this address is associated with the Network card or adapter. The address on the MAC Address screen is the address on the Internet (WAN port) interface, and has no effect on the LAN interface.

- If your ISP asks for the "Network Adapter Address", "Physical Address", "Hardware Address", or "MAC Address" for the PC the DSL/Cable modem is connected to, provide this value.

- If your ISP has already recorded a MAC address, you can change the address used by the LevelOne FBR-1800TX Broadband Router to match the address recorded by your ISP.

### MAC Address Screen

Select MAC Address from the Advanced menu to reach a screen like the example below.

Figure 49: MAC address Screen

#### Data - MAC address Screen

| MAC (hardware) Address | The current MAC (hardware) address is displayed. If your ISP has recorded a Hardware Address, you can "spoof" that address by entering it in the address field. The hardware address consists of 12 characters, where each character is a digit (0..9) or a character between A and F. |
| --- | --- |
| Buttons |  |
| Default | Inserts the default MAC address into the MAC address field. You must click "Save" to actually change the address used. |
| Copy from PC | Inserts the MAC address from your PC into the MAC address field. You must click "Save" to actually change the address used. |
| Save | Save your changes to the LevelOne FBR-1800TX Broadband Router. |
| Cancel | Reverse any changes made since the last "Save". |

|  | If the MAC address is changed, the LevelOne FBR-1800TX Broadband Router must restart. |
| --- | --- |

## Printer Port

These settings are only useful if using LPD printing under Unix. Normally, these settings do not need to be changed.

Figure 50: Printer Port Screen

#### Data - Printer Port Screen

| Logical Printers |  |
| --- | --- |
| No (L1, L2, L3) | Logical printer configuration is only required if using LPD printing under Linux or Unix. Otherwise, these settings are ignored There are 3 Logical Printers (Ports), named L1, L2, and L3. These names can not be changed, and must be used when selecting the logical printer from your Unix system. |
| Pre-string (HEX) | Enter (in HEX) the series of printer commands to be sent to the attached printer BEFORE each print job. |
| Post-string (HEX) | Enter (in HEX) the series of printer commands to be sent to the attached printer AFTER each print job. |
| LF to CR/LF | If checked, Unix-style LF (line feed) characters at the end of each line will be converted to CR/LF (carriage return/line feed) pairs, as used in MSDOS and Windows. |

## Routing

### Overview

- If you don't have other Routers or Gateways on your LAN, you can ignore the "Routing" page completely.

- If the LevelOne FBR-1800TX Broadband Router is only acting as a Gateway for the local LAN segment, ignore the "Routing" page even if your LAN has other Routers.

- If your LAN has a standard Router (e.g. Cisco) on your LAN, and the LevelOne FBR-1800TX Broadband Router is to act as a Gateway for all LAN segments, enable RIP (Routing Information Protocol) and ignore the Static Routing table.

- If your LAN has other Gateways and Routers, and you wish to control which LAN segments use each Gateway, do NOT enable RIP (Routing Information Protocol). Configure the Static Routing table instead. (You also need to configure the other Routers.)

- If using Windows 2000 Data center Server as a software Router, enable RIP on the LevelOne FBR-1800TX Broadband Router, and ensure the following Windows 2000 settings are correct:

- Open Routing and Remote Access

- In the console tree, select Routing and Remote Access , [server name], IP Routing, RIP

- In the "Details" pane, right-click the interface you want to configure for RIP version 2, and then click "Properties".

- On the "General" tab, set Outgoing packet protocol to "RIP version 2 broadcast", and  Incoming packet protocol to "RIP version 1 and 2".

### Routing Screen

The routing table is accessed by the Routing link on the Advanced screen.

#### Using this Screen

Generally, you will use either RIP (Routing Information Protocol) OR the Static Routing Table, as explained above, although is it possible to use both methods simultaneously.

Static Routing Table

- If RIP is not used, an entry in the routing table is required for each LAN segment on your Network, other than the segment to which this device is attached.

- The other Routers must also be configured. See Configuring Other Routers on your LAN later in this chapter for further details and an example.

Figure 51: Routing Screen

#### Data - Routing Screen

| RIP |  |
| --- | --- |
| Enable RIP | Check this to enable the RIP (Routing Information Protocol) feature of the LevelOne FBR-1800TX Broadband Router. The LevelOne FBR-1800TX Broadband Router supports RIP 1 only. |
| Static Routing |  |
| Static Routing Table Entries | This list shows all entries in the Routing Table. - The "Properties" area shows details of the selected item in the list. - Change any the properties as required, then click the "Update" button to save the changes to the selected entry. |
| Properties | - Destination Network - The network address of the remote LAN segment. For standard class "C" LANs, the network address is the first 3 fields of the Destination IP Address. The 4th (last) field can be left at 0. - Network Mask - The Network Mask for the remote LAN segment. For class "C" networks, the default mask is 255.255.255.0 - Gateway IP Address - The IP Address of the Gateway or Router which the LevelOne FBR-1800TX Broadband Router must use to communicate with the destination above. (NOT the router attached to the remote segment.) - Metric - The number of "hops" (routers) to pass through to reach the remote LAN segment. The shortest path will be used. The default value is 1. |
| Buttons |  |
| Save | Save the RIP setting. This has no effect on the Static Routing Table. |
| Add | Add a new entry to the Static Routing table, using the data shown in the "Properties" area on screen. The entry selected in the list is ignored, and has no effect. |
| Update | Update the current Static Routing Table entry, using the data shown in the "Properties" area on screen. |
| Delete | Delete the current Static Routing Table entry. |
| Clear Form | Clear all data from the "Properties" area, ready for input of a new entry for the Static Routing table. |
| Generate Report | Generate a read-only list of all entries in the Static Routing table. |

### Configuring Other Routers on your LAN

It is essential that all IP packets for devices not on the local LAN be passed to the LevelOne FBR-1800TX Broadband Router, so that they can be forwarded to the external LAN, WAN, or Internet. To achieve this, the local LAN must be configured to use the LevelOne FBR-1800TX Broadband Router as the Default Route or Default Gateway.

#### Local Router

The local router is the Router installed on the same LAN segment as the LevelOne FBR-1800TX Broadband Router. This router requires that the Default Route is the LevelOne FBR-1800TX Broadband Router itself. Typically, routers have a special entry for the Default Route. It should be configured as follows.

| Destination IP Address | Normally 0.0.0.0, but check your router documentation. |
| --- | --- |
| Network Mask | Normally 0.0.0.0, but check your router documentation. |
| Gateway IP Address | The IP Address of the LevelOne FBR-1800TX Broadband Router. |
| Metric | 1 |

#### Other Routers on the Local LAN

Other routers on the local LAN must use the LevelOne FBR-1800TX Broadband Router's Local Router as the Default Route. The entries will be the same as the LevelOne FBR-1800TX Broadband Router's local router, with the exception of the Gateway IP Address.

- For a router with a direct connection to the LevelOne FBR-1800TX Broadband Router's local Router, the Gateway IP Address is the address of the LevelOne FBR-1800TX Broadband Router's local router.

- For routers which must forward packets to another router before reaching the LevelOne FBR-1800TX Broadband Router's local router, the Gateway IP Address is the address of the intermediate router.

### Static Routing - Example

Figure 52: Routing Example

#### For the LevelOne FBR-1800TX Broadband Router's Routing Table

For the LAN shown above, with 2 routers and 3 LAN segments, the LevelOne FBR-1800TX Broadband Router requires 2 entries as follows.

| Entry 1 (Segment 1) |  |
| --- | --- |
| Destination IP Address | 192.168.1.0 |
| Network Mask | 255.255.255.0 (Standard Class C) |
| Gateway IP Address | 192.168.0.100 (LevelOne FBR-1800TX Broadband Router's local Router) |
| Metric | 2 |
| Entry 2 (Segment 2) |  |
| Destination IP Address | 192.168.2.0 |
| Network Mask | 255.255.255.0 (Standard Class C) |
| Gateway IP Address | 192.168.0.100 |
| Metric | 3 |

#### For Router A's Default Route

| Destination IP Address | 0.0.0.0 |
| --- | --- |
| Network Mask | 0.0.0.0 |
| Gateway IP Address | 192.168.0.1 (LevelOne FBR-1800TX Broadband Router's IP Address) |

#### For Router B's Default Route

| Destination IP Address | 0.0.0.0 |
| --- | --- |
| Network Mask | 0.0.0.0 |
| Gateway IP Address | 192.168.1.80 (LevelOne FBR-1800TX Broadband Router's local router) |

Appendix A
Troubleshooting

A

This Appendix covers the most likely problems and their solutions.

## Overview

This chapter covers some common problems that may be encountered while using the LevelOne FBR-1800TX Broadband Router and some possible solutions to them. If you follow the suggested steps and the LevelOne FBR-1800TX Broadband Router still does not function properly, contact your dealer for further advice.

## General Problems

| Problem 1: | Can't connect to the LevelOne FBR-1800TX Broadband Router to configure it. |
| --- | --- |
| Solution 1: | Check the following: - The LevelOne FBR-1800TX Broadband Router is properly installed, LAN connections are OK, and it is powered ON. - Ensure that your PC and the LevelOne FBR-1800TX Broadband Router are on the same network segment. (If you don't have a router, this must be the case.) - If your PC is set to "Obtain an IP Address automatically" (DHCP client), restart it. - If your PC uses a Fixed (Static) IP address, ensure that it is using an IP Address within the range 192.168.0.2 to 192.168.0.254 and thus compatible with the LevelOne FBR-1800TX Broadband Router's default IP Address of 192.168.0.1. Also, the Network Mask should be set to 255.255.255.0 to match the LevelOne FBR-1800TX Broadband Router. In Windows, you can check these settings by using Control Panel-Network to check the Properties for the TCP/IP protocol. |

## Internet Access

| Problem 1: | When I enter a URL or IP address I get a time out error. |
| --- | --- |
| Solution 1: | A number of things could be causing this. Try the following troubleshooting steps. - Check if other PCs work. If they do, ensure that your PCs IP settings are correct. If using a Fixed (Static) IP Address, check the Network Mask, Default gateway and DNS as well as the IP Address. - If the PCs are configured correctly, but still not working, check the LevelOne FBR-1800TX Broadband Router. Ensure that it is connected and ON. Connect to it and check its settings. (If you can't connect to it, check the LAN and power connections.) - If the LevelOne FBR-1800TX Broadband Router is configured correctly, check your Internet connection (DSL/Cable modem etc) to see that it is working correctly. |
| Problem 2: | Some applications do not run properly when using the LevelOne FBR-1800TX Broadband Router. |
| Solution 2: | The LevelOne FBR-1800TX Broadband Router processes the data passing through it, so it is not transparent. Use the Special Applications feature to allow the use of Internet applications which do not function correctly. If this does solve the problem you can use the DMZ function. This should work with almost every application, but: - It is a security risk, since the firewall is disabled. - Only one (1) PC can use this feature. |

## Printing

| Problem 1: | When I tried to install the Printing software for Peer-to-Peer printing, I received an error message and the installation was aborted.. |
| --- | --- |
| Solution 1: | This may be caused by an existing installation of the printer port software. Before attempting another installation: - Remove the existing installation - Restart your PC To remove an existing printer port installation: - Open Start - Settings - Control Panel - Add/Remove Programs - Look for an entry with a name like "Shared Port", "Shared Printer Port", "Print Server Driver" or "Print Server Port". - Select this item, click "Add/Remove", and confirm the deletion. |
| Problem 2: | I am using Windows 95, and the Printing software installed and ran, but when I selected a port on a LevelOne FBR-1800TX Broadband Router and clicked "Add", the printer was not installed. |
| Solution 2: | Try installing the printer using the standard Windows tools, as follows: - Start the Add Printer Wizard. - Select Network Printer when prompted "How is the printer attached to your Computer?", and click Next. - When prompted for the Network Path or Queue, enter a dummy value such as \\123, as shown below. (Do NOT select Yes for "Do you print for MS-DOS programs?") - The printer wizard will display a message stating that “The Network Printer is off-line”. This is OK. Continue the Add Printer Wizard until finished. - When finished, go to Control Panel-Printers. The printer icon will be grayed out indicating the printer is not ready. - Right-click the Printer, and select Properties. Then select the Details tab, as shown below. - Click the Add Port button. On the resulting screen, select Other, then Shared Port, as the port to add, as shown below. - Click OK to see the Print Port Configuration screen, as shown below. - Click the Browse Device button, select the desired Multi-function LevelOne FBR-1800TX Broadband Router, and click OK. - Click OK to return to the Printers folders, and right-click on the Printer. Ensure that the Work off-line option is NOT checked. The Printer should no longer be grayed out, and is ready for use. |

Appendix B
Specifications

B

## Multi-Function LevelOne FBR-1800TX Broadband Router

| Model | LevelOne FBR-1800TX Broadband Router |
| --- | --- |
| Dimensions | 253mm(W) * 179mm(D) * 40mm(H) |
| Operating Temperature | 0 C to 40 C |
| Storage Temperature | -10 C to 70 C |
| Network Protocol: | TCP/IP |
| Network Interface: | 9 Ethernet: 8* 10/100BaseT (RJ45) LAN connection 1 * 10/100BaseT (RJ45) for WAN |
| Printer Port | 1 * parallel printer port |
| LEDs | 22 |
| Power Adapter | 12 V DC External |

## FCC Statement

This equipment has been tested and found to comply with the limits for a Class B digital device, pursuant to Part 15 of the FCC Rules.  These limits are designed to provide reasonable protection against harmful interference in a residential installation.

This equipment generates, uses and can radiate radio frequency energy and, if not installed and used in accordance with the instructions, may cause harmful interference to radio communications. However, there is no guarantee that interference will not occur in a particular installation. If this equipment does cause harmful interference to radio or television reception, which can be determined by turning the equipment off and on, the user is encouraged to try to correct the interference by one of the following measures:

- Reorient or relocate the receiving antenna.

- Increase the separation between the equipment and receiver.

- Connect the equipment into an outlet on a circuit different from that to which the receiver is connected.

- Consult the dealer or an experienced radio/TV technician for help.

To assure continued compliance, any changes or modifications not expressly approved by the party responsible for compliance could void the user's authority to operate this equipment. (Example - use only shielded interface cables when connecting to computer or peripheral devices).

### FCC Radiation Exposure Statement

This equipment complies with FCC RF radiation exposure limits set forth for an uncontrolled environment. This equipment should be installed and operated with a minimum distance of 20 centimeters between the radiator and your body.

This device complies with Part 15 of the FCC Rules. Operation is subject to the following two conditions:

(1) This device may not cause harmful interference, and

(2) this device must accept any interference received, including interference that may cause undesired operation.

This transmitter must not be co-located or operating in conjunction with any other antenna or transmitter.

### CE Marking Warning

This is a Class B product. In a domestic environment this product may cause radio interference in which case the user may be required to take adequate measures.