DC21X4 NDIS 5.0 miniport driver for Intel Windows 2000 [tm]

Version v5.05

26-Dec-1999

1. Overview:

The driver is currently supported for motherboards and option

boards based on the Intel 21143 100FX mbps Ethernet Controller.

2. Requirements:

A.  Hardware Requirements:

Intel 21143 100FX mbps Ethernet Controller

B.  Software Revision Requirements:

Windows 2000

3. Files and directories:

---------------------------------------------------------------------

INSTALL.DOC         This file.

---------------------------------------------------

NET21X4.INF         The driver's setup file for Windows 2000

---------------------------------------------------

DC21X4.SYS          The driver's executable

---------------------------------------------------------------------

4. Installing the Hardware Boards:

A. Installing the Intel's 21143 Evaluation Boards:

Refer to the Intel 21143 Evaluation Board

User's Guide.

5. Installing the DC21X4 Driver on Windows 2000:

1. From the Windows NT/Windows 2000 Main Menu, select the Control Panel icon.

2. From the Control Panel menu select the Network icon.

If a dialog box asks you "Do you want to install NT

Network?" select YES and Continue. Then place

the Windows 2000 CDROM in the CD-ROM reader.

Select the drive that matches the configuration of your system

and complete the install.

3. You will see several messages. To add a network adapter, select Yes

and then Continue

4. For each additional message, select continue.

5. From the Add Network Adapter dialog box, scroll through the menu to

select Other.

6. When the Insert Diskettes menu prompts you for a disk, insert the

DC21X4 driver installation diskette for Windows 2000 into drive A.

Then select OK.

7. Select your option

[Intel 21143 based 10/100 mbps Ethernet Controller]

from the Select OEM Option box, then select OK.

8. Modify the default distribution path in the Windows NT/Windows 2000

Setup box to point to the correct drive and path, then click

on [Continue]

9. For PCI options, a Dialog box pops up to select the:

ConnectionType:

------------------------------------------------------------------------

Intel 2114x based 10/100 mbps Ethernet Controller

-------------------------------------------------------------------------

o 100BaseFX (FX 100)          To connect 100BaseFX Ethernet hubs

o 100BaseFX (FX 100) FDx      To connect 100BaseFX Full Duplex hubs

10. Upon completion, the selected adapter is added to the

Installed Adapter Cards list of the Network Setting box

(the number prefixing the adapter is the adapter number).

11. Click on [Close] to complete the Network Setting

initialization.

12. If needed, modify the values in the registry (see section 7 below).

13. Shutdown and reboot the system.

Upgrading Drivers

See the components page available on the Intel World Wide Web

Internet site at:

http://developer.intel.com/design/network/new21/download/dsc-software-nc.htm