# INPROCOMM Networks Department Software Release Notes

| Product: | INPROCOMM IPN2220/5320 Linux AP Driver |
| --- | --- |
| Version: | 1.45.10.2004 |
| Classification: | #### Linux Driver for Kernel 2.4.x |
| Intended Distribution: | #### Quality Assurance and FAE |
| Date Released: | October 05, 2004 |

## Description:

The INPROCOMM IPN2220/5320 Linux AP Driver provides IEEE 802.11g/a Access Point functionality in Admtek adm5120, Intel IXP425, Kendin and Samsung S3C2510 platforms and so on.

## Purpose:

This is the revision release to support AP and WDS functions in Linux OS.

## System Environment:

Admtek adm5120 is the verified platform. The Linux kernel 2.4.x is supported.

## Hardware Supported:

| IPN2220, IPN5320 | Mini-PCI card with shielding case |
| --- | --- |

## Software Supported:

| Version 26 | Wireless tools: iwpriv (Please refer to file ioctl commands in Linux AP.doc) |
| --- | --- |

## File Released:

| IPN2220AP.o | Linux AP driver module file |
| --- | --- |

## Installation Instructions:

The INPROCOMM IPN2220/5320 Linux AP Driver is installed on a target platform by following Linux module installation, i.e., insmod IPN2220AP.o

## Limitations:

Known bugs or features not implemented are:

- Adjustment of transmit power is not supported.

- WDS cipher supports TKIP and CCMP, but compatibility will be a problem. The transmitting and receive security keys of TKIP are the same by internal duplication from input TKIP cipher key. The CCMP must use QoS packet format to solve packet header length problem.

- Mixed security mode is not supported.

- 802.11h is not implemented yet.

- When multiple cards exist simultaneously, the maximum number of supported stations is shared by all APs because of the memory pool limit.

## Release History:

- 1.45.10.2004 – 10/05/2004

  - Support IPN5320 for dual band operation. 802.11a and 802.11d are supported. 802.11h is not implemented yet.

  - Support more RF chip sets (UBEC and AIROHA) for 2.4GHz band.

  - This version is merged from v1.41.1.2 and do some modification.

- 1.44.09.2004 – 9/29/2004

  - Modify BB timing for RFMD difference between sample and MP. TX CLK80M is always on.

  - Antenna diversity is changed to AGC base for AP using.

- 1.43.09.2004 – 9/10/2004

  - (Linux) Bug fixed: fixed the bug in Radius 1x wep key using the EAP-MD5 authentication mode can’t ping problem.

  - Bug fixed: fixed the reserved IE buffer to small when call the cmd_set_RsnIE function, now reserved 80 bytes (32 for max WPA IE, 32 for max WPA2 IE, and 16 bytes is reserved for vendor specific).

  - (Linux) Add the WPA2 support. For this, add a compiling flag support WPA2 using the new hostapd.

- 1.42.08.2004 – 8/19/2004

  - (Linux) Bug fixed: support multiple cards existing simultaneously and rmmod procedure.

  - Set time margin to avoid transmit data rate to degrade at the moment when STA enters sleeping mode from active mode.

- 1.41.07.2004 – 7/23/2004

  - If VLAN is not enabled, the wireless stations can communicate each other even they are in different SSID set. Older driver version permits connection only when the wireless stations are in the same SSID set.

  - Support the direct setting method of 64 hex characters (32 bytes PSK value) for Windows XP SP2.

  - Receive buffer number is changed to 24 from 16 for lower speed CPU case.

- 1.40.07.2004 – 7/15/2004

  - New KAL interface. There is no macro defined in wlan_kal.h. All system call should be function-call. This eliminates the need to include system headers when compiling IPN2220 driver SDK.

  - Add support of Multiple SSID and VLAN-tagging, in current version support one authenticates method at a time. That mean, the authmode cipher and passphrase ioctl is use for all SSID.

  - Default DTIM period is changed to 1 from 3 for Wi-Fi certification. (Item 4.4.2.1 #MCA2  Buffalo-G bug, WLI-CB-G54(A), Driver revision 3.30.15.0)

  - (Linux) Remove unnecessary structure packing in AES, and this solves group key handshake failure on rtl8651 platform.

  - Add TKIP MIC software generation compiling option and related IO control. On some platforms, small packets are better verified their MICs by software other than by IPN2220 hardware.

  - Add information pages in IO Control. Embedded systems can query WLAN driver’s configuration, statistics, status, EEPROM and memory register and present these on the console.

  - Problem fixed: Fragment/Defragment with AES security.

- 1.38.1.8.06.2004 – 06/24/2004

  - Check received buffer is available when receive queue is empty to reduce receive empty interrupt.

  - The received WDS frame, which is not encrypted, should be discarded if the WDS link is established with security enabled.

  - Bug fixed: not all associated stations are listed before.

- 1.38.1.7.06.2004 – 06/14/2004

  - (Linux) Bug fixed: AP does not reject the station when 1x authentication fails.

  - (Linux) Check passphrase length (Max 63 character) of WPA-PSK.

  - The blocking of 1x port should only block data frame, excluding management frame. The old version blocks all frames except 802.1x packet.

- 1.38.1.6.06.2004 – 06/08/2004

  - (Linux) Bug fixed: STA cannot associate when 1x with WEP cipher mode.

  - (Linux) New function: Add the setting of group key re-key period. (Default setting is disabled)

  - (Linux) Do pair-wise key re-keying procedure when receiving EAPoL start frame from the associated STA.

- 1.38.1.5.05.2004 – 05/27/2004

  - Don’t check privacy bit when association request for Symbol-B card although Wi-Fi negative tests check it.

  - Burst packet number is changed to 10 from 100 for RF testing mode.

- 1.38.1.4.05.2004 – 05/25/2004

  - (Linux) Bug fixed: the case WPA-PSK with WEP cipher in Windows XP

  - (Linux) Ignore EAPoL start frame for compatibility with some vendors

- 1.38.1.3.05.2004 – 05/20/2004

  - Set guard time for TKIP MIC checking when re-key occurs.

  - (Linux) Bug fixed: 802.1x packet should be encrypted when re-key

- 1.38.1.2.05.2004 – 05/17/2004

  - Reject station whose privacy setting mismatch to pass Wi-Fi

  - Wrong power saving station number when the station is re-associated sometimes.

  - New feature: preamble mode can be set as auto, short only or long only.

- 1.38.1.1.05.2004 – 05/10/2004

  - Bug fixed: de-fragmentation and receive TKIP MIC

  - Bug fixed: The aging time may be longer than default value because wrong return transmission status when TKIP traffic is heavy.

  - Bug fixed: the reset procedure (stop receive queue, then transmit queue)

- 1.38.04.2004 – 04/28/2004

  - Blocking of traffic should be done before checking TKIP MIC of received packet.

  - Calculate all received TKIP MIC and verify them. Conditional compile flag is still reserved for low-speed CPU to enhance throughput.

  - Adding wpa_rekey_period parameter to set the wpa rekey period in hostapd.conf file.

- 1.37.04.2004 – 04/14/2004

  - Bug fixed: old pair-wise key may be not cleared up when cipher mode is changed to WEP from WPA or WPA-PSK.

  - Wait queue is implemented for TKIP security queue to reduce packet loss when traffic is very heavy.

  - Support data pipe mode. In this mode, AP driver indicate all data frame upwards and does nothing on WLAN to WLAN forwarding.

- 1.36.04.2004 – 04/12/2004

  - Security architecture is modified for RTOS requirement.

  - Solve disconnection problem when WPA-PSK with WEP cipher mode.

  - basicratemode is changed to ratemode and b-only mode is new setting.

  - Please note the hostapd must be v1.3 or later. The old version v1.2 of hostapd is not compatible with this version. (Magic ID is removed.)

  - Add WDS connection indication (judgment) function. Driver will block WDS port after several packets transmission failure to avoid reducing normal AP throughput.

  - Add receive antenna software control.

  - In Japan, auto channel function does not select channel 14.

- 1.34.02.2004 – 02/05/2004

  - Solve problem with Broadcom STA with 4-way handshake, sometimes STA shows message “IE not match”

  - Support ACL and WDS with WEP and CCMP cipher. (TKIP not supported).

  - Only iwpriv is needed. Do not need iwconfig program.

- 1.29.11.2003 – 11/25/2003

  - This release reports a version of 1.29.11.2003 when install module.

  - This driver source is tested in x86, S3C2510 and ADM5120 platforms.

- 1.28.11.2003 – 11/05/2003

  - This release reports a version of 1.28.11.2003 when install module.