# INPROCOMM Software Documentation

| Title: | IOCTL commands in Linux AP |
| --- | --- |
| Version: | V1.12 |
| Classification: | #### Linux, iwpriv |
| Intended Distribution: | #### Quality Assurance, FAE, Customer |
| Date Released: | 2004-08-04 |
| Document maintainer | Chia-Ming Chang |

## Description:

The INPROCOMM IPN2220/5320 Access Point Driver (IPN2220AP.o) provides IEEE 802.11g/a access point functionality in Linux OS. This document covers its available I/O control commands on Linux platform.

## Purpose:

To describe I/O control command support by IPN2220/5320 AP on Linux platform. This is designed be a user interface developer’s reference for end user to control AP functions via WEB or command-line interfaces.

## System Environment:

Linux kernel 2.4, wireless tool 16.

## Hardware Supported:

| WLAN chip | IPN2220, IPN5320 |
| --- | --- |
| Platform | i386, Samsung 2510, Adm5120, Intel IXP425, Realtek RTL8651 and so on. |

## Release History:

- V1.12 – 8/04/2004

- Add two new commands to support dual band solution (IPN5320), i.e., RF band selection before setting channel and country code setting for 802.11d.

- V1.11 – 7/21/2004

- Support XP SP2 direct setting the 64 characters (for 32 bytes PSK value) method, using the passphrase IOCTL, if all the entered 64 characters are 0~9 or A~F, driver will use the value as PSK value.

- V1.10 – 6/29/2004

- Add description on support of Multiple SSID and VLAN-tagging, in current version support one authenticates method at a time. That mean, the authmode cipher and passphrase ioctl is use for all SSID.

- Default DTIM period is changed to 1 from 3 for Wi-Fi certification. (Item 4.4.2.1 #MCA2 Buffalo-G bug, WLI-CB-G54(A), Driver revision 3.30.15.0). The IPN2220AP.o driver should be v1.40 or later.

- V1.9 – 06/10/2004

- Add the Radius 1X setting sample in “Examples on setting security mode:”

- Fixing the value error for Shared key mode in “Examples on setting security mode:”

- Check passphrase length (Max 63 character) of WPA-PSK. The IPN2220AP.o driver should be v1.38.1.7 or later

- V1.8 – 06/08/2004

- Add new setting: the period of group key re-key (Default value is disabled). The IPN2220AP.o driver should be v1.38.1.6 or later.

- V1.7 – 05/21/2004

  - Serious mistake for network authentication mode in this file. The correct matching between driver and IOCTL commands is as follows:

| Authentication Algorithm | Driver Version 1.35 | Other Driver Versions And Future Versions |
| --- | --- | --- |
| Open System | 0 | 0 |
| Shared Key | 1 | 2 |
| Auto Switch between Open System And Shared Key | 2 | 3 |
| Open System With 802.1x | 3 | 1 |
| WPA | 4 | 4 |
| WPA-PSK | 5 | 5 |

- V1.6 – 05/17/2004

  - Add new command (preamble) to set preamble mode.

- V1.5 – 04/27/2004

  - Add wpa_rekey_period parameter in hostapd.conf file

- V1.4 – 04/12/2004

  - basicratemode is changed to ratemode for b-only mode

  - Add receive antenna software control

- V1.2 – 1/8/2004

  - Remove unused “get” commands

- V1.1 – 12/16/2003

  - Remove unused iwconfig

- V1.0 – 10/28/2003

  - Document created.

## Fast-track Examples:

To Start AP:

Load the driver into kernel:

insmod IPN2220AP.o

insmod IPN2220AP.o x_ifname=“wlan” x_MAX_AID=128

Enable the interface:

ifconfig wlan0 up

ifconfig wlan0 192.168.1.1 up

Start kernel bridging:

(1) brctl addbr br0

(2) brctl addif br0 eth0

(3) brctl addif br0 wlan0

(4) brctl stp br0 disable

(5) brctl setfd br0 0

(6) ifconfig eth0 0.0.0.0

(7) ifconfig wlan0 0.0.0.0

(8) ifconfig br0 192.168.1.1 up

To Stop AP

Stop kernel bridging:

(1) ifconfig br0 down

(2) brctl delif br0 eth0

(3) brctl delif br0 wlan0

(4) brctl delbr br0

Disable the interface:

ifconfig wlan0 down

Unload the driver from kernel:

rmmod IPN2220AP

## I/O Control Command List:

| Category | Command | Meaning | Page |
| --- | --- | --- | --- |
| 802.11 | essid | Set Primary SSID’s ESSID | 7 |
|  | broadcastssid | Broadcast/Hide SSID | 7 |
|  | beaconperiod | Beacon Period | 8 |
|  | channel | Set Channel | 7 |
|  | g_currentchan | Show Current Operating Channel | 7 |
|  | datarate | Transmit Data Rate | 7 |
|  | rts | RTS Threshold | 7 |
|  | frag | Fragmentation Threshold | 7 |
|  | ratemode | Basic Rate Mode | 7 |
|  | basicrateset | Basic Rate Set | 7 |
|  | dtimperiod | DTIM Period | 8 |
|  | ctsprotect | CTS Protect Mode | 8 |
|  | burstmode | CFB Burst Mode | 8 |
|  | domain | Country Domain | 8 |
|  | antenna | Receive antenna software setting | 8 |
|  | preamble | CCK preamble mode | 9 |
| Security | authmode | Authentication Mode | 9 |
|  | cipher | Encryption Cipher Suite | 9 |
|  | defwepkeyid | Default WEP Key ID | 10 |
|  | wepkeyid | UI-WEP Key ID | 10 |
|  | wepkey | WEP Key | 10 |
|  | passphrase | Pass Phrase | 9 |
|  | gkrekeyperiod | Period of group key re-key | 10 |
| Management | norelay | Wireless Frame Forwarding | 9 |
|  | nostation | Accept Association with Stations | 10 |
|  | killmac | Kill An Associated Station by Its MAC Address | 11 |
| WDS | wdsaddmac | Add A WDS Link by remote AP’s MAC Address | 10 |
|  | wdsdelmac | Delete A WDS Link by remote AP’s MAC Address | 10 |
|  | wdskey | Encryption Key in One WDS Link | 10 |
|  | wdscipher | Encryption Cipher in One WDS Link | 10 |
|  | wdsapprotocol | Remote AP’s protocol (11b or 11g) | 10 |
| Multi-SSD | ssidid | UI-SSID ID | 11 |
|  | setmssid | Set a SSID’s ESSID | 11 |
|  | vlanid | Set a SSID’s VLAN ID | 11 |
|  | vlanenable | Set a SSID to indicate VLAN-tagged Packet or not | 11 |
|  | ssidenable | Set the designated SSID on/off | 11 |
| Dual Bands | rfband | Switch between 2.4GHz and 5GHz before setting channel | 12 |
|  | country | Enable/Disable and set country information | 12 |
| RF Test | rftest_mode | RF Test Mode | 12 |
|  | rftest_power | RF Test Power | 13 |

## I/O Control Reference:

- SSID (32 Case Sensitive Printable Characters, Space Permitted). This command only works on primary SSID. Please refer to Multiple SSID.

To set:		iwpriv wlan0 essid AP2220

iwpriv wlan0 essid “ipc AP2220”

- Broadcast SSID: Enable—SSID will show in beacons (Default), Disable

To enable:		iwpriv wlan0 broadcastssid 1

To disable:		iwpriv wlan0 broadcastssid 0

- Channel: (0 for auto channel selection. 1 ~ 14) (Default: 11)

To set:		iwpriv wlan0 channel 1

To show the setting:

To show current operating channel:

iwpriv wlan0 g_currentchan

- RTS Threshold (0 ~ 2347) (Default: 2347)

To set:		iwpriv wlan0 rts 500

- Fragmentation Threshold (256 ~ 2346) (Default: 2346)

To set:		iwpriv wlan0 frag 300

- Transmit Rate (Control Frames are excluded)

  - Auto (Default)

To set:		iwpriv wlan0 datarate 0

  - Fixed (1, 2, 5.5, 6, 9, 11, 12, 18, 24, 36, 48, 54Mbps)

Use exact bit-rate:

To set:		iwpriv wlan0 rate 54000000

- Rate Mode (Mixed Mode, Default)

To select mode – mixed:	iwpriv wlan0 ratemode 0

To select mode – g only:	iwpriv wlan0 ratemode 1

To select mode – b only:	iwpriv wlan0 ratemode 2

Note: In Japan, only b-only mode is allowed in channel 14.

- Basic Rate Set (Mixed Mode – 1, 2, 5.5 and 11Mbps, Default)

To set: Using bit-wise &

| 1 | 2 | 5.5 | 11 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x0001 | 0x0002 | 0x0004 | 0x0008 |  |  |  |  |  |
| 6 | 9 | 12 | 18 | 24 |  | 36 | 48 | 54 |
| 0x0010 | 0x0020 | 0x0040 | 0X0080 | 0X0100 |  | 0X0200 | 0X0400 | 0X0800 |

Ex.	1M:				iwpriv wlan0 basicrateset 0x0001

2M:				iwpriv wlan0 basicrateset 0x0002

1M & 2M:		iwpriv wlan0 basicrateset 0x0003

Shown basic rate will be a decimal number.

Ex.	1M & 2M & 5.5M & 11M: 0x000f = 15.

1M & 2M & 5.5M & 11M & 6M & 9M: 0x003f = 63.

- CTS Protection

  - Auto (Default)

To set:		iwpriv wlan0 ctsprotect 0

  - Always

To set:		iwpriv wlan0 ctsprotect 1

  - None (Discard Neighbor NonERP BSS, Associated NonERP STAs)

To set:		iwpriv wlan0 ctsprotect 2

- Transmit Burst Mode: Enable, Disable (Default)

To enable:		iwpriv wlan0 burstmode 1

To disable:		iwpriv wlan0 burstmode 0

- Beacon Interval (20 ~ 1000 TU) (Default: 100)

To set:		iwpriv wlan0 beaconperiod 100

- DTIM Period (1 ~10) Depending System Memory Size (Default: 1)

To set:		iwpriv wlan0 dtimperiod 3

- Regulation Domain

| US | Canada | Europe | Spain | France | Japan1 Chan 1-14 | Japan2 Chan 1-14 |
| --- | --- | --- | --- | --- | --- | --- |
| 0x10 | 0x20 | 0x30 | 0x31 | 0x32 | 0x40 | 0x41 |

To set:		iwpriv wlan0 domain 0x10

- Receive antenna software setting (Default: 1)

To set receive antenna to diversity:		iwpriv wlan0 antenna 0

To set receive antenna to primary:		iwpriv wlan0 antenna 1

To set receive antenna to secondary:	iwpriv wlan0 antenna 2

- CCK preamble mode setting (Default: 0)

Auto: Preamble type depends on associated station:	iwpriv wlan0 preamble 0

Long preamble: only long preamble CCK is permitted:	iwpriv wlan0 preamble 1

Short preamble: only short preamble CCK is permitted:	iwpriv wlan0 preamble 2

- Air Relay (No frame forwarding between wireless stations) (Default: 0 with data forwarding)

To enable data forwarding:	iwpriv wlan0 norelay 0

To disable data forwarding:	iwpriv wlan0 norelay 1

- Network Authentication

  - Open System

To set:	iwpriv wlan0 authmode 0

  - Shared Key (WEP Only)

To set:	iwpriv wlan0 authmode 2

  - Auto Switch between Open System And Shared Key

To set:	iwpriv wlan0 authmode 3

  - Open System With 802.1x

To set:	iwpriv wlan0 authmode 1

  - WPA

To set:	iwpriv wlan0 authmode 4

  - WPA-PSK

To set:	iwpriv wlan0 authmode 5

- Data Encryption (Hardware: Max. 128 Entries)

  - No Encryption

To set:	iwpriv wlan0 cipher 0

  - WEP64

To set:	iwpriv wlan0 cipher 1

  - WEP128

To set:	iwpriv wlan0 cipher 5

  - WEP152

To set:	iwpriv wlan0 cipher 6

  - TKIP

To set:	iwpriv wlan0 cipher 2

  - AES-CCMP

To set:	iwpriv wlan0 cipher 4

- Pass Phrase for WPA-PSK (1 ~ 63 Case Sensitive Printable Characters, Space Permitted or exactly 64 characters using 0~9 and A~F)

To set:		iwpriv wlan0 passphrase “12345678”

- Set the period of group key re-key for WPA and WPA-PSK (Default: 0, disabled)(Unit: Second)

To set:		iwpriv wlan0 gkrekeyperiod	3600 	/* 3600 seconds = 1 hour */

- WEP Key

  - To set/get a WEP key, first set the index of the key (0 ~ 3)

To set:	iwpriv wlan0 wepkeyid 0

Then set the key (10/26/32 hex digits for 64-/128-/152-bit WEP key).

To set:	iwpriv wlan0 wepKey 0123456789

  - To enable a WEP key as default key, given the index of the key (0 ~ 3)

To set:	iwpriv wlan0 defwepkeyid 0

- WDS

  - To add a remote access point by its MAC address (then the corresponding net device can be enabled)

To set:		iwpriv wlan0 wdsaddmac 00:08:22:11:22:33

To enable the newly created net device (if 00:08:22:11:22:33 is the n-th remote AP, then wlan0wds[n-1] will be created):

ifconfig wlan0wds0 up

To include this interface as a port in bridge interface:

brctl addif br0 wlan0wds0

  - To remove a remote access point by its MAC address (after its net device is disabled)

To set:		iwpriv wlan0 wdsdelmac 00:08:22:11:22:33

To disable the net device:

ifconfig wlan0wds0 down

To remove this interface from the bridge interface:

brctl delif br0 wlan0wds0

  - To set the cipher suite used between WDS links (values please see 10.)

To set:		iwpriv wlan0wds0 wdscipher 0

  - To set the WEP key used between WDS links

To set:		iwpriv wlan0wds0 wdskey 0123456789

  - To set the protocol of remote AP

To set 11g:	iwpriv wlan0wds0 wdsapprotocol 1

To set 11b:	iwpriv wlan0wds0 wdsapprotocol 0

- To disallow/allow AP sending beacon and accepting stations

To disallow AP sending beacon and accepting stations:

iwpriv wlan0 nostation 1

To allow AP sending beacon and accepting stations:

iwpriv wlan0 nostation 0

- To kill an associated station by its MAC address

iwpriv wlan0 killmac 00:11:22:33:44:55

- Multiple SSID

IPN2220AP driver supports up to eight SSIDs, include a primary SSID. Each SSID has its own VLAN ID, and can be set VLAN-tag enabled/disabled respectively. All SSIDs must use the encryption suite and authentication mechanism, but they can have different keys.

When using multi-SSID, the AP usually disables broadcasting SSID as well. Otherwise, most wireless station can’t use associate with SSIDs other than the primary SSID.

Please be aware that IPN2220AP won’t forward unicast data frames between stations in different SSIDs. These traffics must go through upper layer.

Command:

ssidid:	Used to specify which SSID for the following commands to have effect on.

setmssid:	Set the SSID ESSID

vlanid:	Set a SSID’s VLAN ID

vlanenable:	Set a SSID to indicate VLAN-tagged Packet or not

ssidenable:	Set a SSID enabled or disabled. Primary SSID(with index 0) is always enabled.

Example:

- Create 2 SSID, ssid-test0 and ssid-test1. Each shall indicate to system without VLAN-tags.

iwpriv wlan0 ssidid 0

iwpriv wlan0 setmssid ssid-test0

iwpriv wlan0 vlanenable 0

iwpriv wlan0 ssidenable 1

iwpriv wlan0 ssidid 1

iwpriv wlan0 setmssid ssid-test1

iwpriv wlan0 vlanenable 0

iwpriv wlan0 ssidenable 1

- Create 2 SSID, ssid-test0(VLAN-ID: 3) and ssid-test1(VLAN-ID: 4). Each shall indicate to system with VLAN-tags.

iwpriv wlan0 ssidid 0

iwpriv wlan0 setmssid ssid-test0

iwpriv wlan0 vlanenable 1

iwpriv wlan0 vlanid 3

iwpriv wlan0 ssidid 1

iwpriv wlan0 ssidenable 1

iwpriv wlan0 setmssid ssid-test1

iwpriv wlan0 vlanenable 1

iwpriv wlan0 vlanid 4

iwpriv wlan0 ssidenable 1

- Dual Bands

  - Switch desired RF band before setting channel (0: 2.4GHz, 1: 5GHz)

Example: set operating channel to 36 in 5GHz

iwpriv wlan0 rfband 1

iwpriv wlan0 channel 36

Example: set operating channel to 6 in 2.4GHz

iwpriv wlan0 rfband 0

iwpriv wlan0 channel 6

  - Enable/Disable and set country information in beacon and probe response packets. The country string consists of 2 characters and is case sensitive (upper case).

| ##### Country | ###### Disable | USA | Canada | Spain | France | Japan | Austria | Belgium | Denmark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ##### String | 00 | US | CA | ES | FR | JP | AT | BE | DK |

| ##### Finland | Germany | Greece | Iceland | Ireland | Italy | Liechtenstein | Luxembourg | Netherlands | Norway |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ##### FI | DE | GR | IS | IE | IT | LI | LU | NL | NO |

| ##### Portugal | Sweden | Switzerland | United Kingdom | Australia | China | Israel | Jordan | Korea | Singapore |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ##### PT | SE | CH | GB | AU | CN | IL | JO | KR | SG |

| ##### Taiwan | Thailand |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ##### TW | TH |  |  |  |  |  |  |  |  |

To set country code to Japan:		iwpriv wlan0 country JP

To disable country information:	iwpriv wlan0 country 00

- RF Test

  - Mode (0, 1, 2, 3)

Stop:	iwpriv wlan0 rftest_mode 0

Rx:		iwpriv wlan0 rftest_mode 1

Tone:	iwpriv wlan0 rftest_mode 2

Burst:	iwpriv wlan0 rftest_mode 3

  - Power (0 ~ 255)

To Set:	iwpriv wlan0 rftest_power 0

Test Procedure:

- Set AP channel

iwpriv wlan0 channel 6

- Set AP rate

iwpriv wlan0 datarate 54000000

- Set RF test power

iwpriv wlan0 rftest_power 0x2e

- Set RF test mode

iwpriv wlan0 rftest_mode 3

Note:  Issue rftest_mode command should be the last command for each mode.

## Wireless Security Setting:

- Security includes Authentication and Encryption

- Authentication and Data Encryption

|  | No | WEP64 | WEP128 | WEP152 | TKIP | AES | Radius Server |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Open |  |  |  |  |  |  |  |
| Open with 802.1x |  |  |  |  |  |  |  |
| Shared Key |  |  |  |  |  |  |  |
| WPA |  |  |  |  |  |  |  |
| WPA-PSK |  |  |  |  |  |  |  |

- Security profiles:

- OPEN-WEP 64/128/152

  - Auth mode: Open

  - Cipher mode: WEP 64/128/152

  - WEP:

- Default key: 0/1/2/3

- Key (0/1/2/3): …

- Open w/1X (Radius) -WEP 64/128

  - Auth mode: Open w/1X

  - Cipher mode: WEP 64/128

  - WEP:

- Default key: 0/1/2/3

- Key (0/1/2/3): …

  - Radius server:

- Radius server’s IP address

- Radius server’s port

- Radius server shared key

- SHARED-WEP 64/128/152

  - Auth mode: Shared

  - Cipher mode: WEP 64/128/152

  - WEP:

- Default key: 0/1/2/3

- Key (0/1/2/3): …

- WPA

  - Auth mode: WPA

  - Cipher mode: TKIP, AES, WEP64/128

  - Radius server (if Cipher mode: TKIP, AES):

- Radius server’s IP address

- Radius server’s port

- Radius server shared key

- All the setting please reference to sample Hostapd.conf

- WPA-PSK

  - Auth mode: WPA-PSK

  - Cipher mode: TKIP, AES

  - Passphrase: string

- Examples on setting security mode:

- Disable Security

iwpriv wlan0 authmode 0; iwpriv wlan0 cipher 0

- WEP-64bit Open-system

iwpriv wlan0 authmode 0; iwpriv wlan0 cipher 1;

iwpriv wlan0 wepkeyid 0; iwpriv wlan0 wepkey 9876543210

iwpriv wlan0 defwepkeyid 0

- WEP-64bit Shared-system

iwpriv wlan0 authmode 2; iwpriv wlan0 cipher 1;

iwpriv wlan0 wepkeyid 0; iwpriv wlan0 wepkey 9876543210

iwpriv wlan0 defwepkeyid 0

- WEP-128bit Open System

iwpriv wlan0 authmode 0; iwpriv wlan0 cipher 5;

iwpriv wlan0 wepkeyid 0

iwpriv wlan0 wepkey 12345678901234567890123456

iwpriv wlan0 defwepkeyid 0

- WEP-128bit Open & Shared-system

iwpriv wlan0 authmode 3; iwpriv wlan0 cipher 5;

iwpriv wlan0 wepkeyid 0;

iwpriv wlan0 wepkey 12345678901234567890123456

iwpriv wlan0 defwepkeyid 0

- WEP-152bit Open System

iwpriv wlan0 authmode 0; iwpriv wlan0 cipher 6;

iwpriv wlan0 wepkeyid 0;

iwpriv wlan0 wepkey 12345678901234567890123456789012

iwpriv wlan0 defwepkeyid 0

- WPA (TKIP)

iwpriv wlan0 authmode 4; iwpriv wlan0 cipher 2

Modify the hostapd.conf (See below)

wpa=1

wep_key_len_broadcast=5 or 13

wep_key_len_unicast=5 or 13

Note: the key_len can be ether 5 or 13 (only use for hostapd setting the Radius key, the length is no matter), and can’t be make the line or set value to 0

- WPA-PSK (AES-CCMP)

iwpriv wlan0 authmode 5; iwpriv wlan0 cipher 4

iwpriv wlan0 passphrase “12345678”

- Radius 1X With WEP

iwpriv wlan0 authmode 1

iwpriv wlan0 cipher 1 or 5

Modify the hostapd.conf (See below)

wpa=0

wep_key_len_broadcast=5 or 13

wep_key_len_unicast=5 or 13

- Radius 1X With Non Security

iwpriv wlan0 authmode 1

iwpriv wlan0 cipher 0

Modify the hostapd.conf (See below)

wpa=0

wep_key_len_broadcast=0

wep_key_len_unicast=0

Note : to reload the hostapd daemon, kill (hostapd PID) then do the ./hostapd hostapd.conf &

## Sample hostapd.conf :

##### hostapd configuration file ##############################################

# Empty lines and lines starting with # are ignored

# AP netdevice name

interface=wlan0

##### IEEE 802.11 related configuration #######################################

# Station MAC address -based authentication

# 0 = accept unless in deny list

# 1 = deny unless in accept list

# 2 = use external RADIUS server (accept/deny lists are searched first)

macaddr_acl=0

# Accept/deny lists are read from separate files (containing list of MAC addresses, one per line). Use absolute path name to make sure that the files can be read .

#accept_mac_file=/etc/hostapd.accept

#deny_mac_file=/etc/hostapd.deny

##### IEEE 802.1X (and IEEE 802.1aa/D4) related configuration #################

# Require IEEE 802.1X authorization, Note: this must be 1 for support 1x packet

ieee8021x=1

# Support WPA, Setting 1 for WPA, 0 for pure Radius

wpa=1

#The WPA rekey period in seconds. 0 = do not rekey

wpa_rekey_period=0

# Optional displayable message sent with EAP Request-Identity

eap_message=hello

# Key lengths for default/broadcast and individual/unicast keys:

# 5 = 40-bit WEP (also known as 64-bit WEP with 40 secret bits)

# 13 = 104-bit WEP (also known as 128-bit WEP with 104 secret bits)

wep_key_len_broadcast=5

wep_key_len_unicast=5

##### RADIUS configuration ####################################################

# for IEEE 802.1X with external Authentication Server, IEEE 802.11

# authentication with external ACL for MAC addresses

# RADIUS authentication server

auth_server_addr=192.168.0.181

auth_server_port=1812

auth_server_shared_secret=greycats