MCR SDRAM 設定速查表

Per chip 16Mb(2MB)

|  | 1 pcs (total 2MB) | 2 pcs (total 4MB) (當兩顆用,Bus Width:16-bit) | 2 pcs (total 4MB) (兩顆當一顆,Bus Width:32-bit) |
| --- | --- | --- | --- |
| 16-bit顆粒 | 0xcaa00000 | 0xcaa00000 | 0xcab00000 |
| 32-bit顆粒 | Not Supported | Not Supported | Not Supported |

Per chip 64Mb(8MB)

|  | 1 pcs (total 8MB) | 2 pcs (total 16MB) (當兩顆用,Bus Width:16-bit) | 2 pcs (total 16MB) (兩顆當一顆,Bus Width:32-bit) |
| --- | --- | --- | --- |
| 16-bit顆粒 | 0xdaa00000 | 0xdaa00000 | 0xdab00000 |
| 32-bit顆粒 | Not Supported | Not Supported | Not Supported |

Per chip 128Mb(16MB)

|  | 1 pcs (total 16MB) | 2 pcs (total 32MB) (當兩顆用,Bus Width:16-bit) | 2 pcs (total 32MB) (兩顆當一顆,Bus Width:32-bit) |
| --- | --- | --- | --- |
| 16-bit顆粒 | 0xeaa00000 | 0xeaa00000 | 0xeab00000 |
| 32-bit顆粒 | 0xdab00000 | Not Supported | Not Supported |

Per chip 256Mb(32MB)

|  | 1 pcs (total 32MB) | 2 pcs (total 64MB) (當兩顆用,Bus Width:16-bit) | 2 pcs (total 64MB) (兩顆當一顆,Bus Width:32-bit) |
| --- | --- | --- | --- |
| 16-bit顆粒 | 0xfaa00000 | 0xfaa00000 | 0xfab00000 |
| 32-bit顆粒 | 0xeab00000 | Not Supported | Not Supported |

Per chip 512Mb(64MB)

|  | 1 pcs (total 64MB) | 2 pcs (total 128MB) (當兩顆用,Bus Width:16-bit) | 2 pcs (total 128MB) (兩顆當一顆,Bus Width:32-bit) |
| --- | --- | --- | --- |
| 16-bit顆粒 | 0xfaa00800 | Not Supported | Not Supported |
| 32-bit顆粒 | 0xfab00000 | Not Supported | Not Supported |

* 若使用32-bit的SDRAM顆粒，其Bus Width一定也是設定成32-bit。