# INPROCOMM Software Documentation

| Title: | User guide to use IPN2220.o object file |
| --- | --- |
| Version: | V1.10 |
| Classification: | #### Normal |
| Intended Distribution: | #### FAE, Customer |
| Date Released: | 2004/7/8 |
| Document maintainer | Chia-Ming Chang |

## Description:

IPN2220 is a high performance 802.11g chip. This chip supports PCI, miniPCI and CardBus interface.

Since IPN2220 is an ASIC, other than firmware-based WLAN chip, Inprocomm provides this KAL-based driver SDK to shorten system manufacturer’s porting efforts.

KAL: Kernel Abstract Layer, which is the layer between Operating System and IPN2220.o driver object code. This document describes what header file should be provided by Operating System and what functions are provided by IPN2220.o library file.

## Purpose:

Only by modifying this kernel abstract layer, it is easy to port IPN2220 AP driver into a new platform with a specific Operating System.

## System Environment:

Depending on customer’s development environment.

## Hardware Supported:

| WLAN chip | IPN2220 |
| --- | --- |
| Platform | Implemented with miniPCI,PCI, CardBus |

## Release History:

- V1.9 - 7/13/2004

  - Support software TKIP MIC verification. On some platforms, short packets’ MICs are better calculated by software than hardware.

  - Add support on multiple SSID and VLAN Tag.

  - Add information pages in IO Control. Embedded systems can query WLAN driver’s configuration, statistics, status, EEPROM and memory register and present these on the console.

- V1.8 - 5/26/2004

  - Add notification on WPA. Global key must be set before any station associating with AP.

  - Add comment on clear key.

- V1.6 - 4/28/2004

  - Fix ambiguous wording

  - Explain how to stop AP function:when tkip mic failures happens and the system wish to shut down this BSS.

- V1.5 – 4/22/2004

  - Add param_get_curTimeStamp.

  - Put FAQ raised by customers in special notes.

- V1.4 – 4/15/2004

  - Add switch mode, rx antenna, and driver version I/O control.

  - Add do_timer_check in driver life cycle

  - Remove security mode unrelated to this porting guide.

- V1.3 – 3/31/2003 (Yeh, Chih Hao)

  - Sync with up-to-date IPN2220 driver

  - Add IPN2220 Driver Life Cycle

  - Add Support WPA Capability

  - Add support Legacy WEP

  - Add support WDS

- V1.0 – 1/7/2003 (Chia-Ming Chang)

  - Document created.

## Files Overview:

- Build environment:

  - config.mk: compiler, compiler flags and link flags

  - Makefile.AP: the Makefile to incorporate IPN2220.o into the final target driver or system image file.

- IPN2220 SDK:

  - wlan_lib.h: IPN2220.o’s function prototypes, which will be invoked by KAL layer. These functions include those used by driver life cycle, data path, and parameter control.

- KAL Sample Code:

  - Please note that the following sample is based on Linux. Your OS need not do all of this implementation.

  - wlan_kal.h: OS must provide these corresponding function prototypes, data structure type, constants, system header files and how to pack a data structure type. This file will be imported when compiling hardware-related files to create IPN2220.o object file.

  - wlan_kal.c: OS dependent file, which includes the functions declared in wlan_kal.h file.

  - wlan_os.c: OS dependent functions, including how to get PCI memory base address, how to create a network device, register interrupt routine and create a periodic timer.

  - wds_os.c: OS dependent functions, including how to create the logical network device as wireless distribution system.

  - skbuff.h: Linux source code. An example illustrates how to process packet buffer.

  - linux_comp.c: A simplified version of sk_buff manuplation, including skb_put, skb_pull, skb_put. You may use this file, if you never port Linux’s sk_buff manipulation function before.

## Initialization Procedure:

- i2220ap_priv_create(): create the private data structure

- i2220ap_pci_probe (): hardware initialization and start default AP functions.

- Request IRQ and Register Interrupt Routine: i2220hw_interrupt()

Remove Procedure:

- i2220ap_remove_pci(): stop hardware operation

- i2220ap_priv_destroy(): free memory of private data structure

- Free IRQ

Other Routines in file wlan_lib.h:

- Interrupt Defer Processing Process: do_isr_tasklet(). If system does not use deferred task mechanism, do_isr_tasklet()can be called  right after invoking i2220hw_interrupt().

- Transmit Packet: i2220ap_hard_start_xmit()

- Receive Packet: IPN2220.o will call kal_netdev_rx(), provided by OS

- Periodic Timer: IPN2220 driver need a periodic timer, i.e., system invokes the function do_timer_check() every 100ms.

- Change MAC Address: i2220ap_set_mac_address()

### Parameters Setting

| ##### Parameter | ##### Get / Set | ##### Function Name | ##### Remarks |
| --- | --- | --- | --- |
| SSID | Set | param_set_ssid | Maximum 32 bytes. |
| Beacon Interval | Set | param_set_bcnPeriod | 20 ~ 1000 Kus |
| DTIM Period | Set | param_set_dtimPeriod | 1 ~ 10 |
| Set Channel | Set | param_set_setChannel | 0: auto, 1 ~ 14 |
| Get Channel | Get | param_get_curChannel | 1 ~ 14 |
| RTS Threshold | Set | param_set_rts | 0 ~ 2347 |
| Fragmentation Threshold | Set | param_set_frag | 256 ~ 2346 |
| Basic Rate Set | Set | param_set_basicRateSet | Bit0: 1M, Bit1: 2M, Bit2: 5.5M, Bit3: 11M, Bit4: 6M, Bit5: 9M, Bit6: 12M, Bit7: 18M, Bit8: 24M, Bit9: 36M, Bit10: 48M, Bit11: 54M |
| Rate Mode | Set | param_set_rateMode | 0: b+g (1,2,5.5,11M), 1: g only (1,2,5.5,11,6,12,24M) 2: B only |
| Authentication Mode | Set | param_set_authmode | 0: Open, 1: Shared key, 2: Both |
| Cipher Mode | Set | param_set_cipherMode | 0: None, 1: WEP64, 5: WEP128, 6: WEP152 |
| WEP Default Key ID | Set | param_set_wepDefaultKeyId | 0 ~ 3 |
| WEP Key ID | Set | param_set_wepKeyId | Assign key ID (0 ~ 3) before setting WEP key |
| WEP Key | Set | param_set_wepKey | Make sure the key ID before setting WEP key. |
| TKIP HW MIC threshold | Set | param_set_hwMICthreshold | Assign HW tkip MIC calculation threshold. Default is 0 byte. |
| SSID Broadcast | Set | param_set_bcastSSID | 0: disable, 1: enable |
| CTS Protect Mode | Set | param_set_ctsProtect | 0: auto, 1: always, 2: none |
| Fixed Data Rate | Set | param_set_dataRate | 0: Auth rate 1Mbps: 1000000, 2M: 2000000 and so on |
| Burst Mode | Set | param_set_burstMode | 0: disable, 1: enable |
| AP relay mode | Set | param_set_noRelay | 0: Switch mode 1: Switch, but disable forwarding between stations 2: Data pipe mode |
| Kill Associated STA | Set | param_set_killmac(Obsolete) | MAC address |
| Receive antenna software control | Set | param_set_rxAntenna() | 0: auto (diversity) 1: primary 2: secondary |
| Disable AP Function | Set | param_set_noStation | 0: Normal, 1: disable AP function, but WDS is still working. |
| Add remote WDS AP | Set | param_add_wdslink | MAC address of Remote AP |
| WDS AP Rate Mode | Set | param_set_wds_rateMode | 0: 802.11b, 1: 802.11g |
| WDS AP Cipher Mode | Set | param_set_wds_cipherMode | Same with Cipher Mode |
| WDS AP Cipher Key | Set | param_set_wds_cipherKey | Same with Cipher Key |
| Delete Remote WDS AP | Set | param_del_wdslink | MAC address of Remote AP |
| Set Failure Bound | Set | param_set_wds_tx_failure_limit | How many packets tx fails |
| Set Disable-Duration | Set | param_set_wds_disable_limit | In unit of second |
| Regulation Domain | Set | param_set_regDomain |  |
| RF Testing Mode | Set | param_set_rftestMode | 0: Stop, 1: Rx, 2: Tone, 3: Burst |
| Network Statistics | Get | param_get_Statistic |  |
| Station Records | Get | param_get_StaList |  |
| Driver Version | Get | param_get_version | Return value: string |
| Chip time stamp | Get | param_get_curTimeStamp | Return value: unsigned long |
| Specify which SSID | Set | param_set_ssid_id | Assign SSID ID (0 ~ 7) before setting Multiple SSID and VLAN function. |
| Set Muliple SSID’s ESSID | Set | param_set_m_ssid | Maximum 32 bytes. |
| Set VLAN ID | Set | param_set_m_vlanid | VLAN ID: 0 ~ 4095 |
| VLAN Tag enable | Set | param_set_m_vlantag_enable | 1: Enable 0: Disable |
| SSID Enable/Disable | Set | param_set_m_enable | 1: Enable 0: Disable |
| Dump memory register values | Get | wlan_read_mcr | Must prepare 4K size buffer for output string |
| Dump baseband register values | Get | wlan_read_bcr | Must prepare 4K size buffer for output string |
| Description on WLAN configuration | Get | wlan_read_config | Must prepare 4K size buffer for output string |
| Description on WLAN information | Get | wlan_read_info | Must prepare 4K size buffer for output string |
| Description on wireless statistics | Get | wlan_read_stat | Must prepare 4K size buffer for output string |
| Dump EEPROM value | Get | wlan_read_eeprom | Must prepare 4K size buffer for output string |

Note: RF Testing Mode command should be the last command for each testing.

### Special Notes

- Please read the appendix A,B,C, D and E first.

- All functions listed in wlan_lib.h should be mutual exclusive from each other.

- Implementation on wlan_kal.h:

  - wlan_kal.h should include headers related with kal_skb_t. All other headers, such as PCI or driver initial, should only be used in wlan_os.c.

  - Since IPN2220 driver SDK does not touch kal_net_device_t, kal_pci_dev_t and kal_dma_addr_t’s structure. You may define they as unsigned long in wlan_kal.h.

  - Use macro as less as possible. Using function calls instead of macro eliminates the need to recompile IPN2220 driver SDK when system library header changes.

  - If the target system does not plan to support unload IPN2220 driver, kal_pci_free_consistent can be defined as null statement.

  - If kal_skb_t has no issues on cache flushing, you may define kal_pci_unmap_single as null statement.

  - You may implement kal_mdelay and kal_udelay as busy wait. You don’t need to use semaphore or other sleep mechanism.

- DMA operation and kal_skb_t:

  - It’s suggested to set PCI latency as 0x20.

  - The starting address of allocated memory must be 4byte-wise boundary.

  - IPN2220 assume net packet’s payload is virtually, and physically continuous, which is different from Windows’s NDIS model.

  - You should arrange kal_skb_t’s payload as non-cacheable. If not, you should implement cache flushing in kal_pci_map_single and kal_pci_unmap_single.

  - IPN2220 driver only use kal_pci_map_single and kal_pci_unmap_single to map packets’ payload.

  - kal_alloc_skb() should return a kal_skb_t-typed pointer. Its payload should be double word aligned. The net packets sent to i2220ap_hard_xmit() should have word-aligned payload.

  - kal_pci_alloc_consistent should allocate and return a non-cacheable virtual address. It should return the mapped bus address via the parameter, dma_p.

  - The starting address of the received Ethernet packet header (translated from wireless packet) is 2byte-wise boundary, not 4byte-wise boundary.

- Performance tuning:

  - When WLAN chip uses 54M bps physical rates, a WLAN frame with 1500K bytes payload needs 280 microseconds to across air. Therefore, the average interval between calling i2220ap_hard_xmi() should be smaller than 280 microsecond. Usually, calling i2220ap_hard_xmit() only costs 150 microsecond or even shorter.

  - If there is 802.11B device continuously transmitting packets in the same channel, IPN2220’s TCP performance should be around 12M. If this 802.11B device is only present but not transmitting, TCP performance should be 16M. When in the pure G mode, it should be able to achieve 20M.

  - We suggest to put packets received by Ethernet in a queue first. Then, forward packets to wireless LAN port in a thread context. Do not call i2220ap_hard_xmit() directly in Ethernet’s receiving done interrupt context, because this impacts on performance.

  - (V1.37 or later ) You may use param_get_curTimeStamp() to acquire wireless LAN chip’s timer. The unit is a microsecond. You may use it to evaluate how long a function call costs.

  - You should return kal_alloc_skb() and kal_free_skb() in a constant timer. We suggest you to keep a packet pool when the system boots up.

- Multiple-entry prevent and driver life cycle

  - All functions cannot be preempted, including timer function. You should do necessary protection before calling functions listed in wlan_lib.h

  - Because IPN2220 driver SDK uses Ethernet frame’s beginning 14bytes to store rfc1042 header{ 0xaa, 0xaa, 0x03, 0x00, 0x00, 0x00 }, which destroys the original Ethernet frame. Bridge-layer should clone Ethernet frame first before calling i2220ap_hard_xmt(), if this packet is not only forwarded to WLAN.

  - Before V1.36(included), IPN2220 driver can works as a switch only. That means IPN driver handles station-to-station traffic within the same bssid. If your bridge layer protocol stack assumes wireless LAN driver indicates all received data frames and the driver does not handle station-to-station forwarding, you need to modify your bridging layer. After V1.37(included), you may use param_set_noRelay() to set IPN2220 driver as data pipe mode, your bridging layer can then handle station-to-station traffic by itself.

- Management frame KAL logic and WPA:

  - kal_auth_request and kal_assoc_request must return a 802.11 status code. This status code is put in a response frame to be sent to wireless LAN stations. If you implement access control list or other functions in the above callbacks, it must be done quickly.

  - Although IPN2220 Driver SDK indicates a station’s all information elements, it does take care of rate parsing. Usually, kal_assoc_request() only needs to handle WPA information element.

  - According to 802.1X , after a successful association, the 802.1X port should be closed. When kal_assoc_request() determines to returns success, it should call cmd_macPort_ctl() to close 802.1X port.

  - kal_disassoc_indicate() is invoked when a station sends disassociate, deauthenticate frames or the driver releases station record.

- WDS

  - The newly created net_device, say dev_new, should be mapped to the same priv value as man device’s before calling param_add_wdslink2(). That means kal_dev_priv(dev_new) should be equal to kal_dev_priv(main_dev).

  - Because WDS has no handshake mechanism, there is no way to tell the remote AP is a 802.11B or 802.11G one. IPN2220 driver assumes it’s 802.11g one. If you know it’s a 802.11B AP, using param_set_wds_rateMode() to set its running protocol can improve performance.

Miscellaneous

- Please refer to the file skbuff.h in Linux kernel source to learn about functions skb_put, skb_push and skb_pull. Or the simplified version, linux_comp.c.

## Wireless Security Setting:

- Security includes Authentication and Encryption

- Authentication and Data Encryption

|  | No | WEP64 | WEP128 | WEP152 | TKIP | AES | Radius Server |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Open |  |  |  |  |  |  |  |
| Open with 802.1x |  |  |  |  |  |  |  |
| Shared Key |  |  |  |  |  |  |  |
| WPA |  |  |  |  |  |  |  |
| WPA-PSK |  |  |  |  |  |  |  |

## A. IPN2220 Driver Life Cycle

System programmer means user of IPN2220 Driver SDK. IPN means IPN2220 Driver SDK. Following order is chronological.

PCI Device probe and resource allocation

  - System programmer takes responsibility to probe IPN2220 device.(Vendor ID: 0x17FEUL, Device ID: 0x2220UL)

  - After matching Vendor ID and Device ID, system programmer should arrange memory base for IPN2220, and set PCI configuration space 0x18(BAR2) accordingly.

  - Usually, OS needs to setup a network device interface to represent IPN2220 WLAN Adaptor, say net_dev.

  - Then, System Programmer is able to call i2220ap_priv_create with kal_pci_dev_t *pdev, kal_net_device_t *netdev and UINT32 membase. These parameters will be memorized and may be used later. For example, pdev is used as the first parameter in kal_pci_alloc_sz, membase is used as the first parameter in kal_pci_readl.

  - i2220ap_priv_create returns a pointer representing IPN2220 driver object, you should keep record of it, and implement kal_dev_priv(wlan_kal.h) in such a way that it’s possible to retrieve IPN2220 driver object by giving net_dev

  - Then, System Programmer is able to call int i2220ap_pci_probe(kal_net_device_t *dev). In SDK, kal_dev_priv (dev) is used to retrieve the driver object back.

Interrupt Handling:

- System Programmer takes responsibility to register his/her own ISR, say kal_hw_interrupt, with respective OS environments. kal_hw_interrupt should clear necessary PCI bus interrupt. IPN2220 only clear IPN2220 chip’s interrupt status.

- When System Programmer’s kal_hw_interrupt is invoked by OS, the programming flow should call i2220hw_interrupt() with paramer = IPN2220 Driver object. If the return value is true, then the interrupt is generated by IPN2220 chip, else not.

- If the interrupt is generated by IPN2220 chip, System Programmer may choose to call do_isr_tasklet with IPN2220 driver object as parameter in the interrupt context or defer to call do_isr_tasklet in kernel thread context.

- During do_isr_tasklet, IPN2220 calls no kal_malloc or kal_free function, but calls kal_alloc_skb, kal_free_skb, kal_netdev_rx, kal_auth_request, kal_assoc_request, kal_tkip_micFailure, kal_assoc_request, kal_disassoc_indicate, kal_indicate_wds_tx_up, kal_indicate_wds_tx_down.

Timer:

- System Programmer take responsibility to register his/her timer function with respective OS environments. When timer function is invoke by OS, it should call IPN2220’s do_timer_check.

- IPN2220 maintains its own timer list, and assumes do_timer_check to be invoked at least 5 times per second.

Data Path:

- Please note IPN2220 supports switch mode by default, although  data-pipe mode is available after V1.37. That means IPN handles Station-to- Station traffic, and will not indicate this kind of data packet upward.

Transmit:

- System Programmer should call i2220ap_hard_start_xmit to send packet.

- kal_skb_t is assumed to be Ethernet format, and kal_skb_data(skb) should point to the beginning of Ethernet destination MAC address.

- To comply with WLAN header format and zero-copy policy, IPN2220 change contents of the socket buffet. And therefore, System programmer should not share the same net packet with other network interface anymore.

- Currently, network packet is assumed to be virtually and physically continuous. If OS’s network packet does not meet both requirements, System Programmer needs to allocate a virtually and physically continuous structure, and copy contents into it, before send it to i2220ap_hard_start_xmit.

- IPN2220 calls kal_free_skb when transmission complete interrupt is generated by IPN2220 chip, and this happens in do_isr_tasklet() context.

Receive:

- IPN2220 calls kal_alloc_skb during driver initializing and do_isr_tasklet. Each timer IPN2220 ask for skb with size = RX_SKB_BUF_SZ(wlan_kal.h). It’s better to be around 1600.

- It indicates new packet upward via kal_netdev_rx. Indicated frame is Ethernet format, and kal_skb_data points to the beginning of destination MAC address.

- It’s System Programmer’s responsibility to free the indicated sk_buffer, after IPN2220 calls kal_netdev_rx.

- Please note, kal_skb_data is not the same position as originally allocated, because IPN2220 received WLAN frame and transformed it into an Ethernet frame.

## B. How to Support Legacy WEP

Note:

1. Return value of 0 means successful

2. All functions are called by OS.

Steps of setting WEP functions.

1. Set 802.11 Authentication mode

typedef enum {

AuthMode_OPEN     = 0,

AuthMode_SHARED   = 1,

AuthMode_SWITCH   = 2,

} param_authMode_t;

int param_set_authmode(kal_net_device_t *dev, param_authMode_t data)

2. Set Cipher Mode.

typedef enum {

CipherMode_NONE   = 0,

CipherMode_WEP40  = 1,

CipherMode_WEP104 = 5,

CipherMode_WEP128 = 6

} param_cipherMode_t;

int param_set_cipherMode(kal_net_device_t *dev, param_cipherMode_t data, int unicast)

Note: Parameter unicast is always 1. After V1.37, param_set_cipherMode(kal_net_device_t *dev, param_cipherMode_t data, int unicast) is changed to param_set_cipherMode(kal_net_device_t *dev, param_cipherMode_t data).

3. Set WEP default key id.

int param_set_wepDefaultKeyId(kal_net_device_t *dev, int data)

Parameter: data = 0, 1, 2, 3

4. Indicate which key id you want to add key.

data = 0, 1, 2, 3

int param_set_wepKeyId(kal_net_device_t *dev, int data)

5. Add key body

int param_set_wepKey(kal_net_device_t *dev, param_cipher_key_t *data)

Note: If you wish to add more than one key, repeat 4 and 5

## C. How to Support WPA

Note:

1. All functions with a name beginning with kal are called by IPN2220 driver.

2. All functions with a name beginning with cmd are called by OS.

3. All functions are assumed to be blocking calls.

4. Upper layer should de-authenticate all associated stations when cipher mode or security key is changed.

5. You should call param_set_cipherMode(dev, CipherMode_NONE) and param_set_authmode(dev, AuthMode_OPEN) before using WPA security.

Steps of setting WPA functions.

1. Set AP’s WPA capibility

int cmd_set_RsnIE(kal_net_device_t *dev, unsigned char *ie, int size)

Parameter: ie: Beginning of WPA Information Element

Note: When you change security and wishes to use WPA no more, please call cmd_set_RsnIE() with size == 0.

2. Add global key.

int cmd_set_macKey(kal_net_device_t *dev, unsigned char *macAddr, cmd_key_t *key)

typedef struct cmd_key_s {

unsigned char   keyId;          /* 0 ~ 3 */

unsigned char   cipherMode;     /* param_cipherMode_t */

unsigned char   keyLen;         /* max 32 */

unsigned char   key[32];

} cmd_key_t;

Please note.

1. You should set group key before any station associate.

2.If macAddr is a broadcast address, then AP's cipher mode is also changed to this key's cipher mode.

3. Station authenticates with AP

int kal_auth_request(kal_net_device_t *dev, unsigned char *macAddr);

Return value: return value should be 802.11 Status Code, ie 0 means success.

4. Station associates with AP

int kal_assoc_request(kal_net_device_t *dev, unsigned char *macAddr, unsigned char *infoElem, int infoLen);

Parameter: infoElem is the beginning of station's information element

Return value: return value should be 802.11 Status Code, ie 0 means success.

Do not send 802.1X packet when kal_assoc_request() is invoked, because association reponse is not sent yet.

5. Close 802.1X port.

int cmd_macPort_ctl(kal_net_device_t *dev, unsigned char *macAddr, int block)

Parameter: block: 0 for not blocking, 1 for blocking

Upper layer should command close port right after assoc_request.

6. 802.1X frame exchange.

802.1X frame is sent upward and downward via the original data path

7. Add station key.

int cmd_set_macKey(kal_net_device_t *dev, unsigned char *macAddr, cmd_key_t *key)

typedef struct cmd_key_s {

unsigned char   keyId;          /* 0 ~ 3 */

unsigned char   cipherMode;     /* param_cipherMode_t */

unsigned char   keyLen;         /* max 32 */

unsigned char   key[32];

} cmd_key_t;

8. TKIP mic failure indication

void kal_tkip_micFailure(kal_net_device_t *dev, unsigned char *macAddr);

Note: Upper layer should record and process proper counter measure. If the upper layer decides to block traffic, please handle properly in upper layer, and call param_set_noRelay(net_dev_p, 1) to stop wireless-to-wireless forwarding. And if system wishes to accept stations no more, param_set_noStation() can be used.

9. When station is disassociated from or disassociate with AP

void kal_disassoc_indicate(kal_net_device_t *dev, unsigned char *macAddr);

10. Deauthenticate station

int cmd_deauth_sta(kal_net_device_t *dev, unsigned char *macAddr) or

int cmd_deauth_sta_with_reason(void *dev, unsigned char *macAddr,int reason_code)

Please Note: If for any reason, upper layer wishes to deauthenticate a station, it may use this function. But, please be careful than IPN2220 calls kal_disassoc_indicate right after. If system wishes to deauthenticate all stations, macAddr should be FF:FF:FF:FF:FF:FF.

11. Clear key

int cmd_set_macKey(kal_net_device_t *dev, unsigned char *macAddr, cmd_key_t *key)

Set key-> cipherMode as CipherMode_NONE. Then you can clear unicast key, if a unicast MAC address is provided. Or you can clear a group key, if a group MAC address is provided.

## D. How to Support WDS

Architecture:

IPN2220 does not follow other company’s method to ask System Programmer to put remote AP’s MAC address in the beginning of each out-going packet. Because its has the draw back of copying data payload.

Instead, each packet is still Ethernet format.

How does IPN2220 know which remote Access Point to go? It utilizes the net_dev pointer. By assuming a unique net_dev pointer value mapping to a unique remote AP, IPN2220 use the same data path to support WDS, which are i2220ap_hard_start_xmit, and kal_netdev_rx.

Setting:

- Create a device to represent the remote AP. Also, its dev priv part should be the same with the return value when you call i2220ap_priv_create in the beginning.

- Use param_add_wdslink2 to add a remote AP. You should provide main_dev, which must be the same with the one when you call i2220ap_priv_create, and the newly created net_dev.

Transmit:

- Assume net_dev_1 represent AP1, you should call i2220ap_hard_start_xmit(net_dev_1,skb);

Receive:

- When remote AP1 sends packet to this AP, IPN2220 indicate this frame by calling kal_netdev_rx(net_dev_1,skb)

- IPN2220 indicate upward all frame received from the remote AP, it will not do forwarding within IPN2220 Driver SDK. System Programmer should handle this in bridging layer.

## E. How to Support Multiple SSID

Introduction:

IPN2220AP supports 8 SSIDs, including a primary SSID(its index is 0). Through utilizing hidden SSID, AP can specify each station in different SSID according to information in the associate request.

IPN2220AP driver does not forward unicast data frames between stations in different SSID, but indicates this kind of data packets to OS. OS can determine whether to forward this packer or not.

Sequence of use:

- Before calling param_set_m_ssid, param_set_m_vlanid, param_set_m_vlantag_enable or param_set_m_enable, you must use param_set_ssid_id to specify which SSID. IPN2220 allows you to specify 0 to 7.

- For example, if we wish to create 2 SSID, say ssid-test0(VLAN-ID: 3) and ssid-test1(VLAN-ID: 4). Each shall indicate data packets to system with VLAN-tags.

- param_set_ssid_id(dev,0);

- param_set_m_ssid (dev,0);

- param_set_m_vlanid(dev,3);

- param_set_m_vlantag_enable (dev,1);

- param_set_m_enable (dev,1);

- param_set_ssid_id(dev,0);

- param_set_m_ssid (dev,0);

- param_set_m_vlanid(dev,3);

- param_set_m_vlantag_enable (dev,1);

- param_set_m_enable (dev,1);