TECHNICAL NOTE

Project:	LSI ADSL product family - All

Title:	OAM External interface

Author:	Stephen Hill

Date:	24th September 2001

Goals of this document

Explain the external interfaces of the OAM driver/interface.

The OAM driver will only be called within the kernel, using standard C function calls.

Proc Interface

The OAM driver installs a read-only proc entry thus /proc/net/oam.

This file gives useful information about the OAM driver and its status. It only displays channels which have been activated for OAM processing.

This is the format:

ADSL adaptor <ADSL_state>

<vpi> ,<vci> <CC><LOC><AIS_state>

Where:

- <ADSL_state> is either “ERROR” or “OK”. It shows the general state of the ADSL interface on the unit.

- <vci> and <vpi> are delimited by a comma and <vci >will always end with a space. <vpi> is left justified and filled by blanks to make up 3 characters widths. <vci> is left justified and filled by blanks to make up 4 characters widths.

- <CC> is always 1 character. CC is continuity check enabled flag. 0x01 for seg_CC enabled, 0x02 for ete_CC enabled.

- <LOC> is always 1 character. LOC is the Loss Of Continuity flag. 0x01 for seg_LOC, 0x02 for ete_LOC.

- <AIS_state> is always 1 character. This is the AIS state of the unit. 0x01 for seg_AIS, 0x02 for ete_AIS. It is non-zero when AIS cells are being received.

E.g. seg_CC enabled and seg_LOC for 0,3, ete_CC and seg_CC enabled for 0,34 – seg_CC failed, seg_AIS and ete_AIS state on 0,35,ete_CC enabled on 10,37 - continuity OK.

ADSL adaptor OK

0  ,3    111

0  ,4    ---

0  ,33   ---

0  ,34   311

0  ,35   --3

0  ,36   ---

10 ,37   2--

External C Interface

All the external interfaces for the ABM driver exist in “oam_ext.h”.

int oam_init();

This initialises OAM processor capability.

void oam_exit();

To be called to shutdown OAM processing.

int oam_process_cell(unsigned int id, void *pCellPayload, unsigned char pti);

Process a received OAM cell. Detection of OAM cells are done prior to calling this function.

int oam_channel_enable(unsigned int id, , oamFlow_t flowType);

Enable a channel for oam processing. The id is a SAR driver reference. Flow type is the type of oam channel; oamFlow_t is defined in “oam_ext.h”.

unsigned char oam_is_channel_enabled(unsigned int id);

Used to determine if channel has been enabled for OAM processing.

void oam_user_cell_seen(unsigned int id);

Hook to call when a user cell is received on a particular channel. Needed to reset the sending of RDI cells.

int oam_adsl_adaptor_defect_set();

Hook to call when ADSL interface is not functioning correctly. This tells the OAM driver to start sending RDI cells on all channels.

int oam_adsl_adaptor_defect_clear();

Hook to call when ADSL interface leaves error state.

Information

The CPID is currently fixed as "LSI_Getafix_NT_" in the oam_cp_id[] array.

The OAM driver/interface currently supports loopback cells and continuity checks as endpoint and source. Please see ITU-T I.610.