TECHNICAL NOTE

Project:	LSI ADSL product family - All

Title:	ABM External interface

Author:	Stephen Hill

Date:	3rd September 2001

Goals of this document

Explain the external interfaces of the ABM driver.

The ABM driver will only be called by the kernel, using standard C function calls.

Goals of the Linux Driver

To create a Linux driver that can be used to allocate buffer memory in kernel space.

Proc Interface

The ABM driver installs a read-only proc entry thus /proc/driver/abm.

This file gives useful information about the ABM driver and its status.

External C Interface

All the external interfaces for the ABM driver exist in “abm_ext.h”. Offsets are not handled directly by the ABM device, they are handled by the devices using buffers from the ABM. E.g. The ethernet module has some registers that allow it to automatically put Rx packets at an offset to the start of the buffer. This allows us to program the ethernet to allow enough space for LLC/SNAP which must be prepended to the packet in order for it to be sent over ATM.

void* abm_nget_bmup(unsigned int size);

This function call will return a pointer to a virtual address space that contains a pointer to a buffer in the ABM. The size of the buffer allocated will be either 2Kb or 16Kb i.e. setting size to 3000 will return a pointer to a buffer size of 16Kb.

void* abm_get_bmup(unsigned char bType);

This function call will return a pointer to a virtual address space that contains a pointer to a buffer in the ABM. Setting bType to 0 will return a pointer to a buffer of size 16kB. Setting bType to a non-zero value will return a pointer to a buffer of size 2kB.

int abm_free_bmup(void* bmup);

This function will free a previously allocated buffer. A value of zero is returned if successful.

int abm_make_link_bmup(void* bmup_src, void* bmup_dst);

This function will link together two previously allocated buffers. A value of zero is returned if successful.

int abm_copy_bmup(void* bmup, unsigned int nCopies);

This function will copy a previously allocated buffer. A value of zero is returned if successful.