# 11n MP Test Program Guide

1. Goal:

Production calibration` before shipment will focus on TX power tuning, power spectrum side lobe measurement and RX receiving sensitivity.

2. User Interface:

After upload, we need click the “Open” button to open a connection.

In General MP test page, you could choose the test function from Test Item, set the parameters (ex. Channel , Antenna , Bandwith, PHY)of functions, then click Start button to start testing.

You could change some parameters (Channel, TX Power, Antenna and DataRate) during the test.

Click the Stop button to stop the test, and the result will report in TX Count, RX OK, RX Error.

Click the Reset button to clear the result.

Below commands are the main functions in a General RF MP test:

Packets TX: Packet TX Test

In this function, we could set Channel, TX power, Antenna, DataRate, TX packet count.

If you check infinitely Packet TX, the test does not stop until pushing stop button.

Packets RX: Packet RX Test

Only two parameters, Channel and Antenna, were used in the function.

Continuous TX SingleTone: Single Tone TX Test (only on CCK mode)

In this function, we could set Channel, TX power, Antenna, and DataRate.

But DataRate Parameters should be: 1, 2, 5.5 or11.

Continuous TX:

In this function, we could set Channel, TX power, Antenna, and DataRate.

Carrier Suppression TX: Continuous TX Test Carrier Suppression (only on CCK mode) In this function, we could set Channel, TX power, Antenna, and DataRate.

But DataRate Parameters should be: 1, 2, 5.5 or 11.

Parameters:

Channel: 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14.

TX power: 00~35.

Antenna: antenna a, b and ac when in any TX mode,

Antenna a, b, ab and ab when in RX mode.

DataRate: 1 Mbps, 2 Mbps, 5.5 Mbps, 11 Mbps. (CCK)

6Mbps, 9Mbps, 12 Mbps, 18Mbps, 24 Mbps, 36 Mbps, 48 Mbps, 54 Mbps. (OFDM)

MCS0, MCS1, MCS2, MCS3, MCS4, MCS5, MCS6, MCS7, MCS8, MCS9, MCS10,

MCS11, MCS12, MCS13, MCS14 and MCS15.

Bandwidth: default is 20M or 40M.

GI: default is long or short.

PHY: AntBdiff (Antenna C differentence).

TX packet count: Total TX packet number. (0 is infinite)

Flash page provides an interface for reading or writing the WLAN parameters in flash.

We could read/write the IP Address, Netmask, Gateway, MACAddress, SSID, Region and Power of each channel

MAC Address: ex: 00e0818600d3

Region: FCC, IC, ETSI, SPAIN, FRANCE or MKK

Power of (channel 1(low), channel 7(Mid), channel 13(high))

On this example, Channel 1 power =10, channel 7 power =9, channel 13 power=8

P.S: HT-1 Stream refers to power of the first of wireless special stream by each low ,mid and high group channel.

HT-1 Stream Difference  refers to the difference of  first and second of wireless special stream by each low, mid and high group channel.

L-OFDM Difference refers the difference of OFDM and MCS by each low, mid and high group channel.

HT20M Difference refers 20M and 40M by each low ,mid and high channel.

(A,B) refers to antenna A or B.

Setup procedure

  - Realtek Bootcode:

---Press button to escape to boot code until the led is light.

---Use the Ethernet upload procedure to upload the image “nfjrom” to the tftp server in bootcode

---After uploading, the program will be executed automatically, and use the windows interface to control .

  - Not Realtek Bootcode:

---Use the file “nfjrom” and upload this image on memory location 0x80300000, and program entry point is also on 0x80300280.

  - Ethernet setting

Use the IP 192.168.1.XXX and the netmask 255.255.255.0.

  - Notes:

After MP program, a BOOT command is recommended and the Linux kernel will be re-boot to check whether the flash Read/Write setting will take effect.