Network Camera HTTP API

(CGI commands)

V2.16

 Copyright Digital Data Communications All rights reserved, 2007.

This document contains information of a proprietary nature. ALL INFORMATION CONTAINED HEREIN SHALL BE KEPT IN STRICT CONFIDENTIAL. None of this information shall be divulged to persons other than Digital Data Communications employees authorized by the nature of their duties to receive information, or individuals and organizations authorized by Digital Data Communications in accordance with existing policy regarding release of company information.

1.	User-level CGI commands (user level privilege)	4

1.1.	Video and Image commands	4

1.1.1.	Stream MPEG-4 video and audio	4

1.1.2.	Stream M-JPEG video	4

1.1.3.	Mobile phone snapshot	5

1.1.4.	Snapshot	6

1.1.5.	SDP (MPEG-4 video only, not for MJPEG video)	6

1.1.6.	Audio Upload (uploading audio streaming to the camera)	6

1.1.7.	RTP/RTSP (MPEG-4 video only, not for MJPEG video)	7

1.2.	Player commands	7

1.2.1.	Query	7

1.3.	I/O port commands	8

1.3.1.	Control I/O ports	8

1.3.2.	Query I/O port status	8

1.4.	Day/Night switch (IR filter) commands	9

1.4.1.	Control day/night switch	9

1.4.2.	Query day/night switch status	9

1.5.	P/T commands (for supporting RS-485 devices models)	9

1.5.1.	Move	9

1.5.2.	P/T speed	10

1.5.3.	Preset positions	10

1.5.4.	User-defined Home	11

1.5.5.	Patrol	11

2.	Admin-level CGI commands (administrator level privilege)	13

2.1.	Query FW version	13

2.2.	Reboot	13

2.3.	Restore to factory defaults	13

2.4.	Get/Set group parameters	13

2.4.1.	Get group parameters	13

2.4.2.	Set group parameters	14

2.5.	Upgrade firmware	14

2.6.	Upload/Download configuration file	15

2.6.1.	Download configuration file	15

2.6.2.	Upload configuration file	15

2.7.	Download logs	15

2.8.	Motion detection commands	15

2.8.1.	Get Motion Detection Settings	16

2.8.2.	Set Motion Detection Settings	17

2.8.3.	Motion Vector Data	18

2.9.	Padding Data Format	18

3.	Appendices	20

3.1.	Appendix A - Base-64 Encoder/Decoder Sample Codes	20

| Revision History |  |  |
| --- | --- | --- |
| Rev | Date | Description |
| V2.16 | 5/23/2007 | Release to Nuuo |

# User-level CGI commands (user level privilege)

## Video and Image commands

### Stream MPEG-4 video and audio

Method: GET

URL: http://<ip>/img/video.asf

Return:  A “video.asf” file will be returned through the HTTP protocol.

HTTP/1.0 200 OK\r\n … content-type: application/octet-stream\r\n … <MPEG-4 video data>

### Stream M-JPEG video

#### HTML page for the end user

Method: GET

URL: http://<ip>/img/mjpeg.htm

Return:

HTTP/1.0 200 OK\r\n … content-type: text/html\r\n … http://<ip>/img/video.mjpeg …

#### Server Push page for the programmer

Method: GET

URL: http://<ip>/img/video.mjpeg[?size=value][&framerate=<value>] (utility)

http://<ip>/img/mjpeg.cgi[?size=value][&framerate=<value>] (OCX)

(The “http://<ip>/img/mjpeg.cgi” command will be obsolete by in the future. Some Web Browsers will cache the first incoming JPEG image and do not refresh the following incoming JPEG images. Please use the  URL to stream JPEG video in these kinds of Web Browsers.)

This MJPEG CGI supports different types of client request. Not only for Server Push method, but also for the proprietary MJPEG streaming. If the client is requesting for the Server Push MJPEG format (with Multipart MIME format), the “User-Agent” field in the HTTP header is required. The “User-Agent” should avoid the “CameraActiveX”, “Viewer” and “AlertCfg” wordings

| Parameter | Value and description |
| --- | --- |
| size | Image resolution 1: 160x120 (or, 160x128 depends by models) 2: 320x240 3: 640x480 |
| framerate | Frame rate 1 ~ 30 |

The parameters “size” and “framerate” are optional.

The camera will return the MJPEG streaming within the current codec settings if there’s no parameter input.

The camera will check the request parameters (size or framerate) within the current codec settings. If the requested parameters matched the current setting, then the camera will return the MJPEG stream by Server Push method. If the requested parameters didn’t match the current setting, then the camera will return an error.

Return (OK situation):

HTTP/1.0 200 OK\r\n … content-type: multipart/x-mixed-released;boundary=<xxx>\r\n --<xxx> content-type:image/jpeg\r\n content-length: <image-size>\r\n\r\n <jpeg image date> --<xxx> content-type:image/jpeg\r\n content-length: <image-size>\r\n\r\n <jpeg image date> ... --<boundary>--

Return (ERROR situation):

HTTP/1.0 400 Bad Request\r\n … content-type: text/plain\r\n \r\n current_ resolution=A\r\n current_framerate=B\r\n

The A and B are in the following format.

| Parameter | Value and description |
| --- | --- |
| A | Image resolution 1: 160x120 (or, 160x128 depends by models) 2: 320x240 3: 640x480 |
| B | Frame rate 1 ~ 30fps |

### Mobile phone snapshot

Method: GET

URL: http://<ip>/img/mobile.cgi

Return:  A 160x120 JPEG image (with “very low” quality) will be returned to client.

HTTP/1.0 200 OK\r\n … content-type: image/jpeg\r\n … <JPEG image data>

Here is an example of the HTML file

Method: GET

URL: http://<ip>/img/mobile.htm

<html> <head> <title>Network Camera - Live Video</title> <META HTTP-EQUIV="Pragma" CONTENT="no-cache"> <META HTTP-EQUIV="Cache-Control" CONTENT="no-cache"> </head> <body> <IMG SRC="/img/mobile.cgi"> </body> </html>

### Snapshot

Method: GET

URL: http://<ip>/img/snapshot.cgi?[size=<value>][&quality=<value>]

| Parameter | Value and description |
| --- | --- |
| size | Image resolution 1: 160x120 (or, 160x128 depends by models) 2: 320x240 3: 640x480 4: 720x480 (reserved for future use) 5: 1024x768 (reserved for future use) 6: 1280x1024 (reserved for future use) 7: 64x48 8: 128x96 |
| quality | Quality level 1: Very high 2: High 3: Normal 4: Low 5: Very low |

Example 1: To sanshot a 640x480 high qulity JPEG image from network camera 192.168.0.99.

http://192.168.0.99/img/snapshot.cgi?size=3&quality=1

Example 2: To sanshot a JPEG image from network camera 192.168.0.99 (with current resolution and quality)

http://192.168.0.99/img/snapshot.cgi

Example 3: To sanshot a low qulity JPEG image with current resolution from the network camera 192.168.0.99.

http://192.168.0.99/img/snapshot.cgi?quality=4

Return:  A JPEG image will be returned to client with user specified resolution and quality.

HTTP/1.0 200 OK\r\n … content-type: image/jpeg\r\n … <JPEG image data>

### SDP (MPEG-4 video only, not for MJPEG video)

Method: GET

URL: http://<ip>/img/media.sdp

Return:  A SDP file will be returned.

HTTP/1.0 200 OK\r\n … <SDP data>

### Audio Upload (uploading audio streaming to the camera)

Method: POST

URL: http://<ip>/img/g726.cgi		G.726 audio stream (16Kbps)

URL: http://<ip>/img/g711a.cgi	G.711 a-law audio stream

URL: http://<ip>/img/g711u.cgi	G.711 u-law audio stream

URL: http://<ip>/img/g729ab.cgi	G.729ab audio stream, reserved for future use

Example (client side):

POST http://192.168.0.99/img726.cgi HTTP/1.0\r\n Authrization: Basic YWRtaW46YWRtaW4= \r\n \r\n

Return:

OK

HTTP/1.0 200 OK\r\n

Client starts to upload the audio stream.

Unauthorized		(Bad username, password)

HTTP/1.0 401 Unauthorized\r\n

### RTP/RTSP (MPEG-4 video only, not for MJPEG video)

User can stream video and audio through the following URLs.

Video and audio:	rtsp://<ip>/img/media.sav

Video only:	rtsp://<ip>/img/video.sav

Audio only:	rtsp://<ip>/img/audio.sav

Note that camera is able to support the following four RTP protocols. But user needs to specify the desired RTP protocol in the player.

- Unicast RTP

- Multicast RTP

- RTP over RTSP (RTP over TCP)

- RTP over RTSP over HTTP (HTTP tunnel)

Return:  Video and/or audio will be returned.

## Player commands

### Query

Method: GET

URL: http://<ip>/util/query.cgi

This CGI indicates the H/W capability. Ex. The camera got I/O port (or not). The camera got Speaker (or not), etc.

Return:

HTTP/1.0 200 OK \r\n … content-type: text/plain\r\n \r\n hostname=sc123456\r\n description=description string\r\n defname=default name\r\n mac=00C002123456\r\n resolutions=640*480,320*240,160*120\r\n mpeg4_resolution=[640¦320¦160]\r\n mjpeg_resolution=[640¦320¦160]\r\n mic_in=[on¦off]r\n speaker_out=[on¦off]\r\n ptctrl=[on¦off]\r\n ioctrl=[on¦off]\r\n serial=[off¦pelco]\r\n

URL: http://<ip>/img/query.cgi

This CGI indicates the accessed user’s privilege with some H/W features. Ex. The user could use Speaker Out, but couldn’t control the I/O ports, etc.

Return:

HTTP/1.0 200 OK \r\n … content-type: text/plain\r\n \r\n mic_in=[on¦off]r\n speaker_out=[on¦off]\r\n ptctrl=[on¦off]\r\n ioctrl=[on¦off]\r\n

## I/O port commands

### Control I/O ports

Method: GET

URL: http://<ip>/io/ioctrl.cgi?<parameter>=<value>[&<parameter>=<value>]

| Parameter | Value and description |
| --- | --- |
| output_1 | Output port 1 0: Off 1: On PS: The ON state and OFF state are defined in the configuration file. |
| output_2 | Output port 2 0: Off 1: On PS: The ON state and OFF state are defined in the configuration file. |

Return: Network Camera will return a Web page that contains the following messages.

HTTP/1.0 204 No Content\r\n … content-type: text/plain\r\n Return-Value: OK\r\n \r\n

### Query I/O port status

Method: GET

URL: http://<ip>/io/query_io.cgi

Return: Network Camera will return a Web page that contains the following messages.

HTTP/1.0 200 OK \r\n … content-type: text/plain\r\n \r\n input_1=[0¦1]\r\n <- 1= High; 0= Low input_2=[0¦1]\r\n output_1=[0¦1]\r\n <- 1= High; 0= Low output_2=[0¦1]\r\n \r\n

## Day/Night switch (IR filter) commands

### Control day/night switch

Method: GET

URL: http://<ip>/io/filterctrl.cgi?<parameter>=<value>

| Parameter | Value and description |
| --- | --- |
| Filter | Switch the filter to the following mode 0: Day Mode 1: Night Mode |

Return: Network Camera will return a Web page that contains the following messages.

HTTP/1.0 204 No Content\r\n … content-type: text/plain\r\n Return-Value: OK\r\n \r\n

### Query day/night switch status

Method: GET

URL: http://<ip>/io/query_filter.cgi

Return: Network Camera will return a Web page that contains the following messages.

HTTP/1.0 200 OK \r\n … content-type: text/plain\r\n \r\n filter=[0¦1]\r\n <- 0= Night; 1= Day \r\n

## P/T commands (for supporting RS-485 devices models)

### Move

Move toward user defined direction or move to preset positions

Method: GET

URL: http://<IP>/pt/ptctrl.cgi?cmd=<direction>,<speed>

| Parameter | Value and description |
| --- | --- |
| direction | Move toward user defined drection U Move UP D Move Down L Move Left R Move Right The P/T will Stop after 1sec of the move action |
| speed | Move Speed (Optional) 1~10 (10 is the fastest speed) |

Return:

HTTP/1.0 204 No Content\r\n … content-type: text/plain\r\n Return-Value: OK\r\n <- ERROR\r\n \r\n

### P/T speed

#### Set the Pan speed.

Set the Pan direction speed

Method: GET

URL: http://<IP>/pt/ptctrl.cgi?cmd=set_pan_speed,<speed>

| Parameter | Value and description |
| --- | --- |
| speed | Move Speed 1~10 (10 is the fastest speed) |

#### Set the Tilt speed.

Set the Tilt direction speed

Method: GET

URL: http://<IP>/pt/ptctrl.cgi?cmd=set_tilt_speed,<speed>

| Parameter | Value and description |
| --- | --- |
| speed | Move Speed 1~10 (10 is the fastest speed) |

Return:

HTTP/1.0 204 No Content\r\n … content-type: text/plain\r\n Return-Value: OK\r\n <- ERROR\r\n \r\n

### Preset positions

#### Set preset position

Set current position to be a preset position (within the preset name)

Method: GET

URL: http://<IP>/pt/ptctrl.cgi?cmd=set_preset,<name>

| Parameter | Value and description |
| --- | --- |
| name | Preset name (16 characters) Currently, it could save up to 9 preset positions |

#### Clear preset position

Delete a specific preset position (by the preset name)

Method: GET

URL: http://<IP>/pt/ptctrl.cgi?cmd=clr_preset,<name>

| Parameter | Value and description |
| --- | --- |
| name | Preset name (16 characters) |

#### Move to the preset position

Move the P/T device to a specific preset position (by the preset name)

Method: GET

URL: http://<IP>/pt/ptctrl.cgi?cmd=goto_preset,<name>

| Parameter | Value and description |
| --- | --- |
| name | Preset name (16 characters) |

Return:

HTTP/1.0 204 No Content\r\n … content-type: text/plain\r\n Return-Value: OK\r\n <- ERROR\r\n \r\n

#### Get the preset lists

Get the name list of all presets

Method: GET

URL: http://<IP>/pt/ptctrl.cgi?cmd=get_preset_list

Return:

HTTP/1.0 200 OK\r\n … content-type: text/plain\r\n \r\n PT1=<name1>\r\n PT2=<name2>\r\n ...... PT9=<name9>\r\n \r\n

### User-defined Home

#### Set home position

Set current position as the user-defined home position

Method: GET

URL: http://<IP>/pt/ptctrl.cgi?cmd=set_home

#### Move to home position

Move to the user-defined home position

Method: GET

URL: http://<IP>/pt/ptctrl.cgi?cmd=goto_home

Return:

HTTP/1.0 204 No Content\r\n … content-type: text/plain\r\n Return-Value: OK\r\n <- ERROR\r\n \r\n

### Patrol

#### Set patrol

Configure the patrol with a sequence of preset positions and a specific delay time (up to 9 preset positions)

Method: GET

URL: http://<IP>/pt/ptctrl.cgi?cmd=set_patrol,<parameter>

| Parameter | Value and description |
| --- | --- |
| parameter | Position & Delay Time for Patrol Action Preset_name,Delay_time;Preset_name,Delay_time;...;Preset_name,Delay_time; Preset_name: The preset name in “set_preset” command Delay_time: The delay time before goto next preset position (in sec) |

#### Clear patrol

Clear the patrol configuration

Method: GET

URL: http://<IP>/pt/ptctrl.cgi?cmd=clr_patrol

#### Start patrol

Start the patrol action (if configured)

Method: GET

URL: http://<IP>/pt/ptctrl.cgi?cmd=goto_patrol

Return:

HTTP/1.0 204 No Content\r\n … content-type: text/plain\r\n Return-Value: OK\r\n <- ERROR\r\n \r\n

# Admin-level CGI commands (administrator level privilege)

## Query FW version

Method: GET

URL: http://<ip>/adm/sysinfo.cgi

Return: Network Camera will return a Web page that contains the following messages.

HTTP/1.0 200 OK \r\n … content-type: text/plain\r\n \r\n Firmware Version: V1.00R01\r\n Serial Number: SQJ00G100001\r\n Pan and Tilt Control Firmware Version: V1.0.14\r\n

## Reboot

Method: GET

URL: http://<ip>/adm/reboot.cgi

Return:

HTTP/1.0 200 OK \r\n … content-type: text/plain\r\n \r\n OK\r\n

## Restore to factory defaults

Method: GET

URL: http://<ip>/adm/reset_to_default.cgi

Return:

HTTP/1.0 200 OK \r\n … content-type: text/plain\r\n \r\n OK\r\n

## Get/Set group parameters

### Get group parameters

Method: GET

URL: http://<IP>/adm/get_group.cgi?<parameter>=<value>

| Parameter | Value and description |
| --- | --- |
| group | 1. To get all group settings group_name (The group name is case insensitive.) 2. To get a specific setting from a group group_name.parameter_name Note that all group_name and parameter_name are defined in the Network Camera Configuration Spec. |

Return:

All group settings or one group setting will be returned as below.

HTTP/1.0 200 OK\r\n … content-type: text/plain\r\n ... \r\n <parameter pair>\r\n <parameter pair>\r\n ...\r\n

Where <parameter pair> is <parameter_name>=<parameter_value>.

### Set group parameters

Method: GET

URL: http://<IP>/adm/set_group.cgi?<parameter>=<value>[&<parameter pair>…]

| Parameter | Value and description |
| --- | --- |
| group | Group name is specified here. (The group name is case insensitive.) Note that all group values are defined in the Network Camera Configuration Spec. |

Return:

Successful request returns all group parameters or the specified parameters as below.

HTTP/1.0 200 OK\r\n … content-type: text/plain\r\n ... \r\n OK\r\n <parameter pair>\r\n <parameter pair>\r\n ...\r\n

## Upgrade firmware

Method: POST

URL: http://<ip>/adm/upgrade.cgi

Message body:

… … <boundary=...> … <FW binary data encoded in base64 format>

The client must wait as least 5 minutes to make sure all flashes have been programmed.

Return: A FW Upgrade Progress window will be returned.

## Upload/Download configuration file

### Download configuration file

Method: GET

URL: http://<ip>/adm/admcfg.cfg

Return:

HTTP/1.0 200 OK \r\n … content-type: application/configuration\r\n \r\n <configuration file encoded in Base64 format>

All data in the configuration file are encoded in a Base64 format. Please refer to “3.1”.

### Upload configuration file

Method: POST

URL: http://<ip>/adm/upload.cgi

Message body:

… <boundary=...> <configuration file encoded in Base64 fvormat>

All data in the configuration file are encoded in a Base64 format. Please refer to “3.1”.

Network camera will be automatically rebooted after received the configuration file.

Return:

HTTP/1.0 200 OK \r\n … content-type: text/plain\r\n \r\n OK\r\n

## Download logs

Method: GET

URL: http://<ip>/adm/log.cgi

Return:

HTTP/1.0 200 OK \r\n … content-type: text/plain\r\n\r\n … <log data>

## Motion detection commands

The user could configure the motion detection in max up to 4 specific area. Each area could have its own name and its own threshold setting. Once the moving indicators exceed the user-configured threshold, then it should have a motion detection event.

Here’s the definition of the detected area coordinate.

For some reasons, the client S/W might need to backward support the motion detection CGI and padding data within the old models of camera. Please refer the appendix section to get the details..

The home (0, 0) position is located at the left-top corner. The whole scale is for 640x480 resolution. So the horizontal position, x, will be expressed as an integer value from 0 to 639 and ascend from left to right. The vertical position, y, will be expressed as an integer value from 0 to 479 and ascend from top to bottom.

640

+-------------------+

|(0, 0)             |

|                   |

|                   |480

|                   |

|                   |

+-------------------+

(639, 479)

The configured area coordinate:

The left-top corner is the start position. The right-bottom corner is the stop position.

(X1, Y1)

+ - - - - - - - +

|                 |

|                 |

+ - -- - - - - +

(X2, Y2)

Whatever the current streaming resolution is, the replied coordinate always maps to the 640x480 scale.

### Get Motion Detection Settings

Method: GET

URL: http://<IP>/adm/get_group.cgi?group=MOTION

(Please refer to Get group parameters command.)

| Parameter | Value and description |
| --- | --- |
| group | Motion This is a required paramter & value to let the CGI know the request is for “motion detection” group. |

Return:

All group settings or one group setting will be returned as below.

HTTP/1.0 200 OK\r\n … content-type: text/plain \r\n ... \r\n md_mode=[0¦1] \r\n  0=Off, 1=On md_switch1=[0¦1] \r\n  0=Off, 1=On md_name1=Window_1 \r\n  12 ASCII characters md_window1=X1, Y1, X2, Y2 \r\n  Relative to 640x480 resolution md_threshold1=125 \r\n  Threshold: 0 ~ 255 md_switch2=[0¦1] \r\n md_name2=Window_2 \r\n md_window2=X1, Y1, X2, Y2 \r\n md_threshold2=125 \r\n md_switch3=[0¦1] \r\n md_name3=Window_3 \r\n md_window3=X1, Y1, X2, Y2 \r\n md_threshold3=125 \r\n md_switch4=[0¦1] \r\n md_name4=Window_4 \r\n md_window4=X1, Y1, X2, Y2 \r\n md_threshold4=125 \r\n \r\n

### Set Motion Detection Settings

Method: GET

URL: http://<IP>/adm/set_group.cgi?group=MOTION&<parameter>=<value>…

(Please refer to Set group parameters command.)

| Parameter | Value and description |
| --- | --- |
| md_mode | Motion detection 0 = Off 1 = On |
| md_switch1 ... md_switch4 | Motion detection window 0 = Off 1 = On |
| md_name1 ... md_name4 | Name of the motion detection area 1 ~ 4 (12 ASCII characters) |
| md_window1 ... md_window4 | The coordinates of motion detection window N (relative to resolution 640x480) X1, Y1, X2, Y2 |
| md_threshold1 ... md_threshold4 | The threshold of motion detection window N (0 - 255) |

Return:

HTTP/1.0 200 OK\r\n … content-type: text/plain\r\n \r\n OK\r\n

Notes:

Linksys 2300 series used a different way to do the motion detection:

- Full Screen:  Detect the whole screen (only 1 640x480 detect area)

- Sub Windows: Detect up to 3 sub-windows area

The S/W (OCX) side needs to do it as the following way (without changing any configure item):

- Use “md_switch1” as the switch of “full screen” or “sub-windows”.

- The md_window1 always sets to 640x480 size.

If “md_switch1=1”, then the camera needs to detect in the whole screen video. The OCX can’t allow the user to configure the rest windows (sub-window 1~3).

If “md_switch1=0”, then the camera will detect in the “md_window2~4” area. (It’s the same as the sub-windows 1~3.)

### Motion Vector Data

To include the motion vector values in the streaming packets.

Whatever the streaming method is - ASF (through HTTP) or RTP (through UDP), the streaming data will include such information to let the client side S/W to judge whether the motion event is triggered or not.

The data locates in the padding bytes of the streaming data.

As to the data format in the streaming packet, please refer to the next section.

## Padding Data Format

The purpose of the padding data field is to let the PC side software (ActiveX or Utility…) could parse the padding data to get the relative information.

The F/W (camera side) should always pad the data if it supports some features (even is disabled).

The following padding data starts from the 1st byte of the padding area (after the normal streaming frames)

Padding format:

| 4 Bytes | 1 Byte | 1 ~ 4 Byte | XXX Byte | 1 Byte | 1 ~ 4 Byte | XXX Byte | …. | 2 Bytes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Total Length | Command_1 | Length | Data | Command_2 | Length | Data |  | Padding End |

Rules:

- The 1st 4 bytes padding is to declaim the total padding length, including these 4 bytes and the “Padding End” (from 1st byte to the last byte, including length and end command).

- The following padding data will divide into 3 parts:

  - Padding command (1 byte)

  - Padding length for the specific command (1~4 bytes)

  - Padding data for the specific command

- The length of the “padding length” depends on the command range.

A. 0x00 ~ 0xBF:		the length field is in “1 byte”

B. 0xC0 ~ 0xDF:		the length field is in “2 bytes”

C. 0xE0 ~ 0xFF:		the length field is in “4 bytes”

- The last 2 bytes are the “Padding End” (0xBF00) command. It equals to the “command=END” + “length=0”.

- The padding command could be in any sequence.

| Command | Description |
| --- | --- |
| 0x01 | Motion Detection Padding The data length is 13. Byte 1: Motion detection is enabled or not (main switch) and whether it is in the effect schedule or not.. 0x00 = 0 = Off 0x01 = 1 = On & not in effect schedule 0x11 = 17 = On & in effect schedule Byte 2 ~ 5 Current motion indicator of the detected window 1 ~ 4 0 ~ 255 Byte 6 ~ 9 Current threshold of the detected window 1 ~ 4 0 ~ 255 Byte 10 ~ 13 Motion detect window 1 ~ 4 is enabled or not. 0 = Off 1 = On |
| 0x02 | Input I/O Ports Padding The data length is 2. Byte 1: The Input port 1 is triggered or not.. and whether it is in the effect schedule or not.. 0x00 = 0 = Idle 0x01 = 1 = Triggered & not in effect schedule 0x11 = 17 = Triggered & in effect schedule Byte 2: The Input port 2 is triggered or not.. and whether it is in the effect schedule or not.. 0x00 = 0 = Idle 0x01 = 1 = Triggered & not in effect schedule 0x11 = 17 = Triggered & in effect schedule |
| 0xBF | End-Of-Padding The data length is 0. There’s no additional padding after this command (0xBF00). |

Examples:

- The padding contains motion detection data only.

| Length |  |  |  | Command | Length | On/Off | Indicator |  |  |  | Threshold |  |  |  | ###### Window - On/Off |  |  |  | End |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x00 | 0x00 | 0x00 | 0x15 | 0x01 | 0x0D | 0 (OFF) | 20 | 30 | 129 | 0 | 128 | 128 | 128 | 128 | 1 | 1 | 1 | 0 | 0xBF | 0x00 |

0x15 = 21 (total length = 21 bytes)

- The padding contains both motion detection and input ports data.

| Length |  |  |  | Command | Length | On | Indicator |  |  |  | Threshold |  |  |  | ###### On/Off |  |  |  | Command | Length | In-1 | ###### In-2 | End |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x00 | 0x00 | 0x00 | 0x19 | 0x01 | 0x0D | 1 | 0 | 3 | 2 | 0 | 128 | 128 | 128 | 128 | 0 | 1 | 1 | 0 | 0x02 | 0x02 | 0 | 0 | 0xBF | 0x00 |

0x19 = 25 (total length = 25 bytes)

- The padding contains some long commands. (Ex. Command 0xC1 needs length 500 data)

| Length |  |  |  | Command | Length |  | Total = 500 bytes |  |  |  |  |  |  |  | ###### End |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x00 | 0x00 | 0x01 | 0xFD | 0xC1 | 0x01 | 0xF4 | Data… | Data | Data | …. | …. | … | … | … | 0xBF | 0x00 |

0x1FD = 509 (total length = 509 bytes)

0x01F4 = 500 (length of command 0xC1 = 500 bytes)

# Appendices

## Appendix A - Base-64 Encoder/Decoder Sample Codes

// Standard BASE64 table // char keyStr[] = "ABCDEFGHIJKLMNOPQRSTUVWXYZabcdefghijklmnopqrstuvwxyz0123456789+/="; // Digital Data Communications BASE64 table char keyStr[] = "ACEGIKMOQSUWYBDFHJLNPRTVXZacegikmoqsuwybdfhjlnprtvxz0246813579=+/"; //-------------------------------------------------------------------- // Description: Encrypt the input data with the base64 // Input: // char i_buf[] - input buffer // Output: // char o_buf[] - output buffer // Return: // encrypted string length //-------------------------------------------------------------------- int encode64(char i_buf[], char o_buf[]) { char chr1 = (char)0; char chr2 = (char)0; char chr3 = (char)0; //These are the 3 bytes to be encoded int enc1 = 0; int enc2 = 0; int enc3 = 0; int enc4 = 0; //These are the 4 encoded bytes int i = 0, j = 0; //Position counter do { //Set up the loop here chr1 = i_buf[i++]; //Grab the first byte if(i < strlen(i_buf)) chr2 = i_buf[i++]; //Grab the second byte if(i < strlen(i_buf)) chr3 = i_buf[i++]; //Grab the third byte //Here is the actual base64 encode part. //There really is only one way to do it. enc1 = chr1 >> 2; enc2 = ((chr1 & 3) << 4) ¦ (chr2 >> 4); enc3 = ((chr2 & 15) << 2) ¦ (chr3 >> 6); enc4 = chr3 & 63; if(chr2 == (char)0) { enc3 = enc4 = 64; } else if(chr3 == (char)0) { enc4 = 64; } o_buf[j++] = keyStr[enc1]; o_buf[j++] = keyStr[enc2]; o_buf[j++] = keyStr[enc3]; o_buf[j++] = keyStr[enc4]; // OK, now clean out the variables used. chr1 = chr2 = chr3 = (char)0; enc1 = enc2 = enc3 = enc4 = (char)0; } while (i < strlen(i_buf)); //And finish off the loop //Now return the encoded values. return j; } //-------------------------------------------------------------------- // Description: decrypt the input data with the base64 // Input: // char i_buf[] - input buffer // Output: // char o_buf[] - output buffer // Return: // decrypted string length //-------------------------------------------------------------------- int decode64(char i_buf[], char o_buf[]) { //These are the 3 bytes to be encoded char chr1 = (char)0; char chr2 = (char)0; char chr3 = (char)0; //These are the 4 encoded bytes int enc1 = 0; int enc2 = 0; int enc3 = 0; int enc4 = 0; int i = 0, j = 0; //Position counter do { //Here’s the decode loop. //Grab 4 bytes of encoded content. enc1 = (int)(strchr(keyStr, i_buf[i++]) - keyStr); if(i < strlen(i_buf)) enc2 = (int)(strchr(keyStr, i_buf[i++]) - keyStr); if(i < strlen(i_buf)) enc3 = (int)(strchr(keyStr, i_buf[i++]) - keyStr); if(i < strlen(i_buf)) enc4 = (int)(strchr(keyStr, i_buf[i++]) - keyStr); //Heres the decode part. There’s really only one way to do it. chr1 = (enc1 << 2) ¦ (enc2 >> 4); chr2 = ((enc2 & 15) << 4) ¦ (enc3 >> 2); chr3 = ((enc3 & 3) << 6) ¦ enc4; o_buf[j++] = (char)chr1; if (enc3 != 64) { o_buf[j++] = (char)chr2; } if (enc4 != 64) { o_buf[j++] = (char)chr3; } //now clean out the variables used chr1 = (char)0; chr2 = (char)0; chr3 = (char)0; enc1 = 0; enc2 = 0; enc3 = 0; enc4 = 0; } while (i < strlen(i_buf)); //finish off the loop //Now return the decoded values. return j; }