Network Camera

Configuration File Spec

V1.1

 Copyright Digital Data Communications All rights reserved, 2007.

This document contains information of a proprietary nature. ALL INFORMATION CONTAINED HEREIN SHALL BE KEPT IN STRICT CONFIDENTIAL. None of this information shall be divulged to persons other than Digital Data Communications employees authorized by the nature of their duties to receive information, or individuals and organizations authorized by Digital Data Communications in accordance with existing policy regarding release of company information.

1.	Configuration file format	4

1.1.	Manufacture group	4

1.2.	SYSTEM group	4

1.3.	LOG group	5

1.4.	NETWORK group	5

1.5.	WIRELESS group	6

1.6.	DDNS group	7

1.7.	HTTP group	7

1.8.	RTSP_RTP group	7

1.9.	UPNP group	8

1.10.	EMAIL group	8

1.11.	FTP group	9

1.12.	VIDEO group	9

1.13.	MPEG4 group	10

1.14.	JPEG group	11

1.15.	Mobile phone group	11

1.16.	AUDIO group	12

1.17.	USER group	12

1.18.	IP_filter group	13

1.19.	MOTION group	13

1.20.	SERIAL group	14

1.21.	PTZ group	14

1.22.	IO group	15

1.23.	EVENT group	15

1.24.	Bonjour group	17

1.25.	QoS group	17

2.	Appendices	19

2.1.	Appendix A - Time Zone List	19

| Revision History |  |  |
| --- | --- | --- |
| Rev | Date | Description |
| V0.1 | 3/6/2006 | Draft release |
| V0.2 | 3/31/2006 | Redefine “output port” and ‘output buttons” Move all internal use configuration settings to the “Manufacture” group. |
| V0.3 | 4/24/2006 | Temp release |
| V0.4 | 4/28/2006 | Add/update the following group parameters COM_port PTZ_CMD |
| V0.5 | 6/15/2006 | Add the “Bonjour” group parameter Need to reference “Spec –SW and FW UI Customization..doc” for Bonjour contents. |
| V.06 | 6/20/2006 | - Add Jabber settings in log group - Add QoS group |
| V0.7 | 7/26/2006 | Modify Video Group: Support both MPEG4 & MJPEG (dual streaming) + Mobile Phone |
| V0.8 | 8/9/2006 | Modify Video/Audio Groups - Separate MPEG4, JPEG and Mobile phone into 3 different groups - Add image upload settings - Add privacy mask settings - Add audio upload settings - Add G.711/G.726/G.729ab selections for audio group Modify the settings for the RS485 feature (COM_port and PTZ_CMD groups) - Add device type settings (protocol) - Add address item for Pelco D protocol Modify Event Group to have “pre-captured” and “post-captured” video length. |
| V0.9 | 8/28/2006 | Add “audio mode” and “operation mode” for audio setting. (This is for some specific models) It got simplex, half-duplex and full-duplex selections. |
| V1.0 | 1/9/2007 |  |
| V1.1 | 2/8/2007 | Add PtzPanSpeed, PtzTiltSpeed and ntp_port |

# Configuration file format

Syntax:

[group1]\n parameter1=value1\n parameter2=value2\n … [group2]\n parameter1=value1\n parameter2=value2\n … [group3]\n parameter1=value1\n parameter2=value2\n …

## Manufacture group

All internal use configuration settings are maintained in this group. Note that this group will not included in the download configuration file.

| Parameter | Values and description |
| --- | --- |
| release_date | FW release date (internal use only, not allowed to change its value). |
| def_name | Default device (host) name |
| default_ip | Default IP address (xxx.xxx.xxx.xxx/yyy.yyy.yyy.yyy) |
| max_user | The maximum user number (20) |
| ddns_last_update | The DDNS last update time |

## SYSTEM group

| Parameter | Values and description |
| --- | --- |
| cfg_ver | Configuration file version number (internal use only, not allowed to change its value). User uploaded configutaion file will be ignored if its version number is not supported by the FW. Restore all settings to the factory default if the uploaded FW do not support the current configuration file version. |
| host_name | Camera name (internal use only, not allowed to change its value) Range: [_0-9A-Za-z] and space character PS: The square brackets are used to describe ranges of characters. |
| comment | Camera description (up to 32 characters) Range: ASCII characters and space character |
| time_format | 24-hour 12-hour (AM/PM) |
| date_format | Date format 0= YYYY-MM-DD 1= MM/DD/YYYY 2= DD/MM/YYYY |
| time_zone | Total 75 time zone areas (0-74, refer to 2) |
| daylight_saving | Auto daylight saving 0= Off 1= On |
| ntp_mode | Synchronize time with the NTP server 0= Off 1= On |
| ntp_server | The NTP server address (up to 64 characters) |
| ntp_date | The NTP synchronization scheduling date 0= Every day 1= Monday 2= Tuesday 3= Wednesday 4= Thursday 5= Friday 6= Staturday 7= Sunday |
| ntp_hour | The NTP synchronization scheduling time (0-23) |
| ntp_min | The NTP synchronization scheduling time (0-59) |
| ntp_port | The NTP server’s port |
| led_mode | LED operation 0= Off 1= On |

## LOG group

| Parameter | Values and description |
| --- | --- |
| log_mode | The system log feature 0= Off 1= On |
| log_level | Log level (0-3, internal use only, not allowed to change its value). |
| syslog_mode | The SYSLOG feature 0= Off 1= On |
| syslog_server | The SYSLOG server address (up to 64 characters) |
| syslog_port | syslog server port |
| im_mode | The jabber feature 0= Off 1= On |
| im_server | The address of instant message server |
| im_account | instant message login name |
| im_password | instant message login password |
| im_sendto | instant message sent-to address |
| ftplog_mode | Enable ftp log mode |
| smtplog_mode | Enable smtp log mode |
| systemlog_mode | Enable system log mode |
| imlog_mode | Enable instant message mode |

## NETWORK group

| Parameter | Values and description |
| --- | --- |
| ip_addr | Current camera IP address (1.0.0.1-255.255.255.254) |
| netmask | Current netmask (128.0.0.0-255.255.255.254) |
| gateway | Default gateway address (1.0.0.1-255.255.255.254) |
| dhcp | Camera IP address mode 0= Fixed IP address 1= DHCP client |
| dns_type | DNS server 0= The DNS server IP addresses are assigned by the DHCP server. 1= User specified DNS server IP addresses |
| dns_server1 | The primary DNS server address ((1.0.0.1-255.255.255.254) |
| dns_server2 | The secondary DNS server address (1.0.0.1-255.255.255.254) |

## WIRELESS group

| Parameter | Values and description |
| --- | --- |
| wlan_type | Wireless network type 0= Ad hoc 1= infrastruture |
| wlan_essid | Wireless SSID (up to 32 ASCII characters, case sensitive) |
| wlan_channel | Channel number (1-14) |
| wlan_domain | The wireless domain 1= AFERICA 2= ASIA 3= AUSTRALIA 4= CANADA 5= EUROPE 6= SPAIN 7= FRANCE 8= ISRAEL 9= JAPAN 10= MEXICO 11= SAMERICAN 12= USA |
| wlan_security | Wireless security 0= None 1= WEP mode 2= WPA-PSK mode (TKIP) 3= WPA-PSK2 mode (AES) |
| wep_authtype | The WEP authentication type 1= Open system 2= Shared key (WEP mode only) |
| wep_mode | The WEP key mode 1= 64-bit (10HEX) 2= 128-bit (26 HEX) 3= 64-bit (5 ASCII) 4= 128-bit (13 ASCII) |
| wep_index | Active WEP key number 1= WEP key 1 2= WEP key 2 3= WEP key 3 4= WEP key 4 |
| wep_kep1 | WEP key 1 |
| wep_kep2 | WEP key 2 |
| wep_kep3 | WEP key 3 |
| wep_kep4 | WEP key 4 |
| wpa_ascii | The WPA passphrase key (8-63 characters) (Or, WEP passphrase, 1-63 characters) |
| wmm | Enable wireless QoS |
| WpaAuthType | The WPA/WPA2 Enterprise authentication type 1= EAP-TLS 2= EAP-TTLS |
| Wpa_TLSUser | EAP-TLS User name (up to 64 ASCII characters) |
| Wpa_TLSPrivateKeyPasswd | EAP-TLS private key password(up to 64 ASCII characters) |
| Wpa_TTLSAuthType | EAP-TTLS authentication type 1=MSCHAP 2=MSCHAPv2 3=PAP 4=EAP-MD5 5=EAP-GTC |
| Wpa_TTLSUser | EAP-TTLS User name(up to 64 ASCII characters) |
| Wpa_TTLSPasswd | EAP-TTLS User’s password(up to 64 ASCII characters) |
| Wpa_TTLSAnonyName | EAP-TTLS/EAP-TLS Anonymous name(up to 64 ASCII characters) |

## DDNS group

| Parameter | Values and description |
| --- | --- |
| ddns_mode | The DDNS feature 0= Off 1= On |
| ddns_service | The DDNS service name 1= DynDNS.org 2= TZO.com 3= 3322.org |
| ddns_account | The DDNS account name (up to 64 ASCII characters) |
| ddns_password | The DDNS account password (up to 32 ASCII characters) |
| ddns_host_name | The DDNS host name (up to 128 ASCII characters) |
| ddns_hour | The start time (hour) to update IP address to the DDNS server (0-23) |
| ddns_minute | The start time (minute) to update the IP address to the DDNS server (0-59) |
| ddns_update_unit | Scheduling update unit 1= Minute 2= Hour 3= Day |
| ddns_update_period | Scheduling update interval (> 0) |

## HTTP group

| Parameter | Values and description |
| --- | --- |
| http_port2 | Enable the HTTP alternative port 0= Off 1= On |
| http_port2_num | The HTTP second port number (1024-65535) |
| ssport_enable | Enable the HTTPS alternative port 0= Off 1= On |
| ssport_number | The HTTPS second port number |

## RTSP_RTP group

| Parameter | Values and description |
| --- | --- |
| rtsp_port | RTSP port number (554, 1024-65535) |
| rtp_port | RTP port (1024-65535, default 5000) |
| rtp_size | RTP packet size (400-1400) |
| mcast_video_addr | Video multicase address (224.2.0.1) |
| mcast_video_port | Video port number (1024-65534, even values only) |
| mcast_audio_addr | Audio multicast address (224.2.0.1) |
| mcast_audio_port | Audio port number (1024-65534, even values only) |
| mcast_hops | Multicast time to leave value (1-255) |
| mcast_enable | Multicast function 0: disable 1: enable |

## UPNP group

| Parameter | Values and description |
| --- | --- |
| upnp_mode | The uPnP feaure 0= Off 1= On |
| upnp_traversal | The uPnP port mapping (traversal) 0= Off 1= On |
| upnp_camera | The uPnP Security Camera 0= Off 1= On |

## EMAIL group

| Parameter | Values and description |
| --- | --- |
| smtp_enable | Enable the first SMTP server |
| smtp_server | The first SMTP server address (up to 64 characters) |
| pop_server | The first POP server address (up to 64 characters) |
| smtp_port | The SMTP port number of the first server(0-65535) |
| smtp_auth | Authentication method of the first server 0= None 1= SMTP 2= POP before SMTP |
| smtp_account | The first STMP/POP server login name (up to 32 characters) |
| smtp_password | The first SMTP/POP server password (up to 16 characters) |
| smtp2_enable | Enable the second SMTP server |
| smtp2_server | The second SMTP server address (up to 64 characters) |
| pop2_server | POP server address (up to 64 characters) |
| smtp2_port | The SMTP port number of the second server(0-65535) |
| smtp2_auth | Authentication method of the second server 0= None 1= SMTP 2= POP before SMTP |
| smtp2_account | The second STMP/POP server login name (up to 32 characters) |
| smtp2_password | The second SMTP/POP server password (up to 16 characters) |
| from_addr | From E-mail address (up to 48 characters) |
| to_addr1 … to_addr3 | Send-to E-mail address (up to 48 characters) |
| send_email | Send E-mail or not ABC Where A= Send to address 1 0: No 1: Yes B= Send to address 2 0: No 1: Yes C= Send to address 3 0: No 1: Yes |
| subject | E-mail subject (up to 48 ASCII character) |

## FTP group

| Parameter | Values and description |
| --- | --- |
| ftp1 | The FTP Uploading feature. 0= Off 1= On |
| ftp1_server | The FTP server IP address (up to 64 characters) |
| ftp1_port | The FTP control port number |
| ftp1_account | The FTP login name (up to 32 characters) |
| ftp1_passwd | The FTP server login password (up to 16 characters) |
| ftp1_path | The FTP server login default path (up to 64 characters) |
| ftp1_passive | Passive mode 0= Off 1= On |
| ftp2 | The FTP Uploading feature. 0= Off 1= On |
| ftp2_port | The FTP control port number |
| ftp2_server | The FTP server IP address (up to 64 characters) |
| ftp2_account | The FTP login name (up to 32 characters) |
| ftp2_passwd | The FTP server login password (up to 16 characters) |
| ftp2_path | The FTP server login default path (up to 64 characters) |
| ftp2_passive | Passive mode 0= Off 1= On |

## VIDEO group

| Parameter | Values and description |
| --- | --- |
| video_schedule | View video scheduling 0= Off 1= On |
| video_define1 … video_define10 | View video scheduling (Total up to 10 schedules can be defined.) D,HHMM,hhmm Where D= 1: Sunday 2: Monday ... 7: Saturday 8: Weekdays 9: Everyday HHMM= Start time Hhmm= End time |
| time_stamp | Time stamp 0= Off 1= On |
| text_overlay | Text overlay 0= Off 1= On |
| text | Text overlay string (up to 20 ASCII characters) |
| image_overlay | Image overlay 0= Off 1= On |
| image_position | The coordinates of uploaded Image (for overlay) at resolution 640x480 (up-left corner) Format: X,Y Range: X = 0-639, Y = 0-479 |
| image_transparent | Enable transparent feature 0= Off 1= On |
| image_trans_color | transparent color 0x0 – 0xFFFFFF |
| mask_window1 … mask_window4 | Privacy Mask window 0= Off 1= On |
| mask_position1 … mask_position4 | The coordinates of Privacy Mask window n at resolution 640x480 (up-left ~ down-right) Format: X1,Y1,X2,Y2 Range: X1 and X2= 0-639, Y1 and Y2= 0-479 |
| mask_color1 … mask_color4 | The color bitmap of the Privacy Mask window n 000000 ~ FFFFFF (RRGGBB) (Ex. FFFFFF: white, 000000: black, FF0000: red…) |
| power_line | Power line frequency 60= 60Hz 50= 50Hz |
| color | Color balance 0= Auto 1= Indoor 2= White light 3= Yellow light 4= Outdoor 5= Black & white |
| exposure | Exposure/Brightness (1-7) |
| sharpness | sharpness (1-7) |
| flip | Flip (Rotate) 0=Off 1= On |
| mirror | Mirror 0=Off 1= On |
| hue | hue (1-7) |
| saturation | saturation (1-7) |
| contrast | contrast (1-7) |
| dn_filter | Set IR filter 0:Day 1:Night |
| dn_sch | Enable Dignt/Night IR filter |
| dn_sch_hr | start hour(0-23) |
| dn_sch_min | start minute(0-59) |
| dn_hrend | end hour(0-23) |
| dn_minendr | end minute(0-59) |

## MPEG4 group

| Parameter | Values and description |
| --- | --- |
| resolution | Resolutio 1= 160x120 2= 320x240 3= 640x480 |
| quality_type | Video quality control 0= Fix bit rate 1= Fix quality |
| quality_level | Quality 1= Very low 2= Low 3= Normal 4= High 5= Very high |
| bit_rate | Bitrate 0= 64K 1= 128K 2= 256K 3= 384K 4= 512K 5= 768K 6= 1024K 7= 1280K |
| frame_rate | Frame rate= 0 (auto), 1-30 FPS |
| bandwidth | Bandwidth 0= Auto 1= 64K 2= 128 3= 256 4= 512 5= 768 6= 1024 7= 1.5M 8= 2M |

## JPEG group

| Parameter | Values and description |
| --- | --- |
| resolution | Resolutio 1= 160x120 2= 320x240 3= 640x480 |
| quality_level | Quality 1= Very low 2= Low 3= Normal 4= High 5= Very high |
| frame_rate | Frame rate= 0 (auto), 1-30 FPS |
| bandwidth | Bandwidth 0= Auto 1= 64K 2= 128 3= 256 4= 512 5= 768 6= 1024 7= 1.5M 8= 2M |

## Mobile phone group

| Parameter | Values and description |
| --- | --- |
| mobile_support | Enable Mobile Streaming 0= Off 1= On |
| resolution | Resolutio Always 1 (160x120) |
| quality_type | Video quality control 0= Fix bit rate (default) 1= Fix quality |
| quality_level | Quality 1= Very low 2= Low 3= Normal 4= High 5= Very high |
| bit_rate | Bitrate 0= 64K (default) 1= 128K 2= 256K 3= 384K 4= 512K |
| frame_rate | Frame rate 1~15FPS (default = 15) |
| mobile_access | access code for mobile phone(up to 16 characters) |

## AUDIO group

| Parameter | Values and description |
| --- | --- |
| audio_in | Audio-in 0= Off 1= On |
| in_volume | Audio-in volume (1-16, internal use only, not allowed to change its value). |
| in_audio_type | Audio-in codec: 0 = G.711 1 = G.726 2 = G.729ab |
| audio_out | Audio-out 0= Off 1= On |
| out_volume | Audio-out volume (1-16, internal use only, not allowed to change its value). |
| out_audio_type | Audio-out codec: 0 = G.711 1 = G.726 2 = G.729ab |
| audio_mode | Audio feature (for some specific models): 0 = disable 1 = enable |
| operation_mode | Audio mode settings (for some specific models): 0 = simplex - listen 1 = simplex - talk 2 = half duplex 3 = full duplex |

## USER group

| Parameter | Values and description |
| --- | --- |
| login_check | Authentication check 0= No 1= Yes |
| admin_timeout | Management login time-out value in seconds 0= Disable n= n minutes |
| admin_name | Administrator login name (up to 20 characters) |
| admin_password | Administrator login password (up to 8 characters) |
| viewer_name | Viewer login name (up to 20 characters) |
| viewer_password | Viewer login password (up to 8 characters) |
| user1 … user20 | User name and password (Name is up to 20 characters and password is up to 8 characters.) user1= name1,password1 ... user20= name20,passwrod20 |
| pt_ctrl | Pan & tilt control 0= No 1= Yes Ex: 1,0,1,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0 User 1 and user 3 with pan & tilt access rights |
| io_ctrl | IO access rights, 20 digitals 0= No 1= Yes Ex: 1,0,1,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,1,0 User 1, user 3 and user 19 can control I/O ports |
| adm_ctrl | Administrator access control 0= No 1= Yes Ex: 1,0,1,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,1,0 User 1, user 3 and user 19 can control I/O ports |

## IP_filter group

| Parameter | Values and description |
| --- | --- |
| ip_filter | IP address filter 0= Disable 1= Enable |
| ip_filter_rule | Default policy 0= Allowed 1= Deny |
| ip_filter1 … ip_filter20 | Allowed IP address filter <allowed_ip_address> or <start_ip_address>, <end_ip_address> |

## MOTION group

| Parameter | Values and description |
| --- | --- |
| md_mode | Motion detection 0= Off 1= Always on |
| md_switch1 … md_switch4 | Motion detection window 0= Off 1= On |
| md_name1 … md_name4 | Name of the motion detection area 0 (12 ASCII characters) |
| md_window1 … md_window4 | The coordinates of Motion Detection window n at resolution 640x480 Format: X1,Y1,X2,Y2 Range: X1 and X2= 0-639 Y1 and Y2= 0-479 |
| md_threshold1 … md_threshold4 | The threshold of Motion Detection window n (0-255) |
| md_sensitivity1 … md_sensitivity4 | The sensitivity of Motion Detection window n (0-10) |

## SERIAL group

| Parameter | Values and description |
| --- | --- |
| SerialMode | The serial port feature 0= Off 1= RS485 |
| RS485Addr | RS485 receiver address (1-255) |
| BaudRate | Baud rate 0= 1200 1= 2400 2= 4800 3= 9600 4= 19200 5= 38400 6= 57600 7= 115200 8= 230400 9= 460800 10= 921600 |
| DataBit | Data bits 0= 7 bits 1= 8 bits |
| StopBit | Stop bits 0= 1 stop bit 1= 1.5 stop bits 2= 2 stop bits |
| Parity | Parity 0= Odd 1= Even 2= None 3= Mark 4= Space |
| FlowControl | Flow control 0= None 1= HW (RTS/CTS) flow control 2= XON/XOFF flow control |

## PTZ group

| Parameter | Values and description |
| --- | --- |
| PtzMode | Pan and tilt control 0= Off 1= On |
| Preset1Name … Preset9Name | The name of preset position n (up to 16 ASCII characters) |
| Preset1Position … Preset9Position | The coordinates (x,y) of preset position n Format: X,Y Range: X= -63 to 63 (by models) Y= -36 to 28 (by models) |
| Patrol1Position … Patrol9Position | Total up to 20 positions can be grouped together. <preset position#>,<time in seconds>; <preset position#>,<time in seconds>;... |
| PredefineHome | User defined HOME position (x,y) |
| PtzPanSpeed | Pan Speed(1-10) |
| PtzTiltSpeed | Tilt Speed(1-10) |

## IO group

| Parameter | Values and description |
| --- | --- |
| in1_trigger in2_trigger | Input port event trigger 0= Off 1= On |
| in1_type in2_type | Input port triggered type 1= Input high 2= Input low 3= Input rising 4= Input falling |
| out1_init out2_init | Initialization state of the output ports 1= High at power on 2= Low at power on |
| out1_button out2_button | Output port buttons A,B,C Where A= O: High/Low 1: Pulse B= 0: ON button maps to high. 1: ON button maps to low. (Valid when A= 0) C= manual button pulse width, mini-seconds(>0, valid when A=1) |
| out1_action out2_action | Actions on output ports when triggered 1= High 2= Low 3= Pulse |
| out1_pulse_duration out2_pulse_duration | Pulse width when event triggered , in mini-seconds |

## EVENT group

| Parameter | Values and description |
| --- | --- |
| event_trigger | Event trigger 0= Off 1= On |
| event_schedule | Event schedule 0= Off (always on) 1= On |
| event_define1 … event_define10 | Event trigger scheduling (Total up to 10 schedules can be defined.) DHHMMhhmm Where DD= 1: Monday 2: Tuesday ... 7: Sunday 8: Weekdays 9: Everyday HHMM= Start time (4 digits) Hhmm= End time (4 digits) |
| event_interval | Interval in minutes before detecting the next event in seconds |
| event_in1 | Actions triggered by input port 1, 4 digits A,B,C,D,E Where A= Send E-mail 0: No 1: Yes B= FTP upload 0: No 1: Yes C= Output port 1 0: No 1: Yes D= Output port 2 0: No 1: Yes E= Instant message 0: No 1:Yes |
| event_in2 | Actions triggered by input port 2 A,B,C,D,E Where A= Send E-mail 0: No 1: Yes B= FTP upload 0: No 1: Yes C= Output port 1 0: No 1: Yes D= Output port 2 0: No 1: Yes E= Instant message 0: No 1:Yes |
| event_mt | Actions triggered by motion detection A,B,C,D,E Where A= Send E-mail 0: No 1: Yes B= FTP upload 0: No 1: Yes C= Output port 1 0: No 1: Yes D= Output port 2 0: No 1: Yes E= Instant message 0: No 1:Yes |
| event_attach | Attached image type 0= MPEG-4 1= JPEG |
| event_mpeg4 | MPEG-4 format A,B,C Where A= File type 0: asf 1: mp4 2: 3gp B= Pre-captured Video length in seconds (0-5 seconds) C= Post-captured Video length in seconds (1-5 seconds) |
| event_jpeg | JPEG format A,B,C Where A= JPEG Frame per second(1-10) B= Pre-captured jpeg length in seconds (0-5 seconds) C= Post-captured jpeg length in seconds (1-5 seconds) |
| event_overwrite | Overwrite oldest video log file when SDRAM is full 0= No 1= Yes |

## Bonjour group

| Parameter | Values and description |
| --- | --- |
| bonjour_mode | The Bonjour feaure 0= Off 1= On |

## QoS group

| Parameter | Values and description |
| --- | --- |
| qos_enable | Enable QoS feature 0:disable 1:enable |
| qos_av_switch | QoS on Audio or Video 0:audio 1: video |
| cos_enable | Enable CoS feature 0:disable 1:enable |
| cos_priority | CoS priority 0-7 |
| cos_vlan_id | CoS vlan ID 0-4095 |

# Appendices

## Appendix A - Time Zone List

The daylight saving time zone areas are marked with ‘*’ character.

(GMT-12:00) International Date Line West (GMT-11:00) Midway (GMT-10:00) Hawaii *(GMT-09:00) Alaska *(GMT-08:00) Pacific Time (US & Canada), Tijuana (GMT-07:00) Arizona *(GMT-07:00) Chihuahua, La Paz, Mazatlan *(GMT-07:00) Mountain Time (US & Canada) (GMT-06:00) Central America *(GMT-06:00) Central Time (US & Canada) *(GMT-06:00) Guadalajara, Mexico City,Monterrey (GMT-06:00) Saskatchewan (GMT-05:00) Bogota, Lima, Quito *(GMT-05:00) Eastern Time (US & Canada) (GMT-05:00) Indiana (East) *(GMT-04:00) Atlantic Time (Canada) (GMT-04:00) Caracas, La Paz *(GMT-04:00) Santiago *(GMT-03:30) Newfoundland *(GMT-03:00) Brasilia (GMT-03:00) Buenos Aires, Georgetown *(GMT-03:00) Greenland *(GMT-02:00) Mid-Atlantic *(GMT-01:00) Azores (GMT-01:00) Cape Verde Is. (GMT) Casablanca, Monrovia *(GMT) Greenwich Mean Time: Dublin, Edinburgh, Lisbon,London *(GMT+01:00) Amsterdam, Berlin, Bern, Rome, Stockholm, Vienna *(GMT+01:00) Belgrade, Bratislava, Budapest, Ljubljana, Prague *(GMT+01:00) Brussels, Copenhagen, Madrid, Paris *(GMT+01:00) Sarajevo, Skopje, Warsaw, Zagreb (GMT+01:00) West Central Africa *(GMT+02:00) Athens, Istanbul, Minsk *(GMT+02:00) Bucharest *(GMT+02:00) Cairo (GMT+02:00) Harare, Pretoria *(GMT+02:00) Helsinki, Kyiv, Riga, Sofia, Tallinn, Vilnius (GMT+02:00) Jerusalem *(GMT+03:00) Baghdad (GMT+03:00) Kuwait, Riyadh *(GMT+03:00) Moscow, St. Petersburg, Volgograd (GMT+03:00) Nairobi *(GMT+03:30) Tehran (GMT+04:00) Abu Dhabi, Muscat *(GMT+04:00) Baku, Tbilisi, Yerevan (GMT+04:30) Kabul *(GMT+05:00) Ekaterinburg (GMT+05:00) Islamabad, Karachi, Tashkent (GMT+05:30) Chennai, Kolkata, Mumbai, New Delhi (GMT+05:45) Kathmandu *(GMT+06:00) Almaty, Novosibirsk (GMT+06:00) Astana, Dhaka (GMT+06:00) Sri Jayawardenepura (GMT+06:30) Rangoon (GMT+07:00) Bangkok, Hanoi, Jakarta *(GMT+07:00) Krasnoyarsk (GMT+08:00) Beijing, Chongqing, Hong Kong, Urumqi *(GMT+08:00) Irkutsk, Ulaan Bataar (GMT+08:00) Kuala Lumpur, Singapore (GMT+08:00) Perth (GMT+08:00) Taipei (GMT+09:00) Osaka, Sapporo, Tokyo (GMT+09:00) Seoul *(GMT+09:00) Yakutsk *(GMT+09:30) Adelaide (GMT+09:30) Darwin (GMT+10:00) Brisbane *(GMT+10:00) Canberra, Melbourne, Sydney (GMT+10:00) Guam, Port Moresby *(GMT+10:00) Hobart *(GMT+10:00) Vladivostok (GMT+11:00) Magadan, Solomon Is., New Caledonia *(GMT+12:00) Auckland, Wellington (GMT+12:00) Fiji, Kamchatka, Marshall Is. (GMT+13:00) Nuku'alofa