PMON Reference Manual (Getting Started with PMON – Version 6.50)

- Introduction

The purpose of this manual is to introduce the BRECIS boot loader for operating systems such as Linux and proprietary operating systems.

BRECIS PMON is derived from the original PMON.  The original sources are at http://www.carmel.com/pmon/. Please refer to this web site for more information.

- Development Environment

The following components are required for software development:

- Host system (Linux Operating System on a PC, RedHat 7.2/7.3)

- MIPS cross-compiler  (Available from BRECIS)

- RS232 Console/Terminal

- Target EVM board

- Build PMON under Linux host

The following steps build/rebuild PMON:

Step 1: Copy PMON from CD to hard drive with directory named “pmon”.

Step 2: Make the pmon directory writeable by issue the following command:

chmod -R 777 pmon

Step 3: Clean up the object code: (clean is a script file)

./clean

Step 4: Build the object code: (bld is a script file)

./bld

- Flash memory organization

The following is the flash memory organization defined by BRECIS. Users can change it according to their needs, however, it is your responsibility to make sure no conflict exist between the blocks.

| address | description |
| --- | --- |
| bfc00000 | pmon code in flash |
| bfc70000 | pmon startup script |
| bfc80000 | Applications (e.g. uClinux) |

- SDRAM memory organization

The following is the SDRAM memory organization defined by PMON.

| address | description |
| --- | --- |
| 80000000 | Exception vectors in SDRAM |
| 80000200 | pmon data area |
| 80040000 | Default start of Applications SDRAM area |
| End-pmon size - 4000 | pmon relocated code |
| End - 4000 | Unused by pmon |

  - Start the RTOS at or above 0x80040000.

  - Do NOT use the last 0x30000 of SDRAM, where PMON resides in SDRAM,

  - PMON reserves that last 0x4000 of SDRAM for RTOS private storage.  PMON will not touch this memory.

- Set up RS232 Console

Set the baud rate to 57600 bps.  Under Linux, one can use Linux terminal application minicom.

- PMON command listing

The following lists the commands used for PMON.

Detail descriptions are listed in Appendix A.

| Command Category | Command | Status |
| --- | --- | --- |
| Downloading | Load a File (load) Define a symbol (sym) List a symbol (ls) | Supported Supported Supported |
| Display/Set Registers/Memory | Display/Set Register (r) Modify Memory (m) Display Memory (d) Disassemble Memory (l) Fill Memory (fill) Copy Memory (copy) Compare Memory (compare) Search Memory (search) Dump Memory (dump) | Supported Supported Supported Supported Partial (no sanity check) Supported Supported Partial (no sanity check) Supported |
| Execution Control | Start Execution (g) Display/Set Breakpoints (b) Set Complex Breakpoint (when) Delete Breakpoint (db) Single Step (t/to) Continue Execution (c) Execute Subroutine (call) | Supported Supported |
| Miscellaneous | Help (h) History (hi) Execute a script (exec) Display/Set environment variable (set) Set Terminal Parameters (stty) Erase flash (erase) Flush the Caches (flush) Display the version number (vers) The Command Shell Paginate (more command) | Supported Supported Supported Supported Supported Supported ?? (need to verify data) Supported Supported Supported |

- Load application

The application (or RTOS) can be loaded through Ethernet into SDRAM. The HOST is assumed to be running Linux (Redhat 7.2/7.3) and PMON has been burned into FLASH before the target is powered up.

The following is an example to set up and download uClinux.

  - Set the Target’s MAC and IP address for MAC0

Under the PMON console, type the following commands:

set etheraddr 00:02:d8:00:00:17

set ipaddress 192.168.1.1

The variable hostport controls what ethernet MAC will be used.

To use MAC0, set hostport to ethernet -or- ethernet0

set hostport ethernet

-or-

set hostport ethernet0

To use MAC1,

set hostport ethernet1

If your EVM board has MAC2, you can use MAC2

set hostport ethernet2

  - Download script file to SDRAM and save it to flash.

Step 1: Create a file similar to the following:

(The following example script is from the uClinix Release in uClinux/documentation/pmon-script.  There is an example script in pmon/examples/pmon-script.)

#!/bin/pmon

# set some initial boot parameters

set ipaddr 192.168.15.2

set etheraddr 00:02:d8:00:00:50

set heaptop 80100000

#	Load binary image at 80100000

load -B

#

# set parameters for kernel

set modetty0 "57600,n,8,1,hw"

set bootserport tty0

set cpuconfig ""

set ethaddr0 00:02:d8:00:00:50

set ethaddr1 00:02:d8:00:00:51

# set ethaddr2 00:02:d8:00:00:52

set baseboardserial 0000000086

set bootprot tftp

set bootserver 0.0.0.0

set bootfile ""

#

# The following is how two interfaces might be configured.

# The following starts the kernel invoking kgdb using either scc0 or console.

# g 80100000 -c kgdb=scc0 ip=192.168.15.2::192.168.15.1::Brecis:eth0:none:100f ip=192.168.16.2:::::eth1:none:an

# g 80100000 -c kgdb=console ip=192.168.15.2::192.168.15.1::Brecis:eth0:none:100f ip=192.168.16.2:::::eth1:none

#

# The following is one way to start the kernel with CONFIG_IP_PNP=y (parsing "ip=" arguments).

g 80100000 -c ip=192.168.15.2::192.168.15.1::Brecis:eth0:none ip=192.168.16.2:::::eth1:none

#

# Set three ethernet addresses via:

# g 80100000 -c ip=192.168.15.2::192.168.15.1::Brecis:eth0:none ip=192.168.16.2:::::eth1:none ip=192.168.17.2:::::eth2:none

#

# Format of the above "g" commands are as follows:

# "g <kernel_entry_address> -c <arguments to the uClinux kernel>

#

# The format of the "ip=argument" is as follows:

#

#  <client-ip>:<server-ip>:<gw-ip>:<netmask>:<host name>:<device>:<PROTO>:<PHY CONFIG>

#  Any of the fields can be empty which means to use a default value:

#      <client-ip>     - address given by BOOTP or RARP

#      <server-ip>     - address of host returning BOOTP or RARP packet

#      <gw-ip>         - none, or the address returned by BOOTP

#      <netmask>       - automatically determined from <client-ip>, or BOOTP

#      <host name>     - <client-ip> in ASCII notation, or returned by BOOTP

#      <device>        - device

#      <PROTO>:

#         off|none         - don't do autoconfig at all (DEFAULT)

#         on|any           - use any configured protocol

#         dhcp|bootp|rarp  - use only the specified protocol

#         both             - use both BOOTP and RARP (not DHCP)

#      <PHY CONFIG>:

#	  an               - auto-negotiate (DEFAULT)

#         10h              - set to 10 Mbit, Half Duplex

#         10f              - set to 10 Mbit, Full Duplex

#         100h             - set to 100 Mbit, Half Duplex

#         100f             - set to 100 Mbit, Full Duplex

#!eof

Lines beginning with “#” are comments.

Step 2: Set up PMON to load the script by typing the command

load -B

Step 3: ping the target to make sure the Ethernet connection is up.

Step 4: Invoke tftp and download the pmon-script.

Step 5: The console shows “loaded data from …” when downloading is complete

Step 6: Save script from SDRAM into Flash.

The following screen shot shows a PMON session doing the initial loading of the pmon-script.

Note: The address bfc70000 is the fixed location for the startup script. The startup script will be executed automatically during next power-up.

The following screen shot shows a ping of the EVM board followed by the tftp of the pmon script to PMON.

  - Load RTOS from Ethernet

This step assumes the RTOS we wish to load is a uClinux kernel.

Step 7: Prepare a Linux script for loading the uClinux image similar to the uClinuxload script file found in the uClinux documentation folder:

#/bin/bash

EVMIPADDR=192.168.15.2

echo "Under pmon:"

echo "				load -B"

echo "				g <options...>"

echo " IP <$EVMIPADDR> put </tftpboot/${USER}/image.bin> 80100000"

tftp $EVMIPADDR <<EOF

bin

rex 1

put /tftpboot/${USER}/image.bin 80100000

EOF

Note: Assume the file name of the uClinux image is image.bin which is found in the /tftpboot/${USER} directory.   It will be loaded into the SDRAM starting from 80100000 because this is where the image.bin is built to execute.

Step 8: Power off and power on the EVM board. The following screen shows the EVM board powered up and waiting for the uClinux image to be downloaded.

Step 9: Execute the uClinuxload script file from the host:

Note: The file size of uClinux image is 4123232 bytes decimal and is equal to hex 3EEA60.

Step 10: The following screen shows up downloading complete and the downloaded uClinux kernel executing (instead of PMON).

  - Break the application download process and return back to PMON

After the script file is burned into flash, PMON will execute the script and may try to load the application (most likely). If you want to get into PMON instead of downloading an application, use control-C to terminate the loading process.

  - Save uClinux in Flash

Generally, the uClinux image.bin is too large to fit in flash.

The uClinux build simultaneously creates a file, image.flash, which is a self extracting gzip file.  One loads this file to flash at the address 0xBFC20000.  One sets up a bbload script and uses bbload to load image.flash.

A screen capture of the command performed under PMON:

A screen capture of the commands performed on the Linux host:

Following is the bbload-script for our example:

(An example bbload-script can be found in file bbload/example-script -or-

in the file, uClinux/documentation/bbload-script).

#!/bin/bbload

# set parameters for kernel

set modetty0 "57600,n,8,1,hw"

set bootserport tty0

set cpuconfig ""

set ethaddr0 00:02:d8:00:00:50

set ethaddr1 00:02:d8:00:00:51

# set ethaddr2 00:02:d8:00:00:52

set baseboardserial 0000000086

set bootprot tftp

set bootserver 0.0.0.0

set bootfile ""

#

# The following is how two interfaces might be configured.

# The following starts the kernel invoking kgdb using either scc0 or console.

# setargs progname kgdb=scc0 ip=192.168.15.2::192.168.15.1::Brecis:eth0:none:100f ip=192.168.16.2:::::eth1:none:an

# setargs progname kgdb=console ip=192.168.15.2::192.168.15.1::Brecis:eth0:none:100f ip=192.168.16.2:::::eth1:none

#

# The following is one way to start the kernel with CONFIG_IP_PNP=y (parsing "ip=" arguments).

setargs progname ip=192.168.15.2::192.168.15.1::Brecis:eth0:none ip=192.168.16.2:::::eth1:none

#

# Set three ethernet addresses via:

# setargs progname ip=192.168.15.2::192.168.15.1::Brecis:eth0:none ip=192.168.16.2:::::eth1:none ip=192.168.17.2:::::eth2:none

#

# Format of the "setargs" bbload command is as follows:

# setargs <program name-argv[0]> <arguments to the uClinux kernel>

#

# The format of the "ip=argument" is as follows:

#

#  <client-ip>:<server-ip>:<gw-ip>:<netmask>:<host name>:<device>:<PROTO>:<PHY CONFIG>

#  Any of the fields can be empty which means to use a default value:

#      <client-ip>     - address given by BOOTP or RARP

#      <server-ip>     - address of host returning BOOTP or RARP packet

#      <gw-ip>         - none, or the address returned by BOOTP

#      <netmask>       - automatically determined from <client-ip>, or BOOTP

#      <host name>     - <client-ip> in ASCII notation, or returned by BOOTP

#      <device>        - device

#      <PROTO>:

#         off|none         - dont do autoconfig at all (DEFAULT)

#         on|any           - use any configured protocol

#         dhcp|bootp|rarp  - use only the specified protocol

#         both             - use both BOOTP and RARP (not DHCP)

#      <PHY CONFIG>:

#	  an               - auto-negotiate (DEFAULT)

#         10h              - set to 10 Mbit, Half Duplex

#         10f              - set to 10 Mbit, Full Duplex

#         100h             - set to 100 Mbit, Half Duplex

#         100f             - set to 100 Mbit, Full Duplex

# Use setpc for address to start execution if flash-based initialization

# Note: no attempt is made to copy a kernel from flash to SDRAM in this case.

setpc bfc20000

#!eof

A screen capture after switching chip selects showing bbload booting from flash:

Appendix A. Detailed Descriptions of the PMON Commands

# B.1. Downloading

# load

The load command downloads programs and data from the host.

## Format

The format for the load command is:

load [-abeistvBS] [-baud] [offset] [-c cmdstr]

where:

| -m | Only load the symbol information. |
| --- | --- |
| -a | suppresses addition of an offset to symbols. |
| -b | suppresses deletion of all breakpoints before the download. |
| -e | suppresses clearing of the exception handlers. |
| -i | ignores checksum errors. |
| -s | suppresses clearing of the symbol table before the download. |
| -t | loads at the top of memory. |
| -v | verbose mode |
| -B | Interprets input from host as binary data for Ethernet transfers. |
| -S | Don't load symbols. |
| -baud | sets the baud rate for transfer. |
| offset | loads at the specified offset. |
| -c cmdstr | sets a command string that pmon sends to the host to start a download operation. String cmdstr is the string that starts the download. Note that the command string must be enclosed in double quotation marks if the string contains any spaces. |

Invoking the load command with no parameters or arguments clears the symbol table, deletes all current breakpoints, allows PMON to receive programs or data from the host.

## Functional Description

The load command accepts programs and data from the host port in LSI Logic's proprietary FastLoad format, Motorola S-record, or binary files. The user can set environment variables to change the data port, the format, and the transfer protocol.   Programs and data are always transferred as binary files over ethernet.

The load command normally clears the symbol table, exception handlers, and all breakpoints. The -s and -b options suppress the clearing of the symbol table and breakpoints, respectively. The value of the EPC register is set automatically to the entry point of the program UNLESS the program is loaded as a binary file.

Please see the ld command if the program being loaded as a binary file is in the ELF image format or the srec format.

The -c option permits a command string to be sent to the host when the load command is issued and the hostport is tty0. This is intended for use in conjunction with the transparent mode. Note that if the command string contains multiple words, the command must be enclosed in double quotation marks, as shown in the example below.

The load command returns the error message "out of memory" if there is insufficient space in the heap for the program's global symbols when PMON is reading an srec format or ELF image format file.  PMON will still attempt to finish loading the image, but the symbol table will be truncated.  To increase the size of the heap , use the set heaptop command to reserve more space in the heap.

## The dlecho, dlproto, heaptop, and hostport Variables

The dlecho, dlproto, and hostport variables control operation of the download.

The dlecho and dlproto variables only have meaning when downloading through the tty0 hostport.  The following table shows how these environment variables affect the operation of the load command.

| Variable | Action |
| --- | --- |
| dlecho off | Do not echo the lines |
| dlecho on | Echo the lines |
| dlecho lfeed | Echo only a linefeed for each line |
| dlproto none | Do not use a protocol |
| dlproto XonXoff | Send Xon and Xoff to control the host |
| dlproto EtxAck | Expect Etx as end of record, send Ack |
| heaptop | The heaptop variable determines the default load address when ethernet is used for downloading. Any data must be loaded at or above this address or pmon data will be destroyed. |
| hostport tty0 | Select tty0 as the port to which the host is connected |
| hostport ethernet | Select ethernet (MAC0) as the port to which the host is connected |
| hostport ethernet0 | Select ethernet (MAC0) as the port to which the host is connected |
| hostport ethernet 1 | Select ethernet (MAC1) as the port to which the host is connected |
| hostport ethernet2 | Select ethernet (MAC2) as the port to which the host is connected |

See the section on downloading beginning on page 1-9 for more information on these variables and the use of the load command.

## Examples

Download to tty0 using a terminal emulator.

PMON> set dlecho off

PMON> set hostport tty0

PMON> set dlproto none

PMON> load

-- use terminal emulator's "send text file" command

Downloading from tty0, ^C to abort

Entry address is 80040000

Total = 0x00043C00 bytes

PMON>

Download to ethernet0 using tftp:

PMON> # set some initial boot parameters

PMON> set ipaddr 10.26.3.55

PMON> set etheraddr 00:02:d8:00:00:52

PMON> set hostport ethernet0

PMON> set heaptop 80100000

PMON> load -B

Downloading from ethernet0, ^C to abort

Loaded data from 80100000 to 8012d4c7, length 2d4c8 (185544 decimal)

PMON>

On the host computer use tftp as follows:

$ tftp 10.26.3.55

tftp> bin

tftp> rex 1

tftp> put pmon.bin

Sent 185544 bytes in 0.6 seconds

tftp> quit

$

## Files

The code for the load command is located in pmon/load.c.

## See Also

set command for the setup of the environment variables.

ld command.

# sym

The sym command sets a symbolic name for a variable.

## Format

The format for this command is:

sym name value

where:

| name | is the name of the variable for which a value is to be set. |
| --- | --- |
| value | is the value to which the variable is set. |

## Functional Description

The sym command sets a symbolic name to the specified value. Normally the load command clears the symbol table. However, there is an option to override the clearing of the symbol table (see the load command for details).

Symbols can be displayed using the ls command.

Examples illustrating the use of this command follow.

PMON> sym start bfc00578

PMON> l bfc00578

start        3c08bfc0 lui     t0,0xbfc0              # 49088

start+0x4    25080594 addiu   t0,t0,0x594            # 1428

start+0x8    3c01a000 lui     at,0xa000              # 40960

start+0xc    01014025 or      t0,t0,at

start+0x10   01000008 jr      t0

start+0x14   00000000 nop

start+0x18   00000000 nop

start+0x1c   40809000 mtc0    zero,C0_BPC

start+0x20   00000000 nop

start+0x24   40809800 mtc0    zero,C0_BDA

PMON> l start 4

start        3c08bfc0 lui     t0,0xbfc0              # 49088

start+0x4    25080594 addiu   t0,t0,0x594            # 1428

start+0x8    3c01a000 lui     at,0xa000              # 40960

start+0xc    01014025 or      t0,t0,at

PMON> sym flush_cache bfc00b70

PMON> l flush_cache 5

flush_cache        3c098001 lui     t1,0x8001              # 32769

flush_cache+0x4    2529cd58 addiu   t1,t1,0xcd58           # -12968

flush_cache+0x8    3c01a000 lui     at,0xa000              # 40960

flush_cache+0xc    01214825 or      t1,t1,at

flush_cache+0x10   8d280000 lw      t0,0(t1)

PMON>

## See Also

ls, load, l, ld, and sh commands.

# ls

The ls command lists the current symbols in the symbol table.

## Format

The format for the ls command is:

ls [-ln] [sym|[-[v|a] adr]

where:

| -l | provides a long listing, showing the address value for each symbol. |
| --- | --- |
| -n | lists the symbols in ascending order of address. |
| sym | is a pattern filter for the symbols to be shown. Both character wildcards ("?") and word wildcards ("*") are permitted. |
| -v | is the verbose option, showing the value in hexadecimal, decimal, and octal. |
| -a | shows the address in symbolic form. |
| adr | is the address for which a symbol or offset from a symbol is sought. |

Invoking the ls command without any options or parameters lists the symbols in alphabetical order without displaying the actual address for each symbol.

## Functional Description

The ls command lists the symbols in the symbol table.

The -l option produces a long listing, which includes the address value of each symbol. The -n option causes the symbols to be listed in ascending order of address. The -a adr option lists the symbol at the next lowest address. The -v adr option prints the result in hex, decimal, and octal. The -v option is useful for computing the value of an expression that may include registers, symbols, and absolute values.

Examples illustrating the use of the ls command follow.

| PMON> ls flush_cache start | List symbols in alphabetic order. |
| --- | --- |
| PMON> ls -l bfc00b70 flush_cache bfc00578 start | List symbols in alphabetic order with addresses. |
| PMON> ls -ln bfc00578 start bfc00b70 flush_cache | List symbols and addresses in ascending order of address. |
| PMON> ls s* start | List symbols starting with the letter "s." |
| PMON> ls -a bfc00598 bfc00598 start+0x20 | List symbol at the next lowest address. |
| PMON> ls -a @epc bfc00598 = start+0x20 | List symbol at the next lowest address from EPC. |
| PMON> ls -v @t0+0t10*4 0x800222e8 = 0t-2147343640 = 0o20000421350 | Display the value of the expression "@t0+0t10*4": |

# B.2. Display/Set Registers/Memory

# r

The r command sets or displays register values.

## Format

The format for the r command is:

r [reg|* [val|field val]]

where:

| reg | is the name of the register or registers (specified by wildcard characters) to display or modify. |
| --- | --- |
| val | is the value to which the specified register or registers should be modified. |
| field val | is the value to which the specified field in the specified register should be modified. |
| * | displays the contents of all registers except floating-point registers. |

Invoking the r command without any parameters or arguments displays a list of all the general-purpose registers.

## Functional Description

The r command sets or displays register values. The character and word wildcards, "*" and "?", can be used in the register name. The '?' character matches any single character, while the '*' character matches any number of any characters. This command accepts both hardware and software names. Examples illustrating the use of the r command follow.

| r | Display all General-purpose registers. |
| --- | --- |
| r * | Display all register values. |
| r 8 | Display $8 (t0). |
| r t0 | Display t0 ($8). |
| r t* | Display t0 through t9. |
| r pc | Display PC register. |
| r pc start | Set PC register to the symbol start value. |
| r 4 45 | Set register 4 to 45. |
| r t0 45 | Set register t0 to 45. |
| r sr 0 | Set SR to zero. |
| r sr bev 1 | Set the BEV bit of SR to one. |
| r pc a0040000 | Set PC to a0040000. |
| r a | Display the value of pseudo register 'a'. |
| r ? | Display the value of all pseudo registers. |

There are 26 pseudo registers (named a thru z). These can be referenced in the same way as real registers, and are typically used to implement complex breakpoint conditions (see the when command).

## Examples

Display all General Purpose registers.

PMON> r

zero      at       v0       v1       a0       a1       a2       a3

$0- 00000000 00000000 00000000 00000000 00000000 00000000 00000000 00000000

t0       t1       t2       t3       t4       t5       t6       t7

$8- 00000000 00000000 00000000 00000000 00000000 00000000 00000000 00000000

s0       s1       s2       s3       s4       s5       s6       s7

$16- 00000000 00000000 00000000 00000000 00000000 00000000 00000000 00000000

t8       t9       k0       k1       gp       sp       s8      ra

$24- 00000000 00000000 00000000 00000000 00000000 80008b40 00000000 00000000

PMON> r *

$0- 00000000 00000000 00000000 00000000 00000001 80000208 00000000

00000000

$8- 00000000 00000000 00000000 00000000 00000000 00000000 00000000

00000000

$16- 00000000 00000000 00000000 00000000 00000000 00000000 00000000

00000000

$24- 00000000 00000000 00000000 00000000 00000000 83fdb7e8 00000000

00000000

PC=80040000          HI=00000000          LO=00000000

C0_SR:  CU  BEV TS PE CM PZ SWC ISC   IM&SW  KUo IEo KUp IEp KUc IEc

0000  0   0  0  0  0  0   0  11111111  0   0   0   0   1   0

C0_CAUSE: BD CE   IP   SW EXCODE

0  0 000000 00   Int

C0_CONFIG: M K23  KU   -  MDU - MM BM BE AT  AR  MT   -   K0

1 010 011 0000  0  0 00  0  1 00 000 011 0000 011

C0_PRID: IMP Rev

131  5

C0_EPC=80040000    C0_BADVA=dcae2dae

C0_CCC: EVI CMP IIE DIE MUL MAD TMR BGE IE0 IE1 IS DE0 DE1 DS IPWE

IPWS

1   0   0   0   0   0   0   0   0   0   2  0   0   0   0    3

C0_CCC: TE WB SR0 SR1 ISC TAG INV

0  0  0   0   0   1   1

C0_DCS: TR UD KD TE DW DR DAE PCE DE T W R DA PC DB

0  0  0  0  0  0  0   0   0 0 0 0  0  0  0

M_SCBUS_ST: BEDE BERR

0    0

M_SCBUS_EA=3c1abfc0

M_EXVI: HEVI SEVI

0    0

C0_INDEX=00000000   C0_RANDOM=00000000 C0_ENTRYLO=00000000

C0_CONTEXT=3f6e5710 C0_PAGEMASK=00000000 C0_WIRED=00000000

C0_COUNT=293e6f84   C0_ENTRYHI=00000000 C0_COMPARE=ffffffff

C0_LLADR=0a87bd1c   C0_BPC=00000000      C0_BDA=00000000

C0_BPCM=00000000    C0_BDAM=00000000     C0_ROTATE=0a009c31

C0_CMASK=fd683a11   C0_ERREPC=bfc2070c

Set the IEC bit of the Status Register.

PMON> r sr iec 1

Display the Status Register.

PMON> r sr

C0_SR: CU BEV TS PE CM PZ SWC ISC IM&SW  KUo IEo KUp IEp KUc IEc

0000 1   0  0  0  0  0   0 00000000 0   0   0   0   0   1

You can use the ls command to display the Status Register as a hex value. This technique can be used for any register.

PMON> ls -v @sr

0x00400001 = 0t4194305 = 0o20000001

## Files

The r command is located in mon/regs.c.

## See Also

l command for disassembling instructions from memory.

# m

The m command displays and modifies memory.

## Format

The format for the m command is:

m [-bhw] [adr [hexval|-s str]..]

where:

| -b | is a flag signifying that byte accesses are to be used. |
| --- | --- |
| -h | is a flag signifying that halfword accesses are to be used. |
| -w | is a flag signifying that word accesses are to be used. |
| adr | is the memory address to display or modify without entering interactive mode. |
| hexval | is the value to insert at the specified address. |
| -s | is a flag signifying that the following parameter is a string value. |
| str | is a string value to copy to the specified address. |
|  | enters interactive mode. |
| = | in interactive mode, reads current address again. |
| ^¦- | in interactive mode, moves back one word. |
| . | exits interactive mode. |

Entering no values with this command causes the command to operate in interactive mode.

## Functional Description

This command can display and then modify memory locations interactively. This command can also set memory to a specified value directly.

If invoked with one or more values following the address, the command is executed immediately, without entering the interactive mode.

If the command is invoked without a value, the command enters the interactive memory mode. In interactive memory mode, the user enters a command at the cursor. The interactive memory mode first displays the address and its current value. Interactive memory mode then lets the user select one of the commands listed in the following table.

If the -s option is specified, pmon displays the memory contents as an ASCII string. A multiple-word string may be specified by enclosing the multiple-word string in quotation marks.

Examples illustrating the use of the m command follow.

PMON> m a0040000	Display memory at address in interactive mode.

a0040000 00 _ 	User can enter command at cursor (_).

PMON> m a0040000 1 2 3 4	Set address 0xa0040000 to 1, address

0xa0020001 to 2, etc., in noninteractive mode.

PMON> m a0020000	Display memory at 0xa0040000.

a0040000 01 CR

a0040001 02 CR

a0040002 03 CR

a0040003 04 .

PMON> m a0040000 22	Set address 0xa0040000 to 0x22.

PMON> m a0040000 	Display memory at 0xa0040000.

a0040000 22 44

a0040001 00

a0040002 00 55

a0040003 00 66

a0040004 00 ^

a0040003 66

a0040004 00 .

PMON> m 80040000 -s even	Set memory starting at 0x80040000 to

the string "even."

PMON> m 80050100 -s "BRECIS Communications"

Set memory starting at 0x80050100 to

the multiple-word string "BRECIS Communications"

## Files

The m command is located in mon/modify.c.

## See Also

fill command, l command, d command, and dump command.

# d

The d command displays memory contents in hex or ASCII format.

## Format

The format for the d command is:

d [-b|h|w|s|S] adr [cnt|-rreg]

where:

| -b | displays the memory contents in groups of bytes. |
| --- | --- |
| -h | displays the memory contents in halfword groups. |
| -w | displays the memory contents in word groups. |
| -s | displays the memory contents as a null terminated string. |
| -S | cnt is the number of data items to be displayed. |
| adr | specifies the base address from which data is displayed. |
| cnt | specifies the number of lines displayed. |
| -rreg | displays the contents of memory as register reg. |

## Functional Description

The d command displays memory, starting at the specified address, in hexadecimal or ASCII format. A -b, -h, -w, or -s option, if specified, sets how the data is displayed. See the examples at the end of this section for illustration of the possible display formats.

The datasz and moresz Variables

If invoked without a -b, -h, -w, or -s option, the datasz variable sets the display format. Setting datasz to -b, -h, -w, or -s has the same effect as the command line options of the same names described in this section. The datasz variable does not effect any other command displays.

If invoked without cnt, the d command first displays the number of lines specified in the environment variable moresz. Next, pmon pauses and displays the more prompt. See the more command for commands available with the more prompt. Also see the more command for more information on the moresz variable.

The following example displays memory starting at 0xa0010000.

PMON> d a0010000

a0010000 bf c0 2b 00 bf c0 2b 00 bf c0 2b 00 bf c0 2b 3c ..+...+...+...+<

a0010010 bf c0 2b 3c bf c0 2b 3c bf c0 2b 20 bf c0 2b 20 ..+<..+<..+...+.

a0010020 bf c0 2b 20 bf c0 2b a8 bf c0 2b 78 bf c0 2b 60 ..+...+...+x..+`

a0010030 bf c0 2b 48 bf c0 2b a8 bf c0 2b a8 bf c0 2b a8 ..+H..+...+...+.

a0010040 bf c0 2b 78 bf c0 2b 60 bf c0 2b 48 bf c0 2e 78 ..+x..+`..+H...x

a0010050 bf c0 2f 08 bf c0 2e c4 bf c0 2e 80 bf c0 2f 90 ../.........../.

a0010060 bf c0 2f 90 bf c0 2f 90 bf c0 2e 78 bf c0 2e 78 ../.../....x...x

a0010070 bf c0 2e 78 00 00 00 00 00 00 00 00 00 00 00 00 ...x............

## Files

The d command is located in mon/dump.c.

## See Also

l command, search command, m command and dump command.

# l

The l command disassembles instructions from memory.

## Format

The format for the l command is:

l [-bctT] [adr [cnt]]

where:

| -b | lists only branches. |
| --- | --- |
| -c | lists only calls. |
| -t | lists the trace buffer. |
| -T | assume TinyRISC (MIPS16) instructions. |
| adr | is the base address from which to disassemble instructions. |
| cnt | is the number of lines to disassemble. |

When invoking this command with no options, disassembly starts at the address in the EPC register and is output to the more command.

## Functional Description

The l command disassembles the memory contents, starting either at the EPC register's current value or at the specified address. The output of this command is passed to the more command, letting the user view one screenful of disassembled output at a time. Optionally, the user can specify a count value, which limits the number of disassembled lines to that number.

The regstyle Variable

The regstyle environment variable determines whether  pmon displays hardware or software register names. Hardware register names are simply $0 through $31. Software registers are defined by the MIPS software conventions. Set regstyle to "hw" for hardware register names. Set regstyle to "sw" for software register names.

Examples illustrating the use of the l command follow.

PMON> set regstyle sw 			Normally the default.

PMON> l bfc00578

bfc00578 3c08bfc0 lui     t0,0xbfc0                         # 49088

bfc0057c 25080594 addiu   t0,t0,0x594                       # 1428

bfc00580 3c01a000 lui     at,0xa000                         # 40960

bfc00584 01014025 or      t0,t0,at

bfc00588 01000008 jr      t0

bfc0058c 00000000 nop

bfc00590 00000000 nop

bfc00594 40809000 mtc0    zero,C0_BPC

bfc00598 00000000 nop

bfc0059c 40809800 mtc0    zero,C0_BDA

PMON> set regstyle hw

PMON> l bfc00578

bfc00578 3c08bfc0 lui     $8,0xbfc0                         # 49088

bfc0057c 25080594 addiu   $8,$8,0x594                       # 1428

bfc00580 3c01a000 lui     $1,0xa000                         # 40960

bfc00584 01014025 or      $8,$8,$1

bfc00588 01000008 jr      $8

bfc0058c 00000000 nop

bfc00590 00000000 nop

bfc00594 40809000 mtc0    $0,C0_BPC

bfc00598 00000000 nop

bfc0059c 40809800 mtc0    $0,C0_BDA

## Files

The l command is located in mon/dis.c.

## See Also

d command, m command, dump command, more and rdsrec commands.

# fill

The fill command writes a hexadecimal pattern or string to a block of memory.

## Format

The format for the fill command is:

fill from to {val|-s str}-

where:

| from | is the base address for the fill operation. |
| --- | --- |
| to | is the end address for the fill operation. |
| val | is the hexadecimal value of the byte that is written to the area to be filled. |
| -s str | specifies that the memory block should be filled with an ASCII string rather than a particular value. String str is the ASCII string to be written to the memory block during the fill operation if the -s parameter is specified. |

## Functional Description

The fill command fills an area of memory with a specified hexadecimal pattern or repeating string. The pattern can be a single byte or multiple bytes. For the fill command to work correctly, to must be greater than from. If the -s option is specified, the next parameter is interpreted as an ASCII string. Multiple-word strings may be specified by enclosing them in quotes.

For example, to clear an area of memory from 0xa0040000 to 0xa0041000, enter:

PMON> fill a0040000 a0041000 0

To fill an area of memory from 0xa0040000 to 0xa00410000 with the string of values 0x41, 0x42, 0x43, 0x44, and 0x45, enter:

PMON> fill a0040000 a00410000 41 42 43 44 45

To fill an area of memory from 0xa0040000 to 0xa00410000 with the ASCII string "hello world," enter:

PMON> fill a0040000 a0041000 -s "hello world"

## Files

The fill command is located in mon/fill.c.

## See Also

m command.

# copy

The copy command copies a specified number of bytes from one location in memory to another.

## Format

The format of the copy command is:

copy [-f] from to size

where:

| from | declares the source address location. |
| --- | --- |
| to | declares the target address location. |
| size | is the size of the block of memory to be copied. This quantity is specified in bytes. |
| -f | Specifies that the destination is flash memory. |

## Functional Description

The copy command replicates a specified number of bytes from one place in memory to another.

The -f option is used when copying to flash memory. This requires that the board supports writes to that address range, and that the flash memories support sector erase.

If to is less than from, copying is performed in ascending order starting at from. If from is less than to, copying is performed in descending order starting at from + size.

When moving a data block down, the source data is copied from the bottom of the block upwards: and when moving a data block up, the source data is copied from the top of the block downwards. By this technique, there is no risk of copying over data in overlapping block move operations; as the data in the overlapping area is copied first. However, if the -f option is specified, copying is always performed in ascending address order.

## Examples

- This example shows how to copy a block of memory, 8 Kbytes in size, with a base address of 0x80040000, to another 8-Kbyte area starting at the address 0x80060000.

- PMON> copy 80040000 80060000 2000

- This example shows how to copy a block of memory, 4 Kbytes in size, with a base address of 0x80040000, to a 4-Kbyte area of flash memory starting at the address 0xbfc80000.

- PMON> copy -f 80040000 bfc80000 1000

- This example shows how an small application program may be copied into flash.

- PMON> load -B

- Downloading from ethernet2, ^C to abort

- Loaded data from 80100000 to 8010fd4e, length fd4f (64847 decimal)

- PMON> copy -f -f 80100000 bfd00000 fd4f

- Updating flash block which starts at bfd00000

- PMON> ld bfd00000

- section 1) .text, copying bfd00058 to 80040000, len 2238

- section 2) .data, copying bfd02290 to 80042238, len 1078

- section 3) .sbss, clearing 800432b0, len 28

- section 4) .bss, clearing 800432d8, len 0

- section 8) .symtab, processing global symbols

- 96 symbols of 96 symbols loaded

- Setting pc to 80040000

- PMON> g

- Welcome to TINY caves, you may direct me with commands:

- N, NE,  E, SE,  S, SW,  W, NW, UP, DOWN, or QUIT

- You're at road near house.

- >quit

- PMON>

## Files

The copy command is located in mon/copy.c.

## See Also

ld command, g command

# compare

The compare command compares the contents of two memory areas.

## Format

The format of the compare command is:

compare adr1 adr2 siz

where:

| adr1 | specifies the start address of the first memory area. |
| --- | --- |
| adr2 | specifies the start address of the second memory area. |
| siz | is the size of the block of memory to be compared. This quantity is specified in bytes. |

## Functional Description

The compare command compares each byte in the two memory areas. Any bytes that are different are displayed with the address, the expected value, and the actual value read.

## Examples

- This example shows how to compare two blocks of memory, 8 Kbytes in size. The first block has a base address of 0x80040000, the second block starts at 0x80060000.

- PMON> compare 80040000 80060000 2000

## Files

The compare command is located in mon/compare.c.

## See Also

copy command

# search

The search command executes a search for a memory pattern.

## Format

The format for the search command is:

search from to {val|-s str}..

where:

| from | is the start address for the search operation. |
| --- | --- |
| to | is the end address for the search operation. |
| val | is the hexadecimal value that is the object of the search. |
| -s str | specifies that the search operation is for a string str. |

## Functional Description

The search command searches memory for a pattern. The pattern may be a single byte, multiple bytes, or an ASCII string.

If the -s option is specified, the next parameter is interpreted as an ASCII string. To search for a multiple-word string, enclose the string in double quotation marks.

The output of this command is printed to the screen via the more command.

The following example searches for 3c and d4 from 0xa0040000 to 0xa0050000:

PMON> search a0040000 a0050000 3c d4

The following example searches for "ABC" from 0xa0040000 to 0xa0050000:

PMON> search a0040000 a0050000 -s "ABC"

## Files

The source for the search command is located in mon/search.c.

## See Also

d command and more command.

# dump

The dump command uploads S-records or binary to the host port.

## Format

The format for the dump command is:

dump [-B] adr siz

-B	select binary (default if host port is an ethernet port)

where:

adr

is the base address of the data to be uploaded.

siz

the number of bytes to be uploaded.

## Functional Description

The dump command uploads Motorola S-records or binary to the host port. For S-records, all uploaded S-records except the terminating S-record are S3-records. The terminating S-record is an S7-record.

When uploading Motorola S-Records:

The uleof and ulcr Variables

After the dump is completed, the string specified in uleof will be transmit- ted. The default value for uleof is "%".

If the variable ulcr is set to "off", the lines will be terminated by a carriage return ("\r") and a linefeed character ("\n").

If ulcr is set to "on", each line will be terminated by a linefeed character ("\n") only.

The default value for ulcr is "off".

To upload an area of memory using ethernet, type:

PMON> dump -B bfc70000 1000

Uploading to ethernet2, ^C to abort

Dumped data from bfc70000 to bfc70fff, length 1000 (4096 decimal)

PMON>

On the host computer, use tftp to get the data being uploaded:

$ tftp 10.26.3.55

tftp> bin

tftp> rex 1

tftp> get pmon-scriptarea

Received 4096 bytes in 0.0 seconds

tftp> quit

$

## Files

The dump command is located in mon/sdump.c.

## See Also

l command, d command, and m command.

# g

The g command starts program execution.

## Format

The format for the g command is:

g  [-st] [-p progname ] [adr [bptadr]] [-c args]    start execution (go)

-p    specify program name progname in argv[0]

-s    don't set client sp

-t    time execution

<adr>    start address

<bptadr>    temporary breakpoint

-c <args>    args to be passed to client

By default, the g command starts program execution at the address in the EPC register, and sets the stack pointer, sp, to the beginning of the stack area.

## Functional Description

The g command starts program execution. If the user does not specify the starting address adr, execution starts at the current value of the EPC register. This command must only be used once after downloading a new program. Use the c command to continue execution after a breakpoint.

If the user specifies the -c option, pmon passes all arguments after -c to

the client program by the following method:

1) If -c is not specified,

a0 will be set to 1  (argc),

a1 will point to an array of pointers (argv)

where the first pointer points to progname

if the -p progname option was specified

and "g" otherwise

This array will be terminated by a NULL pointer

a2 will point to an array of pointers (envp)

where each pointers points to a zero terminated string,

"variable=value"

This array will be terminated by a NULL pointer

a3 will be the NULL pointer

2) if -c is specified,

a0 will be set to the number of arguments (argc),

a1 will point an array of pointers (argv)

where the first pointer points to progname

if the -p progname option was specified

and "g" otherwise

This array will be terminated by a NULL pointer

a2 will point to an array of pointers (envp)

where each pointers points to a zero terminated string,

"variable=value"

This array will be terminated by a NULL pointer

a3 will be the NULL pointer

If adr is specified, a temporary breakpoint (bptadr) may also be specified. The temporary breakpoint remains in effect only until the next time that program execution is halted. The character '.' may be used as a placeholder for the adr if you wish to specifiy a temporary breakpoint without specifying a start address.

Examples illustrating the use of the g command follow.

PMON> g				Start executing at the current value of

the EPC register.

PMON> g 80040000		Start executing at 0x80040000.

PMON> g 80040000 80040008	Start executing at 0x80040000 and

break at 0x80040008.

## Files

The g command is located in mon/go.c.

## See Also

c command

# b

The b command sets and displays breakpoints.

## Format

The format for the b command is:

b

b adr..

b adr -s str

where:

| adr | specifies an address for the breakpoint. Up to 32 breakpoints addresses can be set. |
| --- | --- |
| -s str | executes the command string when the breakpoint is hit. |

Invoking the b command with no options causes the Monitor to print a list of the current breakpoints.

## Functional Description

The b command sets a breakpoint at the specified address or addresses. Multiple addresses may be specified. Specified addresses must be word-aligned.

This command is effectively a "pass-thru" to the when command. Issuing the command, "b main" is equivalent to issuing the command, "when @pc==main stop". However, unlike the when command, the b command can take multiple addresses.

pmon automatically assigns a number to each breakpoint. pmon allocates the lowest available breakpoint number from 0 to 31 to any new breakpoint.

pmon reports a new breakpoint's number immediately after the breakpoint is set (see the examples at the end of this subsection for illustration of this). The assigned numbers can be used in the db (Delete Breakpoint) command.

The brkcmd Variable

When a breakpoint is reached, the command list specified in the environment variable brkcmd is executed. The default setting for brkcmd is:

brkcmd = "l @pc 1"

This command "l @pc 1", specifies that when the breakpoint occurs, pmon will disassemble one line starting at the address of the program counter.

You can change the breakpoint command variable with the set command. For example, you can include additional monitor commands in the brkcmd variable. You must separate additional commands on the command line with a semicolon. For example, entering the following command lists one line after reaching a breakpoint, and then displays all the register values.

set brkcmd "l @epc 1;r *"

By default, breakpoints are cleared when the load command is executed. See the section on the load command later in this document for details on how to override automatic breakpoint clearing after a download operation.

Some examples illustrating the use of the b command follow.

| PMON> b 8004000c | Set a breakpoint at 0x8004000c. |
| --- | --- |
| Bpt 1 = 8004000c |  |
| PMON> b Bpt 0 = 8004022c Bpt 1 = 8004000c | Display all breakpoints. |
| PMON> b 80041248 -s "r" | Set a breakpoint at 0x80041248. Display registers when the breakpoint is encountered. |

## See Also

db, when, .d, ld, and load commands.

# when

The when command sets complex breakpoints

## Format

The format for the when command is:

when condition action

where:

| condition | Specifies the conditions under which the breakpoint will occur. |
| --- | --- |
| action | Specifies the action to be taken when the breakpoint occurs. |

## Functional Description

The when command provides a very flexible way to set breakpoints and to specify the action that should be taken when they are encountered.

Each when command takes two arguments, a condition, and an action. The condition can be something as simple as @pc==main (when the program counter is equal to value of the symbol main), or it can be a complex expression that might include specifying the contents of memory locations.

Actions can be any pmon command string. But it can also include the pseudo command stop. For example, the command, "when @pc==main stop" specifies that when the program counter has the value of the symbol main, execution should stop.

Conditional expressions can be combined in any arbitrary manner to provide extremely complex breakpoint conditions. For example,

PMON> when @pc==main2&&(^tcbchn==task1||^tcbchn==task2) stop

which will stop execution when the pc is equal to main2, and either tcbchn is equal to task1 or task2.

In the preceeding examples we have chosen to stop execution when the condition is met. However, it is possible to specify a list of commands. For example,

PMON> when @pc==sort "r;d -w dat3 1"

specifes that when the pc is equal to 'sort', the commands 'r' and 'd -w dat3 1' will be executed. The ';' is used to separate commands, and the double quotes are necessary because the argument contains spaces.

pmon is responsible for deciding when to use hardware breakpoint registers (if present). In general, pmon  uses software breakpoints unless there is a specific reason that a hardware breakpoint is required.

If you issue a command that requires a hardware breakpoint register. But that pmon finds that there is not one available. pmon will automatically use trace mode when you issue the c command to continue execution. Because trace mode is not real-time, pmon will warn you at the time that you set the breakpoint that this will require non real-time execution.

## Examples

- Stop when a memory location changes.

You can specify data locations by using the dereferencing operator '^'. For example,

PMON> when '^tcbchn!=0' stop

means that execution should stop when the contents of the memory location specified by the symbol tcbchn is not equal to zero. Note that expressions that include the character '!' must be enclosed within single- or double-quotes to supress the normal history substitution mechanism. Single- or double-quotes must also be used if the expression contains spaces.

- Break on the 20th time around a loop.

- when @pc==0x80041234 "r a @a+1"

- when @a==0t20 stop

These commands specify that execution should pause each time the pc has the value 0x80041234. Each execution pauses, the command "r a @a+1" will be executed. This command increments the value of pseudo register 'a'. See the r command for more information on the pseudo registers.

If during one of these pauses in execution pseudo register 'a' has the value 20 (decimal), execution will stop.

- Break when fred() is executed after jim(), but not after mike().

- when @pc==jim "r a 1"

- when @pc==mike "r a 0"

- when "@pc==fred && @a==1" stop

- Stop if dat1 ever gets set back to zero.

- when "^dat1 != 0" "r a 1"

- when "^dat1 == 0 && @a == 1" stop

This requires a hardware data breakpoint register in order to execute this in real-time. In this mode a brief pause will occur each time a write to dat1 occurs.

If you don't have a hardware data breakpoint register, you can maintain real-time performance by only checking the value at specific points in the program.

when "@pc == fred && ^jim != 0" "r a 1"

when "@pc == fred && ^jim == 0 && @a == 1" stop

- Find the maximum value written to a specified memory location.

- when "^dat1 > @a" "r a ^dat1"

Use the command "r a" to examine the value when the program stops.

- Stop if jim() is ever called with a 2nd argument of zero.

- when "@pc == jim && @a1 == 0" stop

- Count how many times jim() is executed.

- when "@pc == jim" "r a @a+1"

Use the command "r a" to examine the value when the program stops.

## See Also

b, db, ld, and load commands.

# db

The db command deletes the specified breakpoints.

## Format

The format for the db command is:

db [numb|*]

where:

| numb | is the breakpoint number to be deleted. |
| --- | --- |
| * | deletes all breakpoints. |

Entering db without any parameters lists all existing breakpoints. Entering an asterisk ("*") instead of a breakpoint number deletes all the existing breakpoints.

## Functional Description

The db command deletes one or more specified breakpoints.

Examples illustrating the use of the db command follow.

| PMON> db 3 | Delete breakpoint 3. |
| --- | --- |
| PMON> db 4 6 | Delete breakpoints 4 and 6. |
| PMON> db * | Delete all breakpoints. |
| PMON> db Bpt 0 = 8004000c | Display all breakpoints. |

## See Also

d, when, ld, and load commands.

# t/to

The t command performs a trace (single step) operation.

## Format

The format for this command is:

t [-vbci]

or:

to [-vbci]

where:

| -v | lists each step (verbose). |
| --- | --- |
| -b | captures only branches. |
| -c | captures only calls (jal instruction). |
| -i | stops on invalid program counter. |
| cnt | traces cnt instructions. |

## Functional Description

The t command executes the instruction addressed by the current value of the EPC register. The to command is similar to the t command, except that the to command treats an entire procedure as a single step. For example, if the current instruction at EPC is a jump and link instruction, jal, the next stop is at EPC+8.

A branch instruction and the instruction in its delay slot are executed as a single step. This also means that two-instruction loops are treated as a single step. This command is implemented by setting a breakpoint at the following instruction.

The command or commands that are executed on completion of the single step is determined by the value of the environment variable brkcmd.

An example illustrating the use of this command follows.

PMON> t

start+0x240 80040240 lui t1,0xa07f

## Files

The t/to commands are located in mon/go.c.

# c

The c command makes program execution continue after a breakpoint has stopped program execution.

## Format

The format for the c command is:

c [bptadr] where:

| bptadr | specifies a single breakpoint. The breakpoint is removed when execution halts at this specified address. |
| --- | --- |

Invoking the c command with no arguments causes the program execution to continue from the address specified in the epc register.

## Functional Description

When the user enters the c command, program execution starts at the address pointed to by the EPC register's current value. Use the g command to start program execution from an address specified on the command line.

As an option, a single temporary breakpoint may be specified. The temporary breakpoint is removed when execution halts. The temporary breakpoint is removed if another breakpoint stops program execution first.

Examples of the c command follow.

| PMON> c | Continue execution until exit or a regular breakpoint is encountered. |
| --- | --- |
| PMON> c 80040104 | Continue execution until 0x80040104 or a regular breakpoint is encountered. |

## Files

The c command is located in mon/go.c.

## See Also

g command

# call

The call command executes a subroutine.

## Format

The format for the call command is:

call adr [-s str|val]..

where:

| adr | is the address of the subroutine to be executed. |
| --- | --- |
| -s str | is a string argument. |
| val | is a value to be passed. |

The call command calls a function using the standard C calling convention. The "-s str" and val options permit arguments to be passed to the subroutine.

## Functional Description

The call command executes a downloaded subroutine, using a jalr instruction to pass control to the specified address. This does not affect the existing value of the saved registers. Instead the subroutine is called directly from C code without restoring the saved registers. Control returns to PMON via the usual subroutine return mechanism.

If the user specifies arguments, these are passed using the standard C calling convention. If the "-s" option is specified, the following argument is assumed to be a string. In this case the address of the string is passed to the subroutine. If a numerical value is specified in place of the "-s", it will be evaluated according to the existing rules and passed to the function. Up to five arguments may be passed.

This command is usually used to provide a method of displaying application-specific data structures. For example, if your application has a complex, linked-list data structure, you might find it helpful to add a function to your program that can display the structure. The call command can then be used to invoke this function from pmon prompt at any time in the execution, even between two single-step operations.

Examples illustrating the use of the call command follow.

PMON> call prstat		Call the function whos name is 'prstat'.

PMON> call prrec a0040000+8	Call the function 'prrec' and pass it

the value 0xa0040008 as the first argument.

PMON> call printf -s "hello world"

Call the function printf and pass it the

address of the string "hello world".

## Files

The call command is located in mon/call.c.

## See Also

g command

# h

The h command provides on-line help.

## Format

The format for the h command is:

h [*|cmd-]

where:

| * | provides detailed help on all the commands. |
| --- | --- |
| cmd | is a command. pmon then provides help on the stated command. |

If the command is executed without any parameters, then pmon lists all the available commands.

## Functional Description

The h command provides on-line help. If issued without arguments, all commands are listed. If issued with one or more command names as an option, it produces more detailed help on those commands.

The "*" option produces detailed help on all the commands, using the more command to control output on the screen.

Examples illustrating the use of the h command follow.

PMON> h

h  The help command            help  The help command

?  The help command            hi  display command history

m  modify memory               r  display/set register

d  display memory              l  list (disassemble) memory

copy  copy memory                 fill  fill memory

search  search memory               compare  compare memory

when  complex breakpoint          g  start execution (go)

c  continue execution          t  trace (single-step)

to  trace over (single-step)    b  set breakpoint at address

db  delete breakpoint           sym  define symbol

ls  list symbols                set  display/set variable

vers  display version info        sh  The command shell

more  paginator                   debug  enter gdb mode

ab  set access (data) bpt       stty  Set/display terminal options

load  load memory from hostport   dump  send srecs/binary to hostport

mt  memory test                 call  call function

ld  load elf or srec            erase  erase flash

exec  execute script from address flush  flush caches

PMON> h stty

stty  [opt][<baud>][<term>]       Set/display terminal s

-v    list possible baud rates and terminal types

-a    list all settings

<baud>    set baud rate

<term>    set terminal type

sane    set sane settings

ixany    allow any char to restart output

-ixany    allow only <start> to restart output

ixoff    enable tandem mode

-ixoff    disable tandem mode

echo    enable echo

-echo    disable echo

# hi

The hi command lists the command history.

## Format

The format for the hi command is:

hi [cnt]

where:

| cnt | is the number of commands to list. |
| --- | --- |

Entering the command with no parameters lists the last 200 executed com- mand lines to the screen.

## Functional Description

The hi command shows the command history, together with the history number for each command, in reverse order (the last command entered is listed first; the first command entered is listed last). The command num- bers are reset to zero each time the system is reset.

Entering the hi command with no arguments lists the last 200 commands. This option is useful for determining the history number for a particular command.

The user can page through the output of the hi command, one screen at a time.

The optional cnt parameter selects a set number of lines to be output. The history list is intentionally in the reverse order to that used in a C shell, so that the latest entry is displayed first. If a command line is identical to the previous command, it is not added to the command history.

Examples illustrating the use of the hi command follow.

PMON> hi 3	Display the three last commands.

14 hi 3

13 hi

12 l

PMON> hi	Display the entire history, using more

13 hi	to control the screen output.

12 l

11 to

10 t

9  l

8  g start main

7  hi

6  g

5  ls -a @epc

4  d Pmon+200+0t13*4

more-(q)

## See Also

sh command, which maintains a command history.

# set

The set command sets and displays environment variables.

## Format

The format for this command is:

set [name [value]]

where:

| name | is the name of the environment variable to set. |
| --- | --- |
| value | is the string to which the environment variable is set. |

Entering the set command with no arguments displays all the current environment variables.

## Functional Description

The set command is used to set or display environment variable values.

In some cases, when pmon displays a variable's current value, pmon prints a list of allowed values enclosed in square brackets; in other cases, no list is shown. In general, when the value is a numeric value, or when the value has an unlimited range of possible values, no list is shown.

The set command does not evaluate the specified value or check the specified value against a list of allowed values. Value checking is only performed when a command uses a variable.

To set a variable to a multiple-word value, enclose the value in single or double quotation marks.

Examples illustrating the use of the set command follow.

PMON> set

brkcmd = "l @pc 1"

datasz = -b          [-b -h -w]

inalpha = hex         [hex symbol]

inbase = 16          [auto 8 10 16]

moresz = 10

regstyle = sw          [hw sw]

rptcmd = trace       [off on trace]

trabort = ^K

uleof = %

ulcr = off         [off on]

validpc = "_ftext etext"

dlecho = off         [off on lfeed]

dlproto = none        [none XonXoff EtxAck]

hostport = ethernet

prompt = "PMON> "

etheraddr = 00:02:d8:00:00:52

ipaddr = 10.26.3.55

heaptop = 80040000

diag = 0           [N[:dev]]

clkfreq = 150

memsize = 0x03fdb800

cputype = 4000  #

PMON> set moresz

moresz = 10

PMON> set moresz 20

Display current value of Cause Register and display all general-purpose registers:

PMON> set brkcmd "l @epc 1;r cause;r"

Environment Variables and Default Values

| Environment Variable | Default Value | Options |
| --- | --- | --- |
| brkcmd | "l @epc 1" | command list |
| datasz | -b | [-b¦-h¦-w] |
| dlecho | off | [off¦on¦lfeed] |
| dlproto | none | [none¦XonXoff¦EtxAck] |
| etheraddr | 00:02:d8:00:00:52 | string |
| ipaddr | 10.26.3.55 | string |
| heaptop | 80040000 | string |
| hostport | ethernet | tty0,ethernet,ethernet0, ethernet1,ethernet2 |
| inalpha | hex | hex symbol |
| inbase | 16 | [auto¦8¦10¦16] |
| moresz | 10 | 0-n |
| prompt | "PMON> " | string |
| regstyle | sw | [hw¦sw] |
| rptcmd | trace | [off¦on¦trace] |
| trabort | ^K | unused |
| ulcr | off | [off¦on] |
| uleof | % | string |
| validpc | "_ftext etext" | string |
| diag | 0 | [N[:dev]] |
| clkfreq | 150 | cpu clock frequency |
| memsize | 0x3fdb800 | size of SDRAM excluding SDRAM used by pmon |
| cputype | 4000 | BRECIS chip type |

Environment variables can be set and displayed using the set command.

Brief descriptions of each of the variables follow, together with references to their complete descriptions.

- brkcmd - This variable specifies a sequence of pmon commands that are executed when a breakpoint halts program execution. See the b command.

- datasz - This variable controls whether data is displayed in byte, half-word, or word groups. See the d command.

- dlecho - This variable controls whether the target board echoes on downloads. An entire line can be echoed, a single line-feed character can be echoed, or there can be no echo at all. See the load command and the section on flow control.

- dlproto - This variable selects the download protocol for transfers via RS-232C. pmon supports Xon/Xoff and EtxAck download protocols. See the load command and the section on flow control.

- etheraddr - This variable specifies the hardware Ethernet address. See the load command and the section on downloading via Ethernet.

- ipaddr - This variable specifies the Internet Protocol address. See the load command and the section on downloading via Ethernet.

- heaptop - This variable specifies the highest allowable address in the heap maintained by pmon from low (0x8000000) memory.  pmon code is also placed near the end of memory. See the load command.

- hostport - This variable selects whether tty0,  ethernet, ethernet0, ethernet1, ethernet2 is the host port. See the load command and the section on flow control.

- inalpha - This variable selects whether strings starting with the ASCII characters a, b, c, d, e, and f are interpreted as symbols or hexadecimal numbers. See the sh command.

- inbase - This variable selects the default input base for numeric values. Users can input octal, decimal, or hexadecimal numbers by changing this variable. See the sh command.

- moresz - This variable specifies how many lines to display during screen-at-a-time display. See the more command.

- prompt - This variable defines the pmon prompt. An example of using this command is when you need to set the prompt to "PMON> " for compatibility with a source-level debugger. To do this use the following commands.

This will set the prompt to "PMON> " (note the space) and save this new value in the non-volatile memory (if supported).

- regstyle - This variable defines whether hardware or software names are displayed for the MIPS 4Km registers in the l command. See the l command.

- rptcmd - When this variable is set to "on," the previous command is executed again when the user enters an empty line. See the sh command.

- ulcr - This variable defines whether there is a carriage return or both a carriage return and a linefeed character at the end of the line during dumps. See the dump command.

- uleof - This variable specifies a string that is sent to the host after a dump to the target has completed. See the dump command.

- validpc - This variable specifies the range of valid PC values during program tracing. See the trace command.

- to the console. However, you may direct it to another port. For exampldiag - This variable specifies the diagnostic level. zero is off, non-zero is on. Valid values are 0 thru 9. By default the diagnostic output is sent e,

- PMON> set diag 2:/dev/tty1

- clkfreq is the MIPS CPU clock frequency set by pmon.

- memsize is the amount of available SDRAM excluding SDRAM used by pmon.

- cputype is the BRECIS chip type.

# ld

The ld command reads an elf image or srec image in SDRAM or flash and loads the image to SDRAM.

## Format

The format for this command is:

ld  [-sbeS] addr                                     load elf or srec

-s    don't clear symbols

-b    don't clear breakpoints

-e    don't clear exception handlers

-S    don't load symbols

Functional Descrition

The ld command accepts programs and data found in SDRAM or flash in

Motorola S-record or ELF format and places the program in SDRAM ready for

execution.

The ld command normally clears the symbol table, exceptions handlers, and

all breakpoints.  The value of the EPC register is set automatically to

the entry point of the program.  Therefore, to execute the program, only

the g command is required.

See Also

load command, g command.

# stty

The stty command displays and sets terminal options.

## Format

The format for this command is:

stty	[-av] [baud] [sane] [term]

[ixany|-ixany] [ixoff|-ixoff]

where:

| -a | gives a long listing showing all current settings. |
| --- | --- |
| -v | displays the possible choices for baud rate and terminal type. |
| baud | sets the baud rate. |
| sane | resets terminal settings to the default. |
| term | sets the terminal emulation type. |
| ixany | allows any character to restart the output. |
| -ixany | allows only START to restart the output. |
| ixoff | enables the tandem mode. |
| -ixoff | disables the tandem mode. |

When invoking the stty command with no parameters, pmon displays the terminal type and baud rate for the tty0 port.

## Functional Description

The stty command displays and sets the terminal options, such as terminal emulation type, baud rate, and ioctl settings. First, to display the current terminal type, baud rate, and ioctl settings, enter:

PMON> stty -a

To change the baud rate or terminal type, simply enter the new set- ting after stty.

Examples illustrating the use of this command follow.

PMON> stty

baud=57600

PMON> stty -a

baud=57600

istrip ixon -ixany -ixoff icanon echo echoe icrnl onlcr

erase=^H stop=^S start=^Q eol=^J eol2=^C vintr=^C

PMON> stty -v

Baud rates:

50     75    110    134    200    150    300    600

1200   1800   2400   4800   9600  19200  38400  57600  76800 115200 1 Terminal types:

tvi920      vt100

baud=57600

# flush

The flush command flushes the data and/or instruction cache.

## Format

The format for the flush command is:

flush [-di]

where:

| -d | flushes the data cache only. |
| --- | --- |
| -i | flushes the instruction cache only. |

Entering flush without any parameters flushes both caches.

## Functional Description

The flush command performs a hard flush of the data and/or instruction cache. All entries will be flushed, even those that had been locked.

## Files

f5000cfl.s

# vers

The vers command prints the version number of pmon.

## Format

The format for the vers command is:

vers [-a]

where:

| -a | List all version numbers |
| --- | --- |

## Functional Description

Entering vers without any parameters prints the primary version number of pmon. This is the value of the file 'version' in either the pmon directory. The -a option lists the version numbers of all the components that were used to build pmon.

## Examples

PMON> version

6.5.0

PMON> vers -a

pmon:6.5.0 mon:5.3.8 lib:5.3.17

# Command shell

The “command shell” is an embedded command that executes the pmon command typed following the prompt.

## Functional Description

The following syntactic rules apply to all command lines entered at the pmon prompt.

- Multiple commands can appear on one line if each command is separated by a semicolon (;).

- Register names are replaced by their contents if the register name is prefixed with an "at" symbol (@).

- Symbol names are replaced by their value if the symbol name is pre fixed with an ampersand symbol (&).

- Control-S pauses the output stream.

- Control-Q restarts the output stream.

- Control-C aborts the current command.

The shell also maintains a command history. Previous command lines are recalled either with Emacs-like commands or with C Shell "!" notation. the following table lists the commands that are supported by pmon.

| Command | Action |
| --- | --- |
| ^P | Recall previous command |
| ^N | Recall next command |
| ^F | Move cursor once character to the right (forward) |
| ^B | Move the cursor one character to the left (back) |
| ^A | Move the cursor to the beginning of the line |
| ^E | Move the cursor to the end of the line |
| ^D | Delete character at cursor position |
| ^H | Delete character to the left of the cursor |
| !str | Recall and execute the last command that started with the string str |
| !num | Recall and execute command number num |
| !! | Recall and execute last command |
| +-/() | Algebraic operators |
| ^addr | Substitute with contents of address addr |
| @name | Substitute with contents of named register |
| &name | Substitute with value of symbol name |
| 0xnum | Treat num as a hexadecimal number |
| 0onum | Treat num as an octal number |
| 0tnum | Treat num as an decimal number |

## The inbase,  inalpha, prompt, and rptcmd Variables

The following paragraphs describe the inbase, inalpha, prompt, and rptcmd environment variables:

inbase - This variable selects the default input base for numeric values. A value of 8, 10, or 16 selects that base as the assumed default. If "auto" is specified, the base is determined according to the usual C language rules (0x = hex, leading 0 = octal, otherwise decimal).

If inbase is set to 8, 10, or 16, then values starting with zero through nine are assumed to be values in the specified base. If inbase is set to "auto", then values starting with zero are assumed to be octal, and numbers starting with one through nine are assumed to be decimal.

The following lists the rules that hold in setting the default numeric base.

| Inbase | Base |
| --- | --- |
| 0x | Hexadecimal |
| 0t | Decimal |
| 0o | Octal |
| [g-zG-Z@_.] | Symbol |
| & | Symbol |
| @ | Register |

inalpha - This variable selects whether arguments starting with a, b, c, d, e, or f are interpreted as symbols or as hexadecimal numbers.

Setting inalpha to "hex" causes pmon to nterpret the argument as a hexadecimal value, if possible. If the argument cannot be interpreted as a hexadecimal value, then pmon checks the symbol table to see if the argument is a known symbol.

Setting inalpha to "symbol" causes pmon to check the symbol table first.

It is also possible to specify values using simple expressions using the arithmetic operators +, -, *, and /. Expressions do not take spaces between the numerals and operators. For example,

PMON> b printf+4

sets a breakpoint at (printf+4). Any combination of register names, symbols, and values may be used. The precedence order of operators is the same as that defined by the C language. Two examples showing the use of simple arithmetic operators follow:

| PMON> ls -v start+0x240 | Show the actual address. |
| --- | --- |
| PMON> d map+0t10*4 | Dump memory at (map+(10*4)). |
| PMON> d @a0+0t56 | Dump memory at 56(a0) |
| PMON> d ^tcbchn | Dump memory at contents of tcbchn |

prompt - This variable specifies the command prompt string. The metacharacter "!" is replaced by the current history number. For example,

PMON> set prompt "!> "

23> _

It is not possible to display system variables in the prompt.

rptcmd - When this environment variable is set to "on", the previous command is repeated when the user enters a blank line. When set to "trace", only trace commands (t or to) are repeated.

## See Also

hi (command history) and set (setup and display environment variables) commands.

# more

The more command provides screen-at-a-time control for user input.

## Format

The more command is an embedded command and is not accessible to the user on the command line.

## Functional Description

The more command is not specified by the user on the command line, but is implicitly used by certain commands. After displaying the number of lines according to the value of the moresz environment variable, the more command displays the prompt "more-" Commands that use the more command include h, hi, d, l, search, and ls.

The user can enter the following commands at the "more-" prompt:

| Command | Action |
| --- | --- |
| Space | Print one more page |
| /str | Search forward for string str |
| n | Repeat last executed search |
| <CR> | Show next line |
| q | Quit from the more prompt and return to the monitor prompt |

The moresz Variable

moresz sets how many lines are displayed on one screen during screen-at-a-time output. If moresz is set to zero, the screen scrolls continuously. The ^S or ^Q control sequence must be used to pause the output, and the ^C control sequence must be used to terminate output.

For example, to set the default number of lines output by the more command to 12, enter:

PMON> set moresz 12

## See Also

set command for the setup of the environment variables.